| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0453-v1 |
| --- | --- |

| Title: | Modification of LFNST for MIP coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Wenhan Qiao, Xinwei Li, Yanzhuo Ma, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn whqiao@stu.xidian.edu.cn xinweili@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In VVC Draft 7, LFNST transform set 0 is always selected and transform coefficients transpose flag is always equal to 0 for a MIP coded block. In this contribution, it is proposed to utilize intra_mip_mode[ xTbY ][ yTbY ] to select LFNST transform set and utilize intra_mip_transposed[ xTbY ][ yTbY ] to determine transform coefficients transpose flag. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {-0.04%, -0.04 %, -0.01 %} for AI and {x.xx%, x.xx %, x.xx %} for RA over VTM 7.0. As for Class A1 sequences, the performance impact for {Y, U, V} is {-0.12%, -0.24 %, -0.01 %} for AI and {x.xx %, x.xx %, x.xx%} for RA over VTM 7.0.

Introduction

In VVC Draft 7[1], the LFNST transform set and transform coefficients transpose flag are both determined by the intra prediction mode predModeIntra of the current transform block. After obtaining the intra prediction mode predModeIntra of the current transform block, the following mapping is needed:

- If current block is MIP coded block, predModeIntra is mapped to PLANAR

- If current block is CCLM coded block, predModeIntra is mapped to the co-located luma intra prediction mode

Then predModeIntra is further derived from wide angle intra prediction mapping with a range of [-14, 83].

- 1.1 Selection of transform sets

There are totally 4 transform sets and 2 non-separable transform matrices (kernels) per transform set are used in LFNST. The mapping from the predModeIntra to the transform set idx lfnstTrSetIdx is pre-defined as shown in Table 1.

Table 1. LFNST transform set selection table

| IntraPredMode | lfnstTrSetIdx |
| --- | --- |

| predModeIntra < 0 | 1 |
| --- | --- |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |

- 1.2 Determine of transpos flag

The transform coefficients transpose flag determines the scan order of the LFNST output (Decoder). Figure 1 shows the scan order with different transform coefficients transpose flag.

Figure 1  Scan order with different transform coefficients transpose flag

The transform coefficients transpose flag is determined by predModeIntra :

- if predModeIntra is less than or equal to 34, the transform coefficients transpose flag is set to 0

- else, the transform coefficients transpose flag is set to 1

For MIP coded blocks, it is mapped to the PLANAR mode, the LFNST transform set 0 is always selected and transform coefficients transpose flag is always equal to 0. In VVC, the MIP blocks with the width and height not less than 16 can perform LFNST. These MIP coded blocks belong to the category of mipSizeId=2. There are 6 MIP modes when mipSizeId equal to 2.

Proposed Methods

In this contribution, it is proposed to utilize intra_mip_mode[ xTbY ][ yTbY ] to select LFNST transform set and utilize intra_mip_transposed[ xTbY ][ yTbY ] to determine transform coefficients transpose flag.

- Select the LFNST transform set by MIP mode idx (intra_mip_mode[ xTbY ][ yTbY ]).

For MIP coded block, the LFNST transform set is selected as shown in Table 2.

Table 2. Modefied LFNST transform set selection table for MIP coded block

| intra_mip_mode[ xTbY ][ yTbY ] | lfnstTrSetIdx |
| --- | --- |

| 0 <= intra_mip_mode[ xTbY ][ yTbY ] <= 2 | 0 |
| --- | --- |
| intra_mip_mode[ xTbY ][ yTbY ] == 3 | 2 |
| intra_mip_mode[ xTbY ][ yTbY ] == 4 | 1 |
| intra_mip_mode[ xTbY ][ yTbY ] == 5 | 3 |

- Determine the LFNST transform coefficients transpose flag by MIP transpose flag (intra_mip_transposed [ xTbY ][ yTbY ]).

For MIP coded block, the LFNST coefficients transpose flag is determined as follow:

- if intra_mip_transposed [ xTbY ][ yTbY ] is euqal to 0, the LFNST transform coefficients transpose flag is set to 0

- else, the LFNST transform coefficients transpose flag is set to 1

Experimental results

The experiments were performed based on VTM7.0 software[2] under JVET common test condition [3]. The coding performance is provided in Table 3. According to the simulation results, it shows that under the CTC, the overall performance impact for {Y, U, V} is {-0.04%, -0.04 %, -0.01 %} for AI and {x.xx%, x.xx %, x.xx %} for RA over VTM 7.0. It is performs better on large resolution sequences. As for A1 sequences, the performance impact for {Y, U, V} is {-0.12%, -0.24 %, -0.01 %} for AI and {x.xx %, x.xx %, x.xx%} for RA. The peak value of the gain under AI condition can reach to -0.19% on the Tango2 sequence of the Y component. Besides, there is no impact on both the encoding time and decoding time.

Table 3. Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.24% | -0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.03% | 100% | 100% |
| Class B | -0.01% | 0.06% | -0.08% | 100% | 100% |
| Class C | -0.02% | 0.03% | 0.05% | 100% | 101% |
| Class E | -0.03% | -0.16% | 0.04% | 99% | 99% |
| Overall | -0.04% | -0.04% | -0.01% | 100% | 100% |
| Class D | 0.00% | -0.09% | 0.05% | 100% | 100% |
| Class F | -0.01% | -0.05% | 0.06% | 100% | 99% |

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.02% | 0.19% | 100% | 100% |
| Class C | 0.00% | -0.08% | -0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.13% | -0.09% | 100% | 99% |
| Class F | 0.00% | 0.01% | 0.02% | 100% | 100% |

Conclusions

In this contribution, it is proposed an alternative method of LSNFT transform set selection and coefficients transpose flag determine if one of 6 MIP modes is used for the current block. This method allows the blocks coded in different MIP modes can choose different transform sets and the blocks with different MIP transpose flag can use different coefficients transpose flag. Coding gain was obtained with no impact on both the encoding time and decoding time .It is recommended to adopt into VVC.

- 5 References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[3] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).