| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0457-v1 |
| --- | --- |

| Title: | MIP input value calculation unification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan, Haixin Wang, Junyan Huo, Danni Wang, Yanzhuo Ma Fuzheng Yang Yuanfang Yu Yang Liu | Email: | swan@nwpu.edu.cn hxwang_2@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn dnwang@stu.xidian.edu yzma@mail.xidian.edu.cnfzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

Proposed is to unify the MIP input value calculation without dependency on the mipSizeId and sample position. The proposed modification can get a good unification to calculate MIP input value. The prediction is identical with the prediction of VVC.

The experimental results are as below:

For AI configuration: 0.00%, 0.00%, and 0.00%, with 100% EncT, 100% DecT;

For RA configuration: 0.00 %, 0.00%, and 0.00%, with 100% EncT, 100% DecT.

Introduction

In VVC[1], matrix-based intra prediction (MIP) is applied to derive the intra predicted samples of luma component as：

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predSize+x *p i  ) +            
			( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i ) ) >> sW ) + pTemp[ 0 ]	   (1)

Input value p[i] is calculated by:

- If mipSizeId is equal to 2, the following applies:

p[ i ] = pTemp[ i + 1 ] − pTemp[ 0 ]	for i = 0.. inSize – 1                                 (2)

- Otherwise (mipSizeId is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )                                           (3)
p[ i ] = pTemp[ i ] − pTemp[ 0 ]	for i = 1..inSize – 1                                     (4)

The calculation method of p[ i ] depends on mipSizeId and sample position. Specifically, for different mipSizeId, p[i] is calculated in a different way. For mipSizeId less than 2, p[ 0 ] and other p[ i ] are calculated in a different way.

In our previous proposal[2],  p[ 0 ] in mipSizeId equal to 0 and 1 can be modified with a sign change as

Input value p[i] is calculated by:

- If mipSizeId is equal to 2, the following applies:

p[ i ] = pTemp[ i + 1 ] − pTemp[ 0 ]	for i = 0.. inSize – 1                                   (5)

- Otherwise (mipSizeId is less than 2), the following applies:

p[ 0 ] = ( 1  <<  ( BitDepth − 1 ) ) - pTemp[ 0 ]                                            (6)
p[ i ] = pTemp[ i ] − pTemp[ 0 ]	for i = 1..inSize – 1                                     (7)

It is asserted that this modification can provide a smaller dynamic range of mWeight.

Proposed Method

Based on (5), (6) and (7), we can find that pTemp[ 0 ] is always in the right of “-”.  (5) and (7) are similar. The different between (5) and (6-7) is the introduction of (1 << BitDepth-1) in (6).

We further propose a unified input value calculation method. Specifically,  (1 << BitDepth-1) is put into the last item of pTemp, pTemp[2 * boundarySize]. In this way, p[ i ] can be calculated by using pTemp[ i + 1] minus pTemp[ 0 ].

Draft spec

- The reduced top and left boundary samples redT and redL are assigned to the boundary sample array pTemp[ x ] with x = 0..2 * boundarySize − 1 as follows:

- If isTransposed is equal to 1, pTemp[ x ] is set equal to redL[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redT[ x ] with x = 0..boundarySize − 1.

- Otherwise, pTemp[ x ] is set equal to redT[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redL[ x ] with x = 0..boundarySize − 1.

- The boundary sample array pTemp[2 * boundarySize  ] is filled by 1  <<  ( BitDepth − 1 ).

- The input values p[ x ] with x = 0.. inSize − 1 are derived as follows:

p[ x ] = pTemp[ x + 1 ] − pTemp[ 0 ]                                                       (255)

- If mipSizeId is equal to 2, the following applies:

p[ x ] = pTemp[ x + 1 ] − pTemp[ 0 ]		(255)

- Otherwise (mipSizeId is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1..inSize − 1	(256)

Analysis

By using the proposed method, the input value in (6) is arranged into the last item of p[ i ]. The mWeight can mathematically changed by put the first column to the last column.

The proposed method can be assumed as a editorial change. This modification is also a cleanup of the spec. The predicted sample of the proposed method is identical to the predicted sample in VVC.

Experimental results

The experiments are performed based on VTM7.0 software [3] under JVET common test condition [4]. Table 1 presents a summary of the results.

Table 1. Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusions

In this contribution, unified input value p[ i ] calculation is proposed, which removes the dependency of p[ i ] on the mipSizeId and sample position and cleans up the spec. It is recommended to adopt the unitfication into VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] J. Huo, H. Wang, Y. Sun, Y. Ma, S. Wan, F. Yang, Y. Yu, Y. Liu,  Non-CE 3: MIP simplification, Document of JVET-P0136, Geneva, Switzerland, October 2019

[3] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[4] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).