| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0461-v1 |
| --- | --- |

| Title: | QP-independent and slice type-independent initialization of context models for the high throughput CABAC mode of JVET-P0300 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Benjamin Bross, Tung Nguyen, Detlev Marpe, Heiko Schwarz, Thomas Wiegand | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer @hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

A QP-independent and slice type-independent initialization method is proposed for the high throughput CABAC mode of JVET-P0300. This opens up the possibility for simplifications in a hardware or software implementation. For example, instead of deriving a context model index (for a certain bin), the context model state (which corresponds to a probability value) can directly be derived. For QPs 2-17, the BD-rates are 4.96 % for AI, x.xx% for RA, x.xx % for LB, and x.xx % for LP. For the lossless setup, the bit rates increase by 7.78% for AI, x.xx % for RA, and 6.78 % for LB.

# Proposed method

The high throughput CABAC mode of JVET-P0300 maintains per context model two variables valMpsGB and pShiftGB. These represent a probability value for arithmetic encoding of associated bins (see JVET-P0300 for details). More precisely, the derivation of the LPS in the arithmetic coding engine employs a simple bit shift as follows:

valMps = valMpsGB
ivlLpsRange = ivlCurrRange >> ( pShiftGB + 1 )

Instead of deriving variables valMpsGB and pShiftGB of a context model from a (slice type-dependent) initialization value and the QP, it is proposed to fix one value for valMpsGB and pShiftGB per context model. By appropriately choosing values for valMpsGB and pShiftGB, this implicitly corresponds to a context reduction and it may simplify context derivation rules.

## Bypass mode

It is further proposed to use the original bypass mode of CABAC in the case when pShiftGB equals 0.

# Simplified context derivation and implicit context reduction

As only few different values can occur for pShiftGB, only few different probability values can occur. For example, 0 <= pShiftGB <= 3 means that only 7 different probabilities can occur for a context model (1/16, 1/8, 1/4,1/2, 3/4, 7/8, and 15/16). In other words, a context model can be interpreted as one out of 7 different probability values.

For a context derivation rule, this opens up the possibility to directly derive values for valMpsGB and pShiftGB instead of a context model index. Furthermore, if two or more context models have the same valMpsGB and pShiftGB, they don’t need to be distinguished in the context derivation rule. This may be beneficial for optimized hardware or software implementations.

# Context model initialization

One value for valMpsGB and pShiftGB has been trained per context model using the initialization value computation method of JVET-N1021 with VTM-7.0 bitstreams produced using the low QP setting with QPs 2, 7, 12, and 17 with 100 frames as well as the lossless setting. For all bitstreams, the normalized rate d() of JVET-N1021 has been calculated for the entropy coding method.

# Experimental results

## QP 2-17 (100 frames)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5,77% | 4,08% | 4,51% | 101% | 84% |
| Class A2 | 4,87% | 4,64% | 4,74% | 92% | 80% |
| Class B | 5,65% | 5,58% | 6,04% | 96% | 84% |
| Class C | 4,41% | 3,93% | 4,05% | 95% | 83% |
| Class E | 3,82% | 3,79% | 4,21% | 92% | 80% |
| Overall | 4,96% | 4,51% | 4,82% | 95% | 82% |
| Class D | 4,35% | 3,74% | 3,95% | 90% | 86% |
| Class F | 6,10% | 5,31% | 5,43% | 96% | 90% |
| Class TGM | 8,65% | 8,69% | 8,79% | 95% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 4,79% | 5,01% | 3,61% | 92% | 88% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 4,28% | 4,15% | 3,20% | 93% | 89% |
| Class F | 7,00% | 5,71% | 4,97% | 95% | 92% |
| Class TGM | 9,38% | 9,29% | 9,47% | 94% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 4,65% | 5,14% | 3,63% | 95% | 86% |
| Class E | 4,82% | 4,13% | 4,17% | 94% | 82% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 4,20% | 4,33% | 3,88% | 94% | 87% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | 8,89% | 9,06% | 9,21% | 95% | 97% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 5,16% | 4,79% | 3,49% | 94% | 81% |
| Class E | 5,27% | 3,99% | 4,10% | 94% | 78% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 4,47% | 4,30% | 3,91% | 98% | 87% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | 8,66% | 8,85% | 9,01% | 96% | 99% |

## Lossless (100 frames)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-7.0 | proposed |  |
| Class A1 | 2,2 | 2,1 | 8,12% |
| Class A2 | 1,6 | 1,5 | 8,89% |
| Class B | 2,2 | 2,0 | 7,38% |
| Class C | 1,9 | 1,8 | 6,98% |
| Class D | 1,9 | 1,8 | 6,84% |
| Class E | 2,8 | 2,6 | 8,06% |
| Class F | 5,3 | 4,9 | 6,91% |
| TGM | 11,8 | 10,8 | 9,52% |
| Overall | 2,1 | 2,0 | 7,78% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 91% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-7.0 | proposed |  |
| Class A1 | 2,2 | 2,0 | -191,38% |
| Class A2 | 1,7 | 0,0 | -300,00% |
| Class B | 2,3 | 2,2 | 6,65% |
| Class C | 2,4 | 2,3 | 6,49% |
| Class D | 2,8 | 2,5 | 7,66% |
| Class E |  |  |  |
| Class F | 33,7 | 30,1 | 10,23% |
| TGM | 107,1 | 96,3 | 10,39% |
| Overall | 2,2 | 1,5 | -28,81% |
| Enc Time[%] | 56% |  |  |
| Dec Time[%] | 49% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-7.0 | proposed |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | 2,3 | 2,2 | 6,78% |
| Class C | 2,4 | 2,3 | 6,39% |
| Class D | 2,7 | 2,5 | 7,53% |
| Class E | 3,1 | 2,9 | 7,31% |
| Class F | 50,7 | 43,0 | 11,89% |
| TGM | 124,9 | 110,5 | 11,56% |
| Overall | 2,6 | 2,4 | 6,78% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 91% |  |  |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).