| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0464-v1 |
| --- | --- |

| Title: | On modifications of intra prediction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Biao Wang, Semih Esenlik, Anand Meher Kotra, Han Gao, Elena Alshina | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com biao.wang@huawei.com semih.esenlik@huawei.com anand.meher.kotra@huawei.com han.gao@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes to modify intra prediction process. In particular, PDPC scaling is harmonized with intra subpartitions (ISP) coding mode, so that the number of reference samples used is maximized. The simulation results for method 1 reportedly show the Y/U/V BD-rate changes of 0.00%/- 0.02%/0.00%, -0.01%/-0.01%/0.03% and -0.04%/0.01%/-0.16% for AI, RA and LDB configurations, respectively. The simulation results for method 2 reportedly show the Y/U/V BD-rate changes of 0.00%/- 0.03%/0.00% and 0.00%/-0.06%/0.07% for AI and RA configurations, respectively. More significant coding efficiency improvements (-0.12%/0.57%/-0.27% and -0.10%/-0.32%/0.24% for method 1 and method 2, respectively) are achieved for class E in LDB case

# Introduction and problem statement

The value of nScale parameter depends on the width or height of the predicted block. However, when a block is coded using intra subpartitions (ISP) mode, the size of this subpartition is used to determine the nScale value.

# Proposed methods

In this proposal, two methods are considered. These methods uses different approach to handle ISP split case. These methods are described in sections below.

## Method 1

In this method, derivation of nScale parameter is performed as follows:

The variable nScale is derived as follows:

- If predModeIntra is equal to INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR18, or INTRA_ANGULAR50, nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 ) >> 2 ).

Otherwise, the following is performed:

- nTbS is set equal to predModeIntra > INTRA_ANGULAR50 ? refH − nTbH : refW − nTbW and nTbS0 equal to predModeIntra > INTRA_ANGULAR50 ? nTbH : nTbW

- when nTbS==nTbS0, nScale is set equal to Min( 2, Log2( nTbS ) − Floor( Log2( 3 * invAngle − 2 ) ) + 8 ),

- otherwise (nTbS!=nTbS0),  the following is performed:

  - nScaleCb = Log2( nTbS) − Floor( Log2( 3 * invAngle − 2 ) ) + 8

  - nScale =  Log2( nTbS0) − Floor( Log2( 3 * invAngle − 2 ) ) + 8

  - nScale = nScale < 0 ? min(0, nScaleCb) : min(2, nScaleCb);

wherein invAngle as specified in clause 8.4.5.2.12. If the value of nScale is less than 0, process 8.4.5.2.14 is terminated, and outputs of this process are set equal to the input predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

## Method 2

In this method it is suggested to modify derivation of nScale parameter in accordance with the modifications to the VVC specification draft [1] as follows:

The variable nScale is derived as follows:

- If predModeIntra is equal to INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR18, or INTRA_ANGULAR50,  nScale is set equal to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 ) >> 2 ).

- Otherwise, the following is performed:

    - nTbS is set equal to predModeIntra > INTRA_ANGULAR50 ? refH − nTbH : refW − nTbW and nTbS0 equal to predModeIntra > INTRA_ANGULAR50 ? nTbH : nTbW

    - if nTbS==nTbS0, nScale is set to Log2( nTbS0) − Floor( Log2( 3 * invAngle − 2 ) ) + 8

    - Otherwise, if Log2( nTbS ) < Floor( Log2( 3 * invAngle − 2 ) ) + 8, nScale is set to – 1,

    - Otherwise nScale is set to 0

using invAngle as specified in clause 8.4.5.2.12. If the value of nScale is less than 0, process 8.4.5.2.14 is terminated, and outputs of this process are set equal to the input predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

# Experimental results

The tests were carried out in accordance with [2] on the basis of VTM-7.0. Results of the tests for method 1 and method 2 are correspondingly shown in Table 1 and Table 2.

Table 1. Method 1 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.11% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.07% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.07% | 100% | 100% |
| Class C | 0.00% | 0.08% | -0.04% | 99% | 100% |
| Class E | -0.01% | -0.09% | 0.15% | 100% | 100% |
| Overall | 0.00% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.09% | 100% | 101% |
| Class F | 0.00% | 0.07% | 0.03% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | 0.08% | 0.05% | 100% | 100% |
| Class B | -0.01% | 0.02% | 0.10% | 100% | 100% |
| Class C | 0.01% | -0.10% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.03% | 100% | 100% |
| Class D | 0.00% | -0.24% | 0.04% | 100% | 100% |
| Class F | -0.01% | -0.01% | -0.02% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.22% | 0.05% | 100% | 99% |
| Class C | -0.01% | -0.13% | -0.34% | 100% | 99% |
| Class E | -0.12% | 0.57% | -0.27% | 100% | 99% |
| Overall | -0.04% | 0.01% | -0.16% | 100% | 99% |
| Class D | -0.04% | 0.25% | -0.06% | 100% | 100% |
| Class F | -0.08% | 0.00% | 0.29% | 100% | 99% |

Table 2. Method 2 simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | 0.16% | 99% | 99% |
| Class A2 | 0.01% | 0.05% | -0.06% | 99% | 99% |
| Class B | 0.01% | -0.09% | -0.11% | 99% | 96% |
| Class C | 0.00% | 0.04% | 0.06% | 99% | 97% |
| Class E | 0.01% | -0.02% | 0.01% | 99% | 96% |
| Overall | 0.00% | -0.03% | 0.00% | 99% | 97% |
| Class D | 0.00% | -0.12% | -0.05% | 100% | 97% |
| Class F | 0.00% | -0.06% | 0.03% | 99% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 99% | 99% |
| Class A2 | -0.02% | 0.01% | 0.01% | 99% | 99% |
| Class B | 0.01% | -0.08% | 0.15% | 99% | 97% |
| Class C | -0.02% | -0.13% | 0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | 0.07% | 99% | 98% |
| Class D | 0.00% | -0.20% | -0.08% | 100% | 100% |
| Class F | -0.07% | -0.03% | 0.00% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.03% | -0.30% | -0.38% | 99% | 97% |
| Class E | -0.10% | -0.32% | 0.24% | 99% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | 0.13% | -0.37% | 99% | 98% |
| Class F | -0.05% | -0.19% | 0.38% | 99% | 97% |

# Conclusion

It is proposed to adopt the modifications to the VVC 8.0 software and to the VVC specification draft 8.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th JVET meeting, Geneva, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).