| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0469 |
| --- | --- |

| Title: | On minimum chroma QT size derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a method of signalling and deriving the minimum size of the chroma quadtree nodes. The proposed method doesn’t have impact on common test condition. It reportedly fixes a signaling issue when the minimum luma coding block size is larger than 4.

# Problem Statement

In current VVC design, when dual tree mode is enabled, the minimum intra chroma quadtree node size is determined by sps_log2_diff_min_qt_min_cb_intra_slice_chroma value and the minimum luma coding block size.

MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma + MinCbLog2SizeY	   (1)

It doesn’t have problem when the minimum luma coding block size is set as current default value of 4. But when minimum luma coding block size is larger the default value, such as 8 samples, and the minimum intra chroma quadtree node size  of 4 samples should be supported, in such case, the value of syntax element sps_log2_diff_min_qt_min_cb_intra_slice_chroma may need to be negative, as calculated below:

sps_log2_diff_min_qt_min_cb_intra_slice_chroma = MinQtLog2SizeIntraC - MinCbLog2SizeY =2 – 3= -1    (2)

This shall not happen because sps_log2_diff_min_qt_min_cb_intra_slice_chroma is designed to be unsigned as in syntax.

Without a fix to this issue, VTM encoder crashes when the minimum luma coding block size is set to larger than 4 and it’s trying to use intra chroma quadtree node size smaller than the luma coding block size. A fix is desirable to support smaller intra chroma quadtree node size when minimum luma coding block size is larger than 4.

# Proposed Method

It is proposed to modify the current signaling and derivation of minimum size of chroma quadtree nodes to make sure that all related signaled syntax elements are in valid value range.

## Proposed Spec Text Change

When intra dual tree mode is enabled, the signaling of syntax sps_log2_diff_min_qt_min_cb_intra_slice_chroma is unchanged, but the semantic may be changed as described as follows:

The horizontal chroma shift ChromaShiftHor is defined as in table-1.

Table 1 - Chroma shift definition

| chroma_format_idc | Chroma format | ChromaShiftHor |
| --- | --- | --- |
| 0 | Monochrome | 0 |
| 1 | 4:2:0 | 1 |
| 2 | 4:2:2 | 1 |
| 3 | 4:4:4 | 0 |

sps_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the default difference between the maximum of 2 and the difference of base 2 logarithm of the minimum size in luma samples and ChromaShiftHor of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in chroma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by pic_log2_diff_min_qt_min_cb_chroma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a CTU with treeType equal to DUAL_TREE_CHROMA is derived as follows:

MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma			 + max(2, MinCbLog2SizeY - ChromaShiftHor)			(3)

# Conclusion

The proposed method fixes an issue with signalling and deriving the minimum size of the chroma quadtree nodes. Without a fix of this issue, current VVC does not allow minimum size of chroma quadtree node size smaller than minimum luma coding block size even when its larger than 4, and VTM would crash when minimum luma coding block size is set to larger than 4. It is proposed to adopt the proposed fix to VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, ISO/IEC JTC1/SC29/WG11 JVET-P2001, Oct. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).