| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0472 |
| --- | --- |

| Title: | AHG15: Quantization matrix signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Le Léannec, Edouard François, Karam Naser | Email: | philippe.delagrange@interdigital.com |

| Source: | InterDigital |
| --- | --- |

_____________________________

# Abstract

This contribution proposes two changes to quantization matrix signalling.

In a first aspect, it removes the scaling_list_pred_mode_flag syntax element, which is said to be mostly redundant.

In a second aspect, scaling_list_copy_mode_flag is replaced by a number (scaling_list_num_coef), which specifies the number of delta-coefficient transmitted, possibly less than matrix size, the missing ones being inferred to zero. This is said to enable efficient QM adjustments at low bit cost.

# A single flag for copy/prediction

The first aspect of this proposal is merely a cleanup. In VVC draft 7, QM syntax uses two flags (scaling_list_copy_mode_flag and scaling_list_pred_mode_flag) to determine if a QM is predicted from another one and if residual coefficients are transmitted.

This can be depicted as follows:

Figure 1: Regular chroma dequantization and inverse transform

The functionality of  the “pred=0” branch (enclosed in dashed line) is mostly redundant with “pred =1” and “id_delta=0” (i.e. default matrix); the only difference is that the scaling matrix predictor is either all-8 (copy=0) or all-16 (default matrix).

It is proposed to remove the “scaling_list_pred_mode_flag” by inferring it to one, and always transmit scaling_list_pred_id_delta (when id is different from {0, 2, 8}). This saves a few bits and simplifies draft text (9 lines are removed).

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag [ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| nextCoef = 0 |  |
| if( id > 13 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

Nota bene: scaling_list_copy_mode_flag can be moved just before if ( !scaling_list_copy_mode_flag[ id ] )

In the semantics, description of scaling_list_pred_mode_flag is removed, description of scaling_list_copy_mode_flag is adjusted, and derivation of ScalingMatrixPred is simplified (one paragraph is removed).

Full draft text modifications are given in attached file.

# Replacing the flag with a number

In a second aspect (mostly independent from the previous one), it is proposed to replace the “copy” flag with an explicit number of delta-coefficients (scaling_list_num_coef), coded in exp-golomb. When scaling_list_num_coef is zero, this takes one bit, the same as a copy flag. When it is non-zero, it allows sending a custom number of delta-coefficients, possibly less than matrix size, with the missing ones inferred to zero.

Figure 1: Partial QM residual

- Since coefficients are dpcm-coded, this means that the last residual coefficient is repeated, e.g. a single delta-coefficient is a global offset from the predictor.

- The low-frequency QM coefficients, which are considered the most significant, are transmitted first. Thus, changing the first few residual coefficients achieves an effective adjustment from the predictor at low cost, because transmitting many zero deltas is costly (1 bit each since they are exp-golomb coded).

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| scaling_list_num_coef | ue(v) |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) { |  |
| nextCoef = 0 |  |
| if( id > 13 && scaling_list_num_coef > 0 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) && i < scaling_list_num_coef ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

Nota bene: in a variant, scaling_list_num_coef could be signalled inside “if ( id != ... )”, and inferred to be equal to matrixSize*matrixSize when not present (i.e. first matrix of a given size).

Semantics changes consist in replacing the description of scaling_list_copy_mode_flag with the description of scaling_list_num_coef, and a few adjustments to the description of scaling_list_delta_coef when referring to scaling_list_copy_mode_flag.

Full draft text modifications are given in attached file.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).