Draft text changes for JVET-Q0473 (AHG15: Demultiplexing joint CbCr before dequantization),
based on JVET-Q0041-v2 (AHG2: Editorial input on VVC draft text).

_____________________________

[…]

      - 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : 2 |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| […] |  |
| } |  |

      - 7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| […] |  |
| } |  |

[…]

      - 7.4.3.3	Sequence parameter set RBSP semantics

[…]

sps_joint_cbcr_enabled_flag equal to 0 specifies that the joint coding of chroma residuals is disabled. sps_joint_cbcr_enabled_flag equal to 1 specifies that the joint coding of chroma residuals is enabled.

same_qp_table_for_chroma equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals. same_qp_table_for_chroma equal to 0 specifies that two chroma QP mapping tables, for Cb and Cr, are signalled in the SPS. When same_qp_table_for_chroma is not present in the bitstream, the value of same_qp_table_for_chroma is inferred to be equal to 1.

qp_table_start_minus26[ i ] plus 26 specifies the starting luma and chroma QP used to describe the i-th chroma QP mapping table. The value of qp_table_start_minus26[ i ] shall be in the range of −26 − QpBdOffset to 36 inclusive. When qp_table_start_minus26[ i ] is not present in the bitstream, the value of qp_table_start_minus26[ i ] is inferred to be equal to 0.

num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 63 + QpBdOffset, inclusive. When num_points_in_qp_table_minus1[ 0 ] is not present in the bitstream, the value of num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_in_val_minus1[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..numQpTables − 1 is derived as follows:

qpInVal[ i ][ 0 ] = −qp_table_start_minus26[ i ] + 26
qpOutVal[ i ][ 0 ] = qpInVal[ i ][ 0 ]
for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j + 1 ] = qpInVal[ i ][ j ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j + 1 ] = qpOutVal[ i ][ j ] + ( delta_qp_in_val_minus1[ i ][ j ] ^ delta_qp_diff_val[ i ][ j ] )
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffset; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(63)
for( j = 0; j <+ num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j ] + 1 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInval[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][j + 1] − qpOutVal[ i ][j ] ) * m + sh ) / ( delta_qp_in_val_minus1[ i ][j] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] + 1 ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When same_qp_table_for_chroma is equal to 1, ChromaQpTable[ 1 ][ k ] is set equal to ChromaQpTable[ 0 ][ k ] for k = −QpBdOffset..63.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffset to 63, inclusive for i = 0.. numQpTables − 1 and j = 0..num_points_in_qp_table_minus1[ i ] + 1.

[…]

      - 7.4.3.4	Picture parameter set RBSP semantics

[…]

pps_cb_qp_offset and pps_cr_qp_offset specify the offsets to the luma quantization parameter Qp′Y used for deriving Qp′Cb and Qp′Cr, respectively. The values of pps_cb_qp_offset and pps_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_cb_qp_offset and pps_cr_qp_offset are not used in the decoding process and decoders shall ignore their value.

pps_slice_chroma_qp_offsets_present_flag equal to 1 indicates that the slice_cb_qp_offset and slice_cr_qp_offset syntax elements are present in the associated slice headers. pps_slice_chroma_qp_offsets_present_flag equal to 0 indicates that these syntax elements are not present in the associated slice headers. When ChromaArrayType is equal to 0, pps_slice_chroma_qp_offsets_present_flag shall be equal to 0.

pps_cu_chroma_qp_offset_list_enabled_flag equal to 1 specifies that the pic_cu_chroma_qp_offset_subdiv_intra_slice and pic_cu_chroma_qp_offset_subdiv_inter_slice syntax elements are present in PHs referring to the PPS and that cu_chroma_qp_offset_flag may be present in the transform unit syntax and the palette coding syntax. pps_cu_chroma_qp_offset_list_enabled_flag equal to 0 specifies that the pic_cu_chroma_qp_offset_subdiv_intra_slice and pic_cu_chroma_qp_offset_subdiv_inter_slice syntax elements are not present in PHs referring to the PPS and that the cu_chroma_qp_offset_flag is not present in the transform unit syntax and the palette coding syntax. When ChromaArrayType is equal to 0, it is a requirement of bitstream conformance that the value of pps_cu_chroma_qp_offset_list_enabled_flag  shall be equal to 0.

chroma_qp_offset_list_len_minus1 plus 1 specifies the number of cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ], syntax elements that are present in the PPS RBSP syntax structure. The value of chroma_qp_offset_list_len_minus1 shall be in the range of 0 to 5, inclusive.

cb_qp_offset_list[ i ] and cr_qp_offset_list[ i ], specify offsets used in the derivation of Qp′Cb and Qp′Cr, respectively. The values of cb_qp_offset_list[ i ] and cr_qp_offset_list[ i ] shall be in the range of −12 to +12, inclusive.

[…]

      - 7.4.8.1	General slice header semantics

When present, the value of the slice header syntax element slice_pic_order_cnt_lsb shall be the same in all slice headers of a coded picture.

The variable CuQpDeltaVal, specifying the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction, is set equal to 0. The variables CuQpOffsetCb and CuQpOffsetCr, specifying values to be used when determining the respective values of the Qp′Cb and Qp′Cr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.

[…]

slice_cb_qp_offset specifies a difference to be added to the value of pps_cb_qp_offset when determining the value of the Qp′Cb quantization parameter. The value of slice_cb_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cb_qp_offset + slice_cb_qp_offset shall be in the range of −12 to +12, inclusive.

slice_cr_qp_offset specifies a difference to be added to the value of pps_cr_qp_offset when determining the value of the Qp′Cr quantization parameter. The value of slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cr_qp_offset + slice_cr_qp_offset shall be in the range of −12 to +12, inclusive.

slice_joint_cbcr_qp_offset specifies a difference to be added to the value of slice_cb_qp_offset  and slice_cr_qp_offset when determining the value of the Qp′Cb and Qp′Cr respectively. The value of slice_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_joint_cbcr_qp_offset is not present, it is inferred to be equal to 0. The values of pps_cb_qp_offset + slice_cb_qp_offset + slice_joint_cbcr_qp_offset and pps_cr_qp_offset + slice_cr_qp_offset + slice_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive.

[…]

      - 7.4.10.10		Transform unit semantics

[…]

cu_chroma_qp_offset_flag when present and equal to 1, specifies that an entry in the cb_qp_offset_list[ ] is used to determine the value of CuQpOffsetCb, and a corresponding entry in the cr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCr. cu_chroma_qp_offset_flag equal to 0 specifies that these lists are not used to determine the values of CuQpOffsetCb and CuQpOffsetCr.

cu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ] and cr_qp_offset_list[ ] that is used to determine the value of CuQpOffsetCb and CuQpOffsetCr. When present, the value of cu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of cu_chroma_qp_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb and CuQpOffsetCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]	(186)

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(187)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb and CuQpOffsetCr are both set equal to 0.

[…]

    - 8.7.1	Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the coding tree node and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb, Qp′Cr, Qp′′Cb and Qp′′Cr are derived.

[…]

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb and qPCr are derived as follows:

qPChroma = Clip3( −QpBdOffset, 63, QpY )		(1118)

qPCb = ChromaQpTable[ 0 ][ qPChroma ]		(1119)

qPCr = ChromaQpTable[ 1 ][ qPChroma ]		(1120)

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr are derived as follows:

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )
	 + QpBdOffset		(1122)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )
	 + QpBdOffset		(1123)

Qp′′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset +
	slice_joint_cbcr_qp_offset + CuQpOffsetCb ) + QpBdOffset	(xxxx)

Qp′′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset +
	slice_joint_cbcr_qp_offset + CuQpOffsetCr ) + QpBdOffset	(xxxx)

    - 8.7.2	Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * pic_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - When cIdx is different from codedCIdx, the transform coefficient levels TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] =
	cSign * TransCoeffLevel[ xTbY ][ yTbY ][ codedCIdx ][ x ][ y ]	(xxxx)

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the prediction mode predMode, and the colour component variable cIdx being set equal to cIdx  as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples res is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ codedCIdx ] is equal to 1, the residual sample array values res[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res[ x ][ y ] = d[ x ][ y ]	(1125)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ codedCIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to cIdx  and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples res.

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If cIdx is equal to codedCIdx and TuCResMode[ xTbY ][ yTbY ] is equal to 0 or 2, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]	(1126)

- Otherwise, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ] >> 1	(1128)

    - 8.7.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2 and cIdx is equal to 1, the following applies:

qP = Qp′′Cb		(1130)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2 and cIdx is equal to 2, the following applies:

qP = Qp′′Cr		(xxxx)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(1132)

[…]

        - 8.8.3.6.3	[Deblocking filter] Decision process for chroma block edges

[…]

The variable QpP is derived as follows:

- The luma location ( xTbP, xTbP ) is set as the top-left luma sample position of the transform block containing the sample p0,0, relative to the top-left luma sample of the picture.

- If TuCResMode[ xTbP ][ yTbP ] is equal to 2 and cIdx is equal to 1, QpP is set equal to Qp′′Cb of the transform block containing the sample p0,0.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2 and cIdx is equal to 2, QpP is set equal to Qp′′Cr of the transform block containing the sample p0,0.

- Otherwise, if cIdx is equal to 1, QpP is set equal to Qp′Cb of the transform block containing the sample p0,0.

- Otherwise, QpP is set equal to Qp′Cr of the transform block containing the sample p0,0.

The variable QpQ is derived as follows:

- The luma location ( xTbQ,  xTbQ ) is set as the top-left luma sample position of the transform block containing the sample q0,0, relative to the top-left luma sample of the picture.

- If TuCResMode[ xTbQ ][ yTbQ ] is equal to 2 and cIdx is equal to 1, QpQ is set equal to Qp′′Cb of the transform block containing the sample q0,0.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2 and cIdx is equal to 2, QpQ is set equal to Qp′′Cr of the transform block containing the sample q0,0.

- Otherwise, if cIdx is equal to 1, QpQ is set equal to Qp′Cb of the transform block containing the sample q0,0.

- Otherwise, QpQ is set equal to Qp′Cr of the transform block containing the sample q0,0.

[note: this may be simplified by just saying “The variable QpP/Q is set equal to qP of the transform block containing …”, provided that further adjustments of qP within 8.7.3 use a different variable name, e.g. qP’ (see ACT, QpPrimeTsMin)]

[…]