| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0473-v2 |
| --- | --- |

| Title: | AHG15: Demultiplexing joint CbCr before dequantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Le Léannec, Edouard François, Philippe Bordes | Email: | philippe.delagrange@interdigital.com |

| Source: | InterDigital |
| --- | --- |

_____________________________

# Abstract

It is proposed to move joint CbCr demultiplexing from after inverse transform to before dequantization, applying regular quantization control to each reconstructed chroma channel, so that most quantization control parameters specific to joint CbCr can be removed. This is said to simplify draft text and improve design consistency. BDRate difference using the CTCs in AI and RA is reported to be 0.00% and +0.01% respectively.

# Current state: demultiplexing CbCr after inverse transform

In VVC draft 7, joint CbCr is demultiplexed after inverse transform, which is illustrated in the following figures:

Figure 1: Regular chroma dequantization and inverse transform

Figure 2: Chroma dequantization and inverse transform in joint CbCr mode

Comparing the two figures, it can be noted that, in joint CbCr mode, the dequantization and inverse transform operators of the second chroma channel are unused, since there are no coded coefficients for the “other” channel.

The “joint” and “other” channels in Figure 2 are either Cb and Cr in this order or swapped, and the “scale” factor is either 1, -1, 1/2, -1/2, depending on chroma cbf flags and a picture-level CSign (either +1 or -1), as detailed in Table 1 below:

Table 1: Joint chroma coded modes in VVC draft 7

| tu_cbf_cb | tu_cbr_cr | Reconstruction of Cb and Cr residuals | TuCResMode | Quant. matrix |
| --- | --- | --- | --- | --- |
| 1 | 0 | resCb[x][y] = resJointC[x][y] resCr[x][y] = (CSign * resJointC[x][y]) >> 1 | 1 | Cb |
| 1 | 1 | resCb[x][y] = resJointC[x][y] resCr[x][y] = CSign * resJointC[x][y] | 2 | Cb |
| 0 | 1 | resCb[x][y] = (CSign * resJointC[x][y]) >> 1 resCr[x][y] = resJointC[x][y] | 3 | Cr |

Demultiplexing Cb and Cr after inverse transform means that what is dequantized is the joint CbCr channel, which may need specific quantization control parameters, distinct from Cb and Cr alone. This is provided by specific syntax elements adopted in VVC, e.g. PPS and slice-level chroma QP offsets (pps_joint_cbcr_qp_offset_value and slice_joint_cbcr_qp_offset), local chroma offset table (joint_cbcr_qp_offset_list), and chroma QP mapping table (see ‘numQpTables’ in SPS).

Almost all cases are covered, except e.g. quantization matrices where the Cb matrix is selected for joint CbCr mode 2 (the mode where Cb and Cr have equal weights, see Table 1).

# Proposal: demultiplexing CbCr before dequantization

It is proposed to move CbCr demultiplexing before dequantization, and remove all specific joint CbCr quantization control parameters, except a PPS or slice-level offset (keep either pps_joint_cbcr_qp_offset_value or slice_joint_cbcr_qp_offset but interpreted as an additional offset on top of Cb and Cr offsets).

The major point here is that both chroma channels are back to work, just as in regular mode, and regular chroma quantization control applies.

Figure 3: Demultiplexing CbCr before dequantization

Of course, this changes the respective weights of Cb and Cr in the joint CbCr mix when the QP of Cb and Cr are different, because the mix is made in the quantized domain. However, this is thought to provide a more consistent chroma dequantization scheme, with simpler control.

There should be very little impact when the QPs of Cb and Cr are equal, as in the CTCs, because operations are mostly linear (thus swappable).

One operation that could introduce a significant difference is the right shift for “other” component, that occurs in joint CbCr modes 1 and 3: quantized coefficients are expected to be small, and right shifting could seriously alter the accuracy. Tests have shown that performing the right shift after inverse transform as illustrated in Figure 4 is preferable to other options, like in-between dequantization and inverse transform, or before dequantization (as in Figure 3) for example. As the joint CbCr right shift can be combined with inverse transform “bdShift”, it should not cause implementation problems.

Figure 4: Demultiplexing CbCr before dequantization, but right shift after inverse transform

The sign change operation (multiplication by CSign, that is derived from pic_joint_cbcr_sign_flag) is performed before dequantization, and tests have shown a benefit in doing so. This should be easy to implement since, before dequantization, sign and absolute level are separate syntax elements, meaning that sign change is just a logical NOT of one bit.

# Conclusion

The current proposal removes most quantization control parameters specific to joint CbCr by moving CbCr demultiplexing before dequantization. It is noted that this may change the respective weights of Cb and Cr in the joint CbCr mix, which then occurs in quantized domain, but is said to improve consistency and simplify control of chroma quantization. Tests using the CTCs show that BDRate performance is not affected (-0.00% in all-intra, and +-0.01% in random-access, combining Y/Cb/Cr numbers with 0.8/0.1/0.1 weights). The draft text is reduced, with 3 syntax elements removed, and implementation should be simpler than current scheme.

Full draft text modifications are given in attached file.

Test software is provided as a patch to be applied over VTM-7.0.

# References

[JVET-P2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October 2019

[JVET-P0608]	S. Iwamura, S. Nemoto, A. Ichigaya (NHK), P. de Lagrange, F. Le Léannec, E. François (InterDigital) “[AHG15] On scaling list for JCCR”, JVET-P0608, Geneva, CH, October 2019

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).