Draft text changes for JVET-Q0474 (AHG15: defining QP at TU level),
based on JVET-Q0041-v2 (AHG2: Editorial input on VVC draft text).

_____________________________

[…]

      - 7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| tu_qp_delta_enabled_flag | u(1) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_tu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_tu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| […] |  |
| } |  |

[…]

      - 7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| if( tu_qp_delta_enabled_flag ) { |  |
| pic_tu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_tu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_tu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_tu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_tu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| […] |  |
| } |  |

[…]

      - 7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| […] |  |
| if( pps_tu_chroma_qp_offset_list_enabled_flag ) |  |
| tu_chroma_qp_offset_enabled_flag | u(1) |
| […] |  |
| } |  |

[…]

      - 7.3.9.2	Coding tree unit syntax

[…]

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( tu_qp_delta_enabled_flag && cbSubdiv <= TuQpDeltaSubdiv ) { |  |
| IsTuQpDeltaCoded = 0 |  |
| TuQpDeltaVal = 0 |  |
| QgTopLeftX = x0 |  |
| QgTopLeftY = y0 |  |
| } |  |
| if( tu_chroma_qp_offset_enabled_flag && cbSubdiv <= TuChromaQpOffsetSubdiv ) |  |
| IsTuChromaQpOffsetCoded = 0 |  |
| […] |  |
| } |  |

[…]

      - 7.3.9.4	Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) && ( x0 + cbWidth <= pic_width_in_luma_samples ) && ( y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| if( tu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= TuQpDeltaSubdiv ) { |  |
| IsTuQpDeltaCoded = 0 |  |
| TuQpDeltaVal = 0 |  |
| QgTopLeftX = x0 |  |
| QgTopLeftY = y0 |  |
| } |  |
| if( tu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= TuChromaQpOffsetSubdiv ) |  |
| IsTuChromaQpOffsetCoded = 0 |  |
| […] |  |
| } |  |

[…]

      - 7.3.9.6	Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| […] |  |
| if( treeType != DUAL_TREE_CHROMA && palette_escape_val_present_flag ) |  |
| if( tu_qp_delta_enabled_flag && !IsTuQpDeltaCoded ) { |  |
| tu_qp_delta_abs | ae(v) |
| if( tu_qp_delta_abs ) |  |
| tu_qp_delta_sign_flag | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) |  |
| if( tu_chroma_qp_offset_enabled_flag && !IsTuChromaQpOffsetCoded ) { |  |
| tu_chroma_qp_offset_flag | ae(v) |
| if( tu_chroma_qp_offset_flag ) |  |
| tu_chroma_qp_offset_idx | ae(v) |
| } |  |
| […] |  |
| } |  |

[…]

      - 7.3.9.10	Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| […] |  |
| if( ( Min( tbWidth, tbHeight ) < 4 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( tu_qp_delta_enabled_flag && !IsTuQpDeltaCoded ) { |  |
| tu_qp_delta_abs | ae(v) |
| if( tu_qp_delta_abs ) |  |
| tu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) && treeType != DUAL_TREE_LUMA ) { |  |
| if( tu_chroma_qp_offset_enabled_flag && !IsTuChromaQpOffsetCoded) { |  |
| tu_chroma_qp_offset_flag | ae(v) |
| if( tu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| tu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| […] |  |
| } |  |

[…]

      - 7.4.3.4	Picture parameter set RBSP semantics

[…]

tu_qp_delta_enabled_flag equal to 1 specifies that the pic_tu_qp_delta_subdiv_intra_slice and pic_tu_qp_delta_subdiv_inter_slice syntax elements are present in PHs referring to the PPS and that tu_qp_delta_abs may be present in the transform unit syntax. tu_qp_delta_enabled_flag equal to 0 specifies that the pic_tu_qp_delta_subdiv_intra_slice and pic_tu_qp_delta_subdiv_inter_slice syntax elements are not present in PHs referring to the PPS and that tu_qp_delta_abs is not present in the transform unit syntax.

[…]

pps_tu_chroma_qp_offset_list_enabled_flag equal to 1 specifies that the pic_tu_chroma_qp_offset_subdiv_intra_slice and pic_tu_chroma_qp_offset_subdiv_inter_slice syntax elements are present in PHs referring to the PPS and that tu_chroma_qp_offset_flag may be present in the transform unit syntax and the palette coding syntax. pps_tu_chroma_qp_offset_list_enabled_flag equal to 0 specifies that the pic_tu_chroma_qp_offset_subdiv_intra_slice and pic_tu_chroma_qp_offset_subdiv_inter_slice syntax elements are not present in PHs referring to the PPS and that the tu_chroma_qp_offset_flag is not present in the transform unit syntax and the palette coding syntax. When ChromaArrayType is equal to 0, it is a requirement of bitstream conformance that the value of pps_tu_chroma_qp_offset_list_enabled_flag  shall be equal to 0.

[…]

      - 7.4.3.6	Picture header RBSP semantics

[…]

pic_tu_qp_delta_subdiv_intra_slice specifies the maximum cbSubdiv value of coding units in intra slice that convey tu_qp_delta_abs and tu_qp_delta_sign_flag. The value of pic_tu_qp_delta_subdiv_intra_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeIntraY + pic_max_mtt_hierarchy_depth_intra_slice_luma ), inclusive.

When not present, the value of pic_tu_qp_delta_subdiv_intra_slice is inferred to be equal to 0.

pic_tu_qp_delta_subdiv_inter_slice specifies the maximum cbSubdiv value of coding units that in inter slice convey tu_qp_delta_abs and tu_qp_delta_sign_flag. The value of pic_tu_qp_delta_subdiv_inter_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeInterY + pic_max_mtt_hierarchy_depth_inter_slice ), inclusive.

When not present, the value of pic_tu_qp_delta_subdiv_inter_slice is inferred to be equal to 0.

pic_tu_chroma_qp_offset_subdiv_intra_slice specifies the maximum cbSubdiv value of coding units in intra slice that convey tu_chroma_qp_offset_flag. The value of pic_tu_chroma_qp_offset_subdiv_intra_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeIntraY + pic_max_mtt_hierarchy_depth_intra_slice_luma ), inclusive.

When not present, the value of pic_tu_chroma_qp_offset_subdiv_intra_slice is inferred to be equal to 0.

pic_tu_chroma_qp_offset_subdiv_inter_slice specifies the maximum cbSubdiv value of coding units in inter slice that convey tu_chroma_qp_offset_flag. The value of pic_tu_chroma_qp_offset_subdiv_inter_slice shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeInterY + pic_max_mtt_hierarchy_depth_inter_slice ), inclusive.

When not present, the value of pic_tu_chroma_qp_offset_subdiv_inter_slice is inferred to be equal to 0.

[…]

      - 7.4.4.2	General constraint information semantics

[…]

no_qp_delta_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that tu_qp_delta_enabled_flag shall be equal to 0. no_qp_delta_constraint_flag equal to 0 does not impose such a constraint.

[…]

      - 7.4.8.1	General slice header semantics

When present, the value of the slice header syntax element slice_pic_order_cnt_lsb shall be the same in all slice headers of a coded picture.

The variable TuQpDeltaVal, specifying the difference between a luma quantization parameter for the transform unit containing tu_qp_delta_abs and its prediction, is set equal to 0. The variables TuQpOffsetCb, TuQpOffsetCr, and TuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the transform unit containing tu_chroma_qp_offset_flag, are all set equal to 0.

[…]

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When nal_unit_type is a value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the current picture is the first picture in an access unit, slice_type shall be equal to 2.

The variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

If slice_type equal to 2 (I),

MinQtLog2SizeY = MinCbLog2SizeY + pic_log2_diff_min_qt_min_cb_intra_slice_luma	(121)

MinQtLog2SizeC = MinCbLog2SizeY + pic_log2_diff_min_qt_min_cb_intra_slice_chroma	(122)

MaxBtSizeY =  1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_bt_min_qt_intra_slice_luma )	(123)

MaxBtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_bt_min_qt_intra_slice_chroma )	(124)

MaxTtSizeY = 1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_tt_min_qt_intra_slice_luma )	(125)

MaxTtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_tt_min_qt_intra_slice_chroma )	(126)

MaxMttDepthY = pic_max_mtt_hierarchy_depth_intra_slice_luma	(127)

MaxMttDepthC =  pic_max_mtt_hierarchy_depth_intra_slice_chroma	(128)

TuQpDeltaSubdiv = pic_tu_qp_delta_subdiv_intra_slice	(129)

TuChromaQpOffsetSubdiv = pic_tu_chroma_qp_offset_subdiv_intra_slice	(130)

Otherwise (slice_type equal to 0 (B) or 1 (P)),

MinQtLog2SizeY = MinCbLog2SizeY + pic_log2_diff_min_qt_min_cb_inter_slice	(131)

MinQtLog2SizeC = MinCbLog2SizeY + pic_log2_diff_min_qt_min_cb_inter_slice	(132)

MaxBtSizeY = 1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_bt_min_qt_inter_slice)	(133)

MaxBtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_bt_min_qt_inter_slice)	(134)

MaxTtSizeY = 1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_tt_min_qt_inter_slice)	(135)

MaxTtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_tt_min_qt_inter_slice)	(136)

MaxMttDepthY = pic_max_mtt_hierarchy_depth_inter_slice	(137)

MaxMttDepthC = pic_max_mtt_hierarchy_depth_inter_slice	(138)

TuQpDeltaSubdiv = pic_tu_qp_delta_subdiv_inter_slice	(139)

TuChromaQpOffsetSubdiv = pic_tu_chroma_qp_offset_subdiv_inter_slice	(140)

MinQtSizeY = 1  <<  MinQtLog2SizeY		(117)

MinQtSizeC = 1  <<  MinQtLog2SizeC		(118)

MinBtSizeY = 1  <<  MinCbLog2SizeY		(119)

MinTtSizeY = 1  <<  MinCbLog2SizeY		(120)

[…]

      - 7.4.10.10		Transform unit semantics

[…]

tu_qp_delta_abs specifies the absolute value of the difference TuQpDeltaVal between the quantization parameter of the current transform unit and its prediction.

tu_qp_delta_sign_flag specifies the sign of TuQpDeltaVal as follows:

- If tu_qp_delta_sign_flag is equal to 0, the corresponding TuQpDeltaVal has a positive value.

- Otherwise (tu_qp_delta_sign_flag is equal to 1), the corresponding TuQpDeltaVal has a negative value.

When tu_qp_delta_sign_flag is not present, it is inferred to be equal to 0.

When tu_qp_delta_abs is present, the variables IsTuQpDeltaCoded and TuQpDeltaVal are derived as follows:

IsTuQpDeltaCoded = 1		(184)

TuQpDeltaVal = tu_qp_delta_abs * ( 1 − 2 * tu_qp_delta_sign_flag )	(185)

The value of TuQpDeltaVal shall be in the range of −( 32 + QpBdOffset / 2 ) to +( 31 + QpBdOffset / 2 ), inclusive.

tu_chroma_qp_offset_flag when present and equal to 1, specifies that an entry in the cb_qp_offset_list[ ] is used to determine the value of TuQpOffsetCb, a corresponding entry in the cr_qp_offset_list[ ] is used to determine the value of TuQpOffsetCr, and a corresponding entry in the joint_cbcr_qp_offset_list[ ] is used to determine the value of TuQpOffsetCbCr. tu_chroma_qp_offset_flag equal to 0 specifies that these lists are not used to determine the values of TuQpOffsetCb, TuQpOffsetCr, and TuQpOffsetCbCr.

tu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] that is used to determine the value of TuQpOffsetCb, TuQpOffsetCr, and TuQpOffsetCbCr. When present, the value of tu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of tu_chroma_qp_offset_idx is inferred to be equal to 0.

When tu_chroma_qp_offset_flag is present, the following applies:

- The variable IsTuChromaQpOffsetCoded is set equal to 1.

- The variables TuQpOffsetCb, TuQpOffsetCr, and TuQpOffsetCbCr are derived as follows:

- If tu_chroma_qp_offset_flag is equal to 1, the following applies:

TuQpOffsetCb = cb_qp_offset_list[ tu_chroma_qp_offset_idx ]	(186)

TuQpOffsetCr = cr_qp_offset_list[ tu_chroma_qp_offset_idx ]	(187)

TuQpOffsetCbCr = joint_cbcr_qp_offset_list[ tu_chroma_qp_offset_idx ]	(188)

- Otherwise (tu_chroma_qp_offset_flag is equal to 0), TuQpOffsetCb, TuQpOffsetCr, and TuQpOffsetCbCr are all set equal to 0.

[…]

    - 8.4.1	General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

[…]

    - 8.5.1	General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

[…]

    - 8.6.1	General decoding process for coding units coded in IBC prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

[…]

    - 8.7.1	Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xTb, yTb ) specifying the top-left luma sample of the current transform block relative to the top-left luma sample of the current picture,

- a variable tbWidth specifying the width of the current transform block in luma samples,

- a variable tbHeight specifying the height of the current transform block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the coding tree node and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb, Qp′Cr and Qp′CbCr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to QgTopLeftX and QgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables QgTopLeftX and QgTopLeftY.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a tile.

–	The current quantization group is the first quantization group in a CTB row of a tile and entropy_coding_sync_enabled_flag is equal to 1.

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma transform unit in the previous quantization group in decoding order.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xTb, yTb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	The CTB containing the luma transform block covering the luma location ( xQg − 1, yQg ) is not equal to the CTB containing the current luma transform block at ( xTb, yTb ), i.e. all of the following conditions are true:

–	( xQg − 1 )  >>  CtbLog2SizeY is not equal to ( xTb )  >>  CtbLog2SizeY

–	( yQg )  >>  CtbLog2SizeY is not equal to ( yTb )  >>  CtbLog2SizeY

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xTb, yTb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	The CTB containing the luma transform block covering the luma location ( xQg, yQg − 1 ) is not equal to the CTB containing the current luma transform block at ( xTb, yTb ), i.e. all of the following conditions are true:

–	( xQg )  >>  CtbLog2SizeY is not equal to ( xTb )  >>  CtbLog2SizeY

–	( yQg − 1 )  >>  CtbLog2SizeY is not equal to ( yTb )  >>  CtbLog2SizeY

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

- If all the following conditions are true, then qPY_PRED is set equal to the luma quantization parameter QpY of the transform unit containing the luma transform block covering ( xQg, yQg − 1 ):

- availableB is equal to TRUE.

- The current quantization group is the first quantization group in a CTB row within a tile.

- Otherwise, qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(1115)

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + TuQpDeltaVal + 64 + 2 * QpBdOffset ) % ( 64 + QpBdOffset ) ) − QpBdOffset	(1116)

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffset		(1117)

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma transform unit that covers the luma location ( xTb + tbWidth / 2, yTb + tbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPChroma = Clip3( −QpBdOffset, 63, QpY )		(1118)

qPCb = ChromaQpTable[ 0 ][ qPChroma ]		(1119)

qPCr = ChromaQpTable[ 1 ][ qPChroma ]		(1120)

qPCbCr = ChromaQpTable[ 2 ][ qPChroma ]		(1121)

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset + TuQpOffsetCb )
	 + QpBdOffset		(1122)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset + TuQpOffsetCr )
	 + QpBdOffset		(1123)

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset + 
	slice_joint_cbcr_qp_offset +TuQpOffsetCbCr ) + QpBdOffset	(1124)

    - 8.7.2	Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * pic_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xTbY, yTbY ), the width of the current transform block in luma samples nTbW * ( cIdx > 0 ? SubWidthC : 1) and the height of the current transform block nTbH * ( cIdx > 0 ? SubHeightC : 1), and the variable treeType as inputs.

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the prediction mode predMode, and the colour component variable cIdx being set equal to codedCIdx as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

[…]

        - 8.8.3.6.1	[Deblocking filter] Decision process for luma block edges

[…]

The variables QpQ and QpP are set equal to the QpY values of the transform units which include the transform blocks containing the sample q0,0 and p0,0, respectively.

[…]

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| […] |  |  |  |  |  |  |
| transform_unit( ) | tu_cbf_luma[ ][ ] | Table 108 | 0..3 | 4..7 | 8..11 |  |
|  | tu_cbf_cb[ ][ ] | Table 109 | 0..1 | 2..3 | 4..5 |  |
|  | tu_cbf_cr[ ][ ] | Table 110 | 0..2 | 3..5 | 6..8 |  |
|  | tu_qp_delta_abs | Table 111 | 0..1 | 2..3 | 4..5 |  |
|  | tu_chroma_qp_offset_flag | Table 112 | 0 | 1 | 2 |  |
|  | tu_chroma_qp_offset_idx | Table 113 | 0 | 1 | 2 |  |
|  | transform_skip_flag[ ][ ][ ] | Table 114 | 0..1 | 2..3 | 4..5 |  |
|  | tu_joint_cbcr_residual_flag[ ][ ] | Table 115 | 0..2 | 3..5 | 6..8 |  |
| […] |  |  |  |  |  |  |

[…]

| Table 123 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| […] |  |  |  |
| transform_unit( ) | tu_cbf_luma[ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cb[ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cr[ ][ ] | FL | cMax = 1 |
|  | tu_qp_delta_abs | 9.3.3.10 | - |
|  | tu_qp_delta_sign_flag | FL | cMax = 1 |
|  | tu_chroma_qp_offset_flag | FL | cMax = 1 |
|  | tu_chroma_qp_offset_idx | TR | cMax = chroma_qp_offset_list_len_minus1, cRiceParam = 0 |
|  | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | tu_joint_cbcr_residual_flag[ ][ ] | FL | cMax = 1 |
| […] |  |  |  |

[…]

      - 9.3.3.10	Binarization process for tu_qp_delta_abs

Input to this process is a request for a binarization for the syntax element tu_qp_delta_abs.

Output of this process is the binarization of the syntax element.

The binarization of the syntax element tu_qp_delta_abs is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of tu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( tu_qp_delta_abs, 5 )		(1501)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with cMax = 5 and cRiceParam = 0.

When prefixVal is greater than 4, the suffix bin string is present and it is derived as follows:

- The suffix value of tu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = tu_qp_delta_abs − 5		(1502)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.3.3.5 for suffixVal with the Exp-Golomb order k set equal to 0.

[…]

| Table 128 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| […] |  |  |  |  |  |  |
| tu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| tu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| tu_chroma_qp_offset_flag | 0 | na | na | na | na | na |
| tu_chroma_qp_offset_idx | 0 | 0 | 0 | 0 | 0 | na |
| […] |  |  |  |  |  |  |