| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0474 |
| --- | --- |

| Title: | AHG15: defining QP at TU level |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Le Léannec, Fabrice Urban, Karam Naser | Email: | philippe.delagrange@interdigital.com |

| Source: | InterDigital |
| --- | --- |

_____________________________

# Abstract

This contribution proposes to define QP at the same level QP-delta is signaled, i.e. TU level instead of CU level. This changes the way QP is assigned to leading TUs that do not carry coded coefficients within a CU that carries a QP-delta, removing QP-delta back-propagation and aligning the process with leading CUs without coded coefficients within a QG. This is said to be a cleanup, which is mostly a semantic change, and removes the need for fixes adopted from JVET-O0046 and JVET-P0436 by solving the root cause.

# Current state: QP defined at CU level, but signaled at TU level

In VVC draft 7, local QP adjustment, e.g. cu_qp_delta_abs, is signaled once per Quantization Group (QG) as soon as the current TU contains coded coefficients (i.e. cbf is not zero). However, a valid QP is needed for all image blocks, even those without any coded coefficients, for several reasons:

- In dual-tree, the variable QpY used to derive chroma QPs is taken from the collocated luma block, which may or may not carry coded coefficients.

- Deblocking filter uses both luma and chroma QPs to take decisions on both sides of a boundary, regardless of whether underlying blocks carry coded coefficients or not.

- Luma QP prediction uses the QP of neighboring blocks, which may or may not carry coded coefficients.

The QP of blocks that do not carry coded coefficients is defined as follows:

For blocks that follow (in coding order) the one that carries QP-delta in a given QG, QP-delta (CuQpDeltaVal) is known, so it is applied to compute block QP: QpY = qPY_PRED + CuQpDeltaVal

But for leading blocks in a QG that do not carry coded coefficients, QP-delta in unknown, and the rule is different when considering CUs or TUs, because QP is defined at CU level.

Leading CUs that do not carry coded coefficients are assigned the predicted QP: QpY = qPY_PRED

However, leading TUs within a CU that carries a QP-delta, the rule is different because the QP applies to the whole CU. Thus, QP-delta is back-propagated to leading TUs.

This can happen with ISP, SBT, or when the CU is larger than maximum TU size (e.g. 128x128 CU). For the 128x128 CU case, back-propagation has been prevented by adoption of JVET-O0046 and JVET-P0436, to match VPDU constraints, by forcing transmission of QP-delta in the first TU.

Mostly luma QP was considered in the previous paragraphs, but similar considerations apply for chroma QP adjustments.

# Proposal: QP defined at TU level

It is proposed to remove all back-propagation of QP-delta by defining QP at TU level, which is argued to be more consistent because QP-delta is signaled at TU level. This is mostly a semantic change, with some renaming work in the syntax.

The fixes of JVET-O0046 and JVET-P0436 are not needed any more because the root cause is addressed.

However, an implementation issue may arise because of ISP, which may refine the granularity of TUs (hence QP) to less than 4x4 which is the granularity of CUs (hence granularity of QPs in VVC draft 7). This may increase decoder SRAM needs to store QP. To avoid this, the transmission of QP-delta can be forced in the first TU when TU size is less than 4 in either direction. This has been included in current proposal.

# Draft text changes

Example of renaming work in syntax in coding_tree syntax structure is given below:

| coding_tree( x0, y0, [...], qgOnY, qgOnC, cbSubdiv, [...] ) { | Descriptor |
| --- | --- |
| [...] |  |
| if( tucu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= TuCuQpDeltaSubdiv ) { |  |
| IsTuCuQpDeltaCoded = 0 |  |
| TuCuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( tucu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= TuCuChromaQpOffsetSubdiv ) |  |
| IsTuCuChromaQpOffsetCoded = 0 |  |
| [...] |  |

More renaming work in transform_unit is show below, along with removed conditions for big CUs and added condition for small TUs as said above:

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| [...] |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ Min( tbWidth, tbHeight ) < 4 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( tucu_qp_delta_enabled_flag && !IsTuCuQpDeltaCoded ) { |  |
| tucu_qp_delta_abs | ae(v) |
| if( tucu_qp_delta_abs ) |  |
| tucu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ) && treeType != DUAL_TREE_LUMA ) { |  |
| if( tucu_chroma_qp_offset_enabled_flag && !IsTuCuChromaQpOffsetCoded) { |  |
| tucu_chroma_qp_offset_flag | ae(v) |
| if( tucu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| tucu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| [...] |  |

On the semantics side, example changes in derivation process for QP and deblocking filter process are shown below:

8.7.1	Derivation process for quantization parameters

[...]

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If [...]

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma transformcoding unit in the previous quantization group in decoding order.

- The derivation process for neighbouring block availability [...]. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

– [...]

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the transformcoding unit containing the luma coding block covering ( xQg − 1, yQg ).

[...]

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma transformcoding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

[...]

8.8.3		Deblocking filter process	
8.8.3.6	Edge filtering process for one direction	
8.8.3.6.1	Decision process for luma block edges

[...]

The variables QpQ and QpP are set equal to the QpY values of the transformcoding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

[...]

# Conclusion

This contribution proposes to define QP at the same level QP-delta is specified in the syntax, i.e. TU level instead of CU level, which removes back-propagation of QP-delta to leading TUs without coded coefficients within a CU that carries a QP-delta. This is said to solve the root cause of the problems raised by JVET-O0046 and JVET-P0436, by providing consistent causal assignment of QP to all image blocks that do not contain coded coefficients. Potential implementation issues caused by TUs smaller than 4xN/Nx4 are addressed.

Full draft text modifications are given in attached file.

# References

[JVET-P2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October 2019

[JVET-O0046]	B. Heng, M. Zhou, T. Hellman, “AHG16: Changing cu_qp_delta signalling to facilitate VDPU-level processing”, JVET-O0046, Gothenburg, SE, July 2019

[JVET-P0436]	J. Zhao, S. Paluri, S. Kim, “AHG15: On CU Adaptive Chroma QP Offset Signalling”, JVET-P0436, Geneva, CH, October 2019

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).