| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, 6–17 January 2020 | Document: JVET-Q0476 |
| --- | --- |

| Title: | On chroma QP offsets for zero-CBF leading chroma coding blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Bappaditya Ray Geert Van der Auwera Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the 16th JVET meeting, deblocking parameter derivation was modified to include block-level chroma QP offsets. However, it is asserted that block-level chroma QP offsets for the leading coding blocks in chroma “quantization groups” (QGs) with zero CBF values (ZCBF leading chroma CBs) are not initialized at the beginning of the chroma QGs (except the first chroma QG of the slice). As a result, chroma QP offset values for the ZCBF leading chroma CBs is derived from preceding chroma QGs. It is assserted that this was perhaps a bug or an accidental omission. It is proposed to initialize the chroma QP offset variables (CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr) to zero at the beginning of the chroma QG such that the ZCBF leading chroma CBs have zero (block-level) chroma QP offset. It is asserted that this proposed method aligns with the initialization of luma delta QP variables at the beginning of luma quantization groups.

# Introduction

In JVET-P1001, the deblocking parameter derivation was modified to consider block-level chroma QP offsets in the derivation of β and tC. The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr in VVC Draft 7 are not initialized to zero at the beginning of a chroma quantization group (QG). It is to be noted that although chroma QG is not explicitly defined in the specification, it is used in this document to denote the chroma region where explicit chroma QP offset may be signalled (IsCuChromaQpOffsetCoded is set to zero at the start of a chroma QG).

For the ZCBF leading chroma CBs in a chroma QG, the absence of intialization results in CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr variables to be copied from preceding chroma QG in decoding order. It is the authors’ opinion that this was not the intended purpose of JVET-P1001, and the omission of initialization of chroma CuQpOffset variables was accidental.

This issue was brought to attention in the bug tracker (#659). One suggestion was to initialize the chroma QP offset variables to zero at the beginning of a chroma QG. It is to be noted that the ZCBF leading luma CBs in a luma QG already have delta QP value have intialized to zero (the variable CuQpDeltaVal is set to zero when IsCuQpDeltaCoded is initalized to zero at the beginning of a luma QG).

# Proposal

It is the authors’ opinion that the above suggested method is a straight-forward fix for the above problem and is aligned with the treatment of luma delta QP for ZCBF leading luma CBs of a luma QG. Therefore, it is proposed to initialize the CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr variables at the start of a chroma QG to be zero. This will align the treatment of the ZCBF leading chroma CBs in chroma QGs with that of the ZCBF leading luma CBs in luma QGs. The changes in the Specification associated with this proposal are highlighted below.

# Changes to the Specification

## Changes to dual tree syntax structure

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= CuQpDeltaSubdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= CuChromaQpOffsetSubdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuQpOffsetCb = 0 |  |
| CuQpOffsetCr = 0 |  |
| CuQpOffsetCbCr = 0 |  |
| } |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| … |  |

## Changes to coding tree syntax structure

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= CuQpDeltaSubdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= CuChromaQpOffsetSubdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuQpOffsetCb = 0 |  |
| CuQpOffsetCr = 0 |  |
| CuQpOffsetCbCr = 0 |  |
| } |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).