| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0478-v1 |
| --- | --- |

| Title: | Non-CE1: QP fix for deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

JVET-P1001 and JVET-P1002 proposed to align chroma deblocking QP to dequantization QP. However, in the current draft, it is not the case for transform skip coded blocks. This document proposes to clean it up, which leads to a full alignment. It is asserted that the proposed change does not influence CTC performance.

# Introduction

JVET-P1001 [1] and JVET-P1002 [2] proposed to employ the dequantization QP in the chroma deblocking process and were adopted at the last meeting. However, for Transform Skip (TS) coded blocks, the current deblocking always uses the non-TS QPs which could be different from those used in the dequantization process during a clipping operation. Such a design is conflicted with the principle of the adopted proposals.

# Proposed method

In the current draft, for TS-coded blocks, the derived QP needs to be further clipped to be no smaller than QpPrimeTsMin. However, the QpPrimeTsMin is not considered in the deblocking process, which makes deblocking QP different from the dequantization QP. In addition, since QpPrimeTsMin can be in the range of [4, 52], the difference between the dequantization QP and the one used in deblocking filter may be large and influences the deblocking process.

This document proposed to unify the solution that for both TS coded and non-TS coded blocks, the luma and chroma QPs in the dequantization process is used in the deblocking processes.

The text changes based on JVET-P2001-vE [3] are shown as below. The newly added texts are highlight by grey.

##### 8.8.3.6.1 Decision process for luma block edges

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a location ( xBl, yBl ) specifying the top-left sample of the current block relative to the top-left sample of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable bS specifying the boundary filtering strength,

- a variable maxFilterLengthP specifying the maximum filter length,

- a variable maxFilterLengthQ specifying the maximum filter length.

Outputs of this process are:

- the variables dE, dEp and dEq containing decisions,

- the modified filter length variables maxFilterLengthP and maxFilterLengthQ,

- the variable tC.

…

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

If the transform_skip_flag of the coding unit containing the sample q0,0 is equal to 1, QpQ is modified to be:

QpQ = Max( QpPrimeTsMin, QpQ + QpBdOffset ) - QpBdOffset

If the transform_skip_flag of the coding unit containing the sample p0,0 is equal to 1, QpP is modified to be:

Qpp = Max( QpPrimeTsMin, Qpp + QpBdOffset ) - QpBdOffset

##### …

##### 8.8.3.6.3 Decision process for chroma block edges

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

- a chroma picture sample array recPicture,

- a chroma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left chroma sample of the current picture,

- a chroma location ( xBl, yBl ) specifying the top-left sample of the current chroma block relative to the top-left sample of the current chroma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component index,

- a variable bS specifying the boundary filtering strength,

- a variable maxFilterLengthP specifying the maximum filter length,

- a variable maxFilterLengthQ specifying the maximum filter length.

Outputs of this process are

- the modified filter length variables maxFilterLengthP and maxFilterLengthQ,

- the variable tC.

…

The variable QpP is derived as follows:

- The luma location ( xTbP, xTbP ) is set as the top-left luma sample position of the transform block containing the sample p0,0, relative to the top-left luma sample of the picture.

- If TuCResMode[ xTbP ][ yTbP ] is equal to 2, QpP is set equal to Qp′CbCr of the transform block containing the sample p0,0.

- Otherwise, if cIdx is equal to 1, QpP is set equal to Qp′Cb of the transform block containing the sample p0,0.

- Otherwise, QpP is set equal to Qp′Cr of the transform block containing the sample p0,0.

If transform_skip_flag[ xTbP ][ yTbP ][cIdx ] is equal to 1, the variable QpP is modified as follows:

QpP = Max( QpPrimeTsMin, QpP )

The variable QpQ is derived as follows:

- The luma location ( xTbQ,  xTbQ ) is set as the top-left luma sample position of the transform block containing the sample q0,0, relative to the top-left luma sample of the picture.

- If TuCResMode[ xTbQ ][ yTbQ ] is equal to 2, QpQ is set equal to Qp′CbCr of the transform block containing the sample q0,0.

- Otherwise, if cIdx is equal to 1, QpQ is set equal to Qp′Cb of the transform block containing the sample q0,0.

- Otherwise, QpQ is set equal to Qp′Cr of the transform block containing the sample q0,0.

If transform_skip_flag[ xTbQ ][ yTbQ ][cIdx ] is equal to 1, the variable QpQ is modified as follows:

QpQ = Max( QpPrimeTsMin, QpQ )

- The variable QpC is derived as follows:

QpC = ( QpQ − QpBdOffset + QpP − QpBdOffset + 1 )  >>  1	(1321)

…

# Simulation results

The proposed method does not change the CTC performance compared to VTM7.

# Conclusion

This document proposed to fix the QPs derivation in the deblocking process. It is recommended to adopt the proposed changes to the next version of VVC draft.

# References

- A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina, “Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539)”, JVET-P1001, Oct. 2019

- J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu “Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539)”, JVET-P1002, Oct. 2019

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).