| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-xxxxx |
| --- | --- |

| Title: | VVC conformance testing draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Elena Alshina Kei Kawamura Iole Moccagatta Sean McCarthy Karsten Suehring Wade Wan | Email: | jill.boyce@intel.com elena.alshina@huawei.com ki-kawamura@kddi.com iole.moccagatta@intel.com Sean.McCarthy@dolby.com karsten.suehring@hhi.fraunhofer.de wade.wan@broadcom.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 1 of a conformance testing specification for Versatile Video Coding (VVC).

# General

This document provides a preliminary description of a conformance testing bitstream set and testing process for VVC. See clause 3 for a preliminary list of planned bitstreams and volunteered contributors. It is encouraged for participants to contribute additional bitstreams that test other features and combinations.

# Procedure

## ftp site

An ftp site at ITU-T is used to exchange bitstreams. The site for ftp download of bitstreams is:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

(user id: avguest, password: Avguest201007)

ftp encryption should be set to “Require implicit ftp over TLS”. If using FileZilla, set user id and ftp encryption by configuring site manager as follows:

The site is also accessible by HTTP (without a password) at:

http://wftp3.itu.int/av-arch/jvet-site/bitstream_exchange/VVC

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation will be available at this directory. (See clause 3 for a preliminary list of planned bitstreams.) It includes the list of bitstreams, configuration features and settings, and status of verification.

The ftp site for uploading bitstream files is as follows:

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, password: Avguest201007)

ftp encryption should be set to “Require implicit ftp over TLS”. If using FileZilla, do so by configuring site manager Encryption to “Require implicit ftp over TLS”.

The guest account has write access to the dropbox, but not read access. Once a bitstream is uploaded to the dropbox, please send email to xx@xx.com to notify the coordinators of its availability. After some potential checking, the bitstream will be moved to the downloading area of the ftp site.

## Bitstream and associated files

Volunteers should upload the bitstream and associated files in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

*.bit – bitstream as described in section 2.3 (mandatory)

*.txt – description (mandatory)

*.yuv.md5 – MD5  check sum of the complete decoded yuv file (mandatory)

*.md5 – MD5sum of the bitstream file (mandatory)

*.opl – output picture log as described in section 2.4 (mandatory)

*.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

All bitstreams are required to use the decoded picture hash SEI message to enable validity checking.

The following naming convention is to be used.

Feature_BistreamID_Source_VersionLetter

- Feature is summarized in Sec. 2

- BitstreamID is a single capital letter, used when more than one bitstream is used for a particular feature.

- Source is the name of the company who provided the bitstream. [Ed. (JB): Should this still be included in the file names?]

- Version is the version number of the file, in case a bitstream must be replaced because of an identified problem (starting from 1.)

For example, if you archive is named "feature_A_companyA_1.zip", the files would be named

feature_A_companyA_1.bit

feature_A_companyA_1.txt

feature_A_companyA_1.yuv.md5

feature_A_companyA_1.md5

feature_A_companyA_1.opl

feature_A_companyA_1.cfg

## Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information should be provided.

- Bitstream file name

- Explanation of bitstream features

- Profile, tier, and level for the bitstream

- Max picture width and height, and any additional picture width and height values used (i.e., for reference picture resample or scalability)

- Picture rate

- VTM decoder release version number able to correctly decode the bitstream with identical output, or description of VTM decoder incompatibility (preferably including a corresponding error tracking issue number.)

- VTM release version number used to generate the bitstream, if applicable

- Contact name and email (to be confirmed; please check with the coordinators)

## Output picture log file for each output layer set (name.opl)

Per picture in output order: Picture width, picture height, MD5sum for each of Y, U, and V

Each output picture log file contains one row for each output picture in the bitstream, in output order.

Each row contains the following information, as CSV.

- PicOrderCntVal

- pic_width_max_in_luma_samples

- pic_height_max_in_luma_samples

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

[Ed. (JB): VTM8.0 is expected to provide a decoder option to output the .opl file.]

## Acceptance of bitstreams

The uploaded bitstreams should be verified by at least two organizations. The following aspects should be verified.

- Confirm if bitstream is correctly decoded with perfect match of the .yuv.md5 and .opl files associated with the bitstream

- Confirm if all intended features are included in the bitstream

## Organization of ftp site file directory structure

The ftp site has 2 high level directories, named draft_conformance and under_test.

In the under_test directory, bitstream zip files are placed in a sub-directory corresponding to a VTM decoder release number which can correctly decode the bitstream, or in the VTM-incompatible directory if there is no VTM version that can correctly decode the bitstream.

## Recommendations

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases.

It is recommended to include both low resolution (512x512) and short duration (10 frame) test bitstreams as well as typical resolutions and longer duration test bitstreams for each test category.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

# Conformance bitstream list

The conformance bitstreams are listed in three categories, described below.

Table 1: Coding tools

- Streams with a minimum “basic” set of coding tools, where all the coding tools for which an enable flag is present in the syntax has the enable flag set to 0 to disable the corresponding tool.

- Streams with a single coding tool enabled.

- Streams with sets of varying combinations of coding tools enabled.

The purpose of these streams is to check that the decoder can correctly decode combination of tools and particular cases due to the interaction of tools and coding modes (i.e. IBC and dual tree, LMCS and dual tree, palette and DB, DB and trans-quant bypass, etc.).

In addition, Table 1 contains streams whose purpose is to check that the decoder can properly decode minimum and maximum parameters and syntax elements of basic coding tools, i.e. CTU partition, transform, quantization, etc.

Table 2: Features and high-level syntax

- Streams with multiple coding tools and high-level syntax features.

The purpose of these streams is to check that the decoder can correctly decode combination of tools and features & High Level Syntax, and particular cases due to the interaction between tools and features & High Level Syntax (i.e. loop filters across slices and sub-pictures, loop filters and virtual boundaries, etc.). These streams also check that the decoder can correctly decode all the features supported in VVC (i.e. spatial scalability, gradual decoder refresh, etc.).

Table 3: Bit depths and chroma formats

- Streams with all allowable combinations of bit-depth and chroma formats for the supported profiles

4:4:4 specific coding tools should be enabled in 4:4:4 streams.

Table 1: Coding tools

(10-bit 4:2:0 is used in all the bitstreams in this table.)

| Categories | Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- | --- |
| All | Tool set 0 | SET0 | Basic set, with all tools with enable flags disabled |  |
| All | Tool set 1 | SET1 | ???? |  |
| All | Tool set 2 | SET2 | SET1 + ???? |  |
| All | Tool set 3 | SET3 | SET2 + ???? |  |
| CTU partition | CTC tool set | SETCTC | All tools enabled in CTC version X |  |
| CTU partition | Chroma separate tree | CST |  | Tzu-Der Chuang (MediaTek) |
| Transform and quantization | Dependent quantization | DQ |  | H. Schwarz (HHI) |
| Intra coding | Cross-component linear model | CCLM | Exercise different variants of down-sample filters Corner cases for liner model parameters | Alexey Filippov (Huawei) |
|  |  |  | Corner cases for enabling conditions of coding structures | Kei Kawamura (KDDI) |
| Transform and quantization | Multiple transform set | MTS | include MTS combinations of (intra, inter): (implicit, none), (explicit, none), (implicit, explicit), (explicit, explicit) | Moonmo Koo (LGE) Mehdi Salehifar (LGE) |
| In-loop filter | Adaptive loop filter | ALF | ALF virtual boundary processing for line buffer reduction | Anand Meher Kotra (Huawei) |
|  |  |  | Exercise clipping values of Non-linear ALF | Kyohei Unno (KDDI) |
| Inter coding | Affine motion model | AFF | include Affine AMVP and | Chen Huanbang (Huawei) |
|  |  |  | Affine Merge; Control flags |  |
| Inter coding | Subblock-based temporal merging candidates | SbTMVP |  | Na Zhang (Bytedance) |
| Inter coding | Adaptive motion vector resolution | AMVR | include SIF | M. Winken (HHI) |
| Inter coding | Triangular partition mode | TPM | include triangle part. pred. | C.-W Kuo (Panasonic) |
| Inter coding | Bi-directional optical flow | BDOF |  | Ching-Yeh Chen (MediaTek) |
| Inter coding | Combined intra/inter prediction | CIIP |  | Chih-Wei Hsu (MediaTek) |
| Inter coding | Merge with MVD | MMVD |  | Kiho Choi (Samsung) |
| Inter coding | Bi-predictive with CU weights | BCW |  | Ching-Yeh Chen (MediaTek) |
| Intra coding | Multi-reference line prediction | MRLP |  | B. Bross (HHI) |
| SCC coding | Intra block copy mode | IBC |  | Xiaozhong Xu (Tencent) |
| Intra coding | Intra sub-partitioning | ISP |  | S. de Luxan Hernandez (HHI) |
| Inter coding | Decoder motion vector refinement | DMVR | Exercise enabling conditions, MV wraparound and MV clip at picture boundary | Semih Esenlik (Huawei) |
|  |  |  | Exercise corner cases of SAD variations | Kyohei Unno (KDDI) |
| Transform and quantization | Sub-block transform | SBT | Different block sizes; control flag | Zhao Yin (Huawei) |
| In-loop filter | Luma mapping with chroma scaling | LMCS | Exercise multiple APSes | T. Lu (Dolby) |
| Entropy coding | Sign data hiding | SDH |  | T. Lu (Dolby) |
| Inter coding | Symmetric motion vector difference | SMVD | Long-term reference handling mvd_l1_zero_flag handling | Chen Huanbang (Huawei) |
| Intra coding | Quantized residual DPCM | RDPCM |  |  |
| Intra coding | Matrix based intra prediction | MIP |  | J. Pfaff (HHI) |
| Transform and quantization | Low frequency non-separable transform | LFNST | Various block sizes /shapes Combinations with intra coding tools | Moonmo Koo (LGE) Mehdi Salehifar (LGE) M. Siekmann (HHI) |
| Transform and quantization | Transform tool set | MTS, LFNST | Tool on or off of MTS and LFNST together with implicit or explicit MTS | Moonmo Koo (LGE) Mehdi Salehifar (LGE) |
| Transform and quantization | Joint coding of chrominance residuals | JCCR |  | H. Schwarz (HHI) J. Lainema (Nokia) |
| Inter coding | Temporal motion vector predictor | TMVP |  |  |
| In-loop filter | Sampled adaptive offset | SAO |  | Kiho Choi (Samsung) |
| Inter coding | Prediction refinement using optical flow | PROF | Various non-translational motion parameters for PROF | Yuwen He (InterDigital) |
| In-loop filter | Deblocking | DBF | Exercise luma adaptive deblock filter and long tap filter | K. Andersson (Ericsson) |
|  |  |  | Deblocking at 4 x 4 grid | Anand Meher Kotra (Huawei) |
| In-loop filter | Adaptive loop filter | ALF | Use multiple APSes | Vadim Seregin (Qualcomm) |
| Inter coding | Weighted prediction | WP | Various combinations with other inter tools | Philippe Bordes (InterDigital) |
| Intra coding | Intra prediction | IP | Enable all modes, especially the wide-angle modes. | Alexey Filippov (Huawei) J. Lainema (Nokia) |
| Intra coding | Luma intra prediction mode | MPM | Enable all conditions to generate MPM candidate | Jin Heo(LGE) |
| CTU partition | CTU sizes | CTU | Exercise range of CTU, CU sizes | Chih-Wei Hsu (MediaTek) |
|  |  | CU |  |  |
| CTU partition | Trees and partitioning | BT | Exercise range of sizes and depths of BT, TT, QT | A. Wieckowski (HHI) |
|  |  | TT |  | Chih-Wei Hsu (MediaTek) |
|  |  | QT |  |  |
| CTU partition | Boundary partition |  | Boundary are sizes 8…120 samples, all combinations of QT and BT | Han Gao (Huawei) |
| Transform and quantization | Transform |  | min and max transform |  |
|  |  |  | min number of entropy coded coeff. |  |
|  |  |  | max number of coeff. |  |
| Transform and quantization | Quantization |  | CU level delta QP CU level Chroma delta QP transform-quant bypass with DB |  |
|  | Scaling list |  | Exercise multiple APSes Quantization Matrices | Philippe de Lagrange (InterDigital) |
| Entropy coding | Entropy coding |  | max bins and bits |  |
|  |  |  | min bits |  |
| Inter coding | All merge modes |  | max number of merge candidates |  |
| Intra coding | Position dependent prediction combination (PDPC) |  | Force clipping. Different PU sizes and shapes. |  |
| SCC coding | Palette mode |  | For 4:4:4 |  |
| SCC coding | Adaptive Color Transform | ACT | For 4:4:4 |  |

Table 2: Features and high-level syntax

| Categories | Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- | --- |
| High level syntax (HLS) | Tile rows / columns |  | pictures partitions in tiles, bricks and slices Different tile sizes in same picture loop filters on/off |  |
|  |  |  | loop filters on/off across | X. Ma and B. Wang (Huawei) |
|  | Slices |  | raster-scan slices |  |
|  |  |  | rectangular slices |  |
|  |  |  | loop filters on/off across |  |
|  | Sub-pictures |  | various number of sub-pictures | X. Ma and B. Wang (Huawei) |
|  |  |  | sub-pictures of varying sizes |  |
|  |  |  | multiple tiles in some sub-pictures |  |
|  |  |  | loop filters on/off across |  |
| HLS | Reference picture resizing |  |  |  |
| HLS | Ref pic wrap-around |  |  |  |
| HLS | Spatial scalability |  | Exercise Inter-layer ref pic list | X. Ma and B. Wang (Huawei) |
| HLS | Temporal scalability |  |  |  |
| HLS | Lossless and near-lossless |  |  | T. Nguyen |
| HLS | Transform skip |  |  | T. Nguyen |
| HLS | Gradual decoder refresh |  |  | R. Sjöberg (Ericsson) |
| HLS | Reference picture lists |  |  | R. Sjöberg (Ericsson) |
| HLS | Long term ref picture |  |  | R. Sjöberg (Ericsson) |
| HLS | Virtual boundaries |  | Loop filters on/off at non-tile/slice boundaries | Jian-Liang Lin (MediaTek) |
| HLS | 360 Video w/ cube map layout and SEI |  |  |  |
| HLS | 360 Video bitstream created using sub-picture extraction & merging |  |  |  |
| HLS | Wavefronts |  |  | Ikai Tomohiro (Sharp) |
| HLS | Conformance cropping window |  |  |  |
| HLS | Transfer characteristics |  | PQ, HLG | PQ: T. Lu (Dolby) |
| HLS | NAL unit type |  | Exercise all types, including STSA and random access |  |

Table 3: Chroma formats and bit depths

| Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- |
| 8b 4:0:0 |  | Main 10 profile |  |
| 8b 4:2:0 |  | Main 10 profile |  |
| 8b 4:2:2 |  | Main 4:4:4 10 profile |  |
| 8b 4:4:4 |  | Main 4:4:4 10 profile |  |
| 10b 4:0:0 |  | Main 10 profile |  |
| 10b 4:2:2 |  | Main 4:4:4 10 profile |  |
| 10b 4:4:4 |  | Main 4:4:4 10 profile |  |