| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0482-v1 |
| --- | --- |

| Title: | AHG9: Signalling of constant parameter values |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Peisong Chen Brian Heng Tim Hellman Minhua Zhou | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution contends that, with the adoption of the picture header structure at the October 2019 meeting in Geneva, the need for signalling constant parameter values in the PPS should be revisited.  The original motivation behind the adoption of this concept was to avoid sending redundant slice header parameters that stay constant for one or more complete pictures.  This was reported to be especially bandwidth inefficient for configurations with many slices with one example to be a 24 slice OMAF example.  This contribution asserts that, since parameters can now be signalled in the picture header, the overhead and benefit of signalling fixed values in the PPS would be minimal even for configurations with a large number of slices.

# Introduction

The need for signalling constant parameter values to avoid repeating the same parameter for every slice was originally proposed in JVET-O0238 [1].  This contribution provided examples such as a 24 slice per picture OMAF non-CTC configuration with bit rate savings of -0.24%/-1.92%/0.72% RA/LDB/LDP.  This is certainly undesirable and was the main motivation behind its adoption at the July 2019 meeting.

The picture header was proposed in JVET-P0239 [2] and adopted at the October 2019 meeting.  This was also motivated by addressing the issue of unnecessarily repeating parameters for every slice by moving parameters from the slice header to the picture header.  This effectively changed parameters from being sent N times per picture for a pictures with N slices, to only once per picture no matter how many slices were in a picture.

This contribution asserts that, since parameters can now be signalled in the picture header, the overhead and benefit of signalling constant parameter values in the PPS is not needed.  It is claimed that even for configurations with a large number of slices, the bit rate savings would be much smaller than when this concept was adopted.

Also, as shown in JVET-P0152 [3], JVET Bug #575, JVET Bug #691, … the constant parameter feature has already shown itself to be error prone and easy to use incorrectly.

Thus, it is proposed to remove this signalling for a cleaner design, eliminate the additional testing/verification involved with different combinations of parameter signalling, and avoid the potential errors that can be introduced when the feature is used improperly.

# Discussion

The signalling of constant parameter values in the current draft text [4] is facilitated with the constant_slice_header_params_enabled_flag syntax element in the PPS.  This flag signals the presence or absence of seven subsequent syntax elements.  The syntax of these PPS signalling syntax elements are shown below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| ... |  |
| } |  |

The sole purpose of the constant_slice_header_params_enabled_flag syntax element is for coding efficiency.  The table below shows that 12 bits per PPS are required for a bitstream to set each of the syntax elements equal to 0 compared to using 1 bit for the constant_slice_header_params_enabled_flag.  This contribution asserts that this savings of 11 bits per PPS is not significant enough to warrant its inclusion in the standard and therefore, the conditional flag should be removed.

|  | Descriptor | Bits when coding syntax element to be 0 |
| --- | --- | --- |
| pps_dep_quant_enabled_idc | u(2) | 2 |
| pps_ref_pic_list_sps_idc[ 0 ] | u(2) | 2 |
| pps_ref_pic_list_sps_idc[ 1 ] | u(2) | 2 |
| pps_mvd_l1_zero_idc | u(2) | 2 |
| pps_collocated_from_l0_idc | u(2) | 2 |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) | 1 |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) | 1 |
| TOTAL |  | 12 |

Each of the seven syntax elements after constant_slice_header_params_enabled_flag signals the presence or absence of other syntax elements in the picture header and/or slice header, as summarized in the table below:

|  | Presence of PH Syntax Element | Presence of SH Syntax Element |
| --- | --- | --- |
| pps_dep_quant_enabled_idc | pic_dep_quant_enabled_flag |  |
| pps_ref_pic_list_sps_idc[ i ] | pic_rpl_sps_flag[ i ] | slice_rpl_sps_flag[ i ] |
| pps_mvd_l1_zero_idc | mvd_l1_zero_flag |  |
| pps_collocated_from_l0_idc |  | collocated_from_l0_flag |
| pps_six_minus_max_num_merge_cand_plus1 | pic_six_minus_max_num_merge_cand |  |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | pic_max_num_merge_cand_minus_max_num_triangle_cand |  |

Despite the name “constant slice header parameters”, note that most of these syntax elements are in the picture header now.

This contribution proposes removing all these PPS signalling syntax elements.  For six of the seven PPS syntax elements (all except for pps_collocated_from_l0_idc), there already is an existing picture header syntax element.  For the collocated_from_l0_flag syntax element in the slice header, it is proposed to create an associated syntax element in the picture header to replace the existing PPS syntax element.

# Proposal

The contribution proposes to simplify the signalling of constant slice header parameters by removing the aspects that are currently in the PPS so that all constant slice signalling is in the picture header.  More specifically, these changes can be summarized as:

- Removing constant_slice_header_params_enabled_flag and the 7 dependent syntax elements from the PPS

- Create a pic_collocated_from_l0_idc syntax element in the picture header that implements the current functionality of the pps_collocated_from_l0_idc.

The proposed syntax changes in this section provide a conceptual overview of the proposed changes with the detailed syntax and semantic changes compared to the current draft text [3] in an attachment to this contribution.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| ... |  |
| } |  |

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(pic_temporal_mvp_enable_flag) |  |
| pic_collocated_from_l0_idc | u(2) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| pic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if( pps_dep_quant_enabled_idc = = 0 ) |  |
| pic_dep_quant_enabled_flag | u(1) |
| if( !pic_dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( pic_rpl_present_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( slice_type != I ) { |  |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc && !pic_collocated_from_l0_idc) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( sps_sao_enabled_flag && !pic_sao_enabled_present_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_present_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# References

[1] M. Pettersson, P. Wennersten, R. Sjöberg, M. Damghanian, “AHG17: Parameters in SPS or slice headers”, document JVET-O0238, JVET, Gothenburg, Sweden, July 2019.

[2] W. Wan, T. Hellman, B. Heng, “AHG17: Picture Header”, document JVET-P0239, JVET, Geneva, Switzerland, October 2019.

[3] O. Chubach, C.-C. Chen, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, “CE4-related: On maximum number of subblock-based merging candidates”, document JVET-P0152, JVET, Geneva, Switzerland, October 2019.

[4] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).