| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0487-v1 |
| --- | --- |

| Title: | AHG8: RPR Scaling Window Issues |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Tim Hellman Brian Heng Peisong Chen | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

Contribution JVET-P0590 introduced a scaling window to Reference Picture Resampling (RPR).  This contribution contends that this addition caused two issues: (1) the present specification text disabling Temporal Motion Vector Prediction (TMVP) when RPR is enabled is ambiguous, and (2) the software and specification do not match for certain scaling window configurations.  Fixes for both of these purported issues are proposed.

# Background

Two issues relating to RPR and scaling windows are discussed in this contribution:  the TMVP RPR constraint, and a spec/code mismatch.  Both of these issues arise when the scaling windows in the present and reference picture are different, but result in a 1:1 scale factor.

Contribution JVET-P0590 [1] introduced a per-picture scaling window, as shown in Figure 1.  This window is used to compute the scale factor between the current picture and its reference pictures.  Each reference can have its own scaling window, and they can all be different from the current picture’s scaling window.

Figure 1: Scaling Window Definition

When scaling windows are used, there are several ways to create 1:1 scaling factors. For example, in addition to the obvious case (same coded picture width and height, same scaling window offsets), one could have:

- Same coded picture width and height, same scaling window width and height, but different scaling window offsets (as shown in Figure 2).

- Completely different coded picture width and height, but same scaling window width/height.

Figure 2: Scaling factor equal to 1 with misaligned offsets.

## Issue 1: TMVP Constraint

Temporal Motion Vector Prediction (TMVP) needs to be disabled when the co-located reference picture differs in coded size and scale factor from the current picture, otherwise undefined temporal motion vectors could be referenced. Specifically, even if the scale factor is 1:1, if the coded picture sizes are not identical, TMVP can refer to regions outside the reference picture boundaries.

The constraint that controls this restriction is presently written in the semantics of the collocated_ref_idx syntax element in the slice header of the current draft text [2] as:

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

The term “resolutions” in the highlighted section is ambiguous here, as that term is not defined anywhere.  The important point is that the true (i.e. coded) width and height of the current and co-located pictures must be the same.  The suggested replacement language is:

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture values of pic_width_in_luma_samples and pic_height_in_luma_samples in the current picture and in the reference picture referred to by collocated_ref_idx shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

For odd cases like the misaligned 1:1 scaling shown in Figure 2, TMVP would still be allowed based on the suggested language above. Given that the TMVP references would be misaligned in such a case, the result would likely be sub-optimal. However, nothing about the TMVP process is broken or undefined in this case. Therefore, while it would likely be unwise to use TMVP in such a manner, this contribution is not suggesting more restrictive conditions to handle this case (i.e. requiring that pic_width, pic_height, left_offset, right_offset, top_offset, and bottom_offset must all be identical in order to enable TMVP).

## Issue 2: Code/Spec Mismatch when Scale Factor is 1

Since the scaling window in Figure 1 has offsets for each edge, it’s possible to create configurations in which the scaling ratios between the current and reference picture are equal to 1, yet the two scaling windows are different.  For example, in Figure 2 the two pictures have the same width and height, but one has an offset at the top and the other has it at the bottom.  As described earlier, this also results in a 1:1 scale factor.

The specification and code differ in how they handle this situation:

- Specification: In Section 8.5.6.3.1, RPR offsets and scaling is applied to all motion vectors, regardless of whether the scaling factor is 1:1 or not.  The 1:1 scale factor (i.e. 214) is treated the same as any other scale factor. Therefore, during this process any left_offset or top_offset will be applied accordingly, even if the scaling factor is 1:1.

- VTM 7.x Code: In the xPredInterBlkRPR( ) function, all RPR-related logic is only applied if the scaling factor is not 1:1.  Specifically:

const bool scaled = scalingRatio != SCALE_1X;

if( scaled )

{

“RPR prediction”

}

else

{

“standard prediction”

}

The standard inter-prediction process is the same as it was before RPR, and makes no adjustments for RPR left/top scaling window offsets. Therefore, when the scalingRatio is 1:1 above, any left/top offsets will be ignored and motion compensation will fail to match the specification text.

Logically it seems that the specification is correct, as the application of the offset shouldn’t depend on whether the scale factor is 1:1 or not.  Source code can be provided that corrects the source to match the specification.

# Conclusion

This proposal contends that two issues were introduced by the addition of the RPR scaling window.  Solutions are provided for both issues, so it therefore recommends that the draft text and source code changes be adopted.

# References

[1] V. Seregin, A Ramasubramonian, et al, “AHG8: Scaling window for scaling ratio derivation”, document JVET-P0590, JVET, Geneva, CH, October 2019.

[2] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).