| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0489-v2 |
| --- | --- |

| Title: | CE3-related: Modified TS residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Hongtao Wang Muhammed Coban Alican Nalci (Qualcomm) | Tel: Email: | +1 858-658-5673 martak@qti.qualcomm.com hongtaow@qti.qualcomm.com mcoban@qti.qualcomm.com analci@qti.qualcomm.com |
|  | Cheung Auyeung, Xiang Li Xin Zhao Shan Liu (Tencent) |  | cauyeung@tencent.com xlxiangli@tencent.com xinzzhao@tencent.com shanl@tencent.com |
| Source: | Qualcomm Incorporated, Tencent |  |  |

_____________________________

# Abstract

This document proposes modifications to transform skip coefficient coding to improve its coding efficiency for lossless coding. Level prediction is removed, the abs_level_gt2_flag flag is moved to the 1st pass while the coeff_sign_flag is moved to the second pass, and the context derivation of sig_coeff_flag, abs_level_gt1_flag and the abs_level_gt2_flag flags are modified. The rice parameter value is calculated similarly as for transform coefficients with the normalization based on the number of available neighboring coefficients.

The coding results for the proposed method comparing to VTM-7.0 anchor (CTC) are summarized below:

- Natural:   -0.03% (AI), -0.01% (RA), -0.06% (LB)

- Class F:   0.03% (AI), 0.02%% (RA), -0.22% (LB)

- TGM:     0.03% (AI), -0.05% (RA), -0.32% (LB)

The coding results for the proposed method comparing VTM-7.0 anchor (lossless) are summarized below:

- Natural:   -6.48%, (AI), -6.08% (RA), -5.23% (LB)

- Class F:   -4.37% (AI), -4.68% (RA), -4.52% (LB)

- TGM:    -2.34% (AI), -3.36% (RA), -2.19% (LB)

When compared with VTM-7.0 using regular transform coefficient coding (RRC) as anchor, the lossless results are summarized below:

- Natural:   -1.32% (AI), -0.54% (RA), -0.29% (LB)

- Class F:   -3.00% (AI), -2.28% (RA), -2.39% (LB)

- TGM:    -3.81%(AI), -3.92%(RA), -3.32%(LB)

# Introduction

This contribution proposes modifications to transform skip coefficient coding which improves its performance for lossless coding and have negligible impact on lossy coding. In the proposed method, level prediction is removed, the abs_level_gt2_flag flag is moved to the 1st pass while the coeff_sign_flag is moved to the second pass and the context derivation for sig_coeff_flag, abs_level_gt1_flag and the abs_level_gt2_flag are modified. The rice parameter is calculated similarly as for transform coefficient with addition of normalization based on number of available neighboring coefficients.

# Proposed method

The modifications proposed by this method can be summarized as follows:

- The VVC draft 7 uses level prediction for transform skip coefficients when entropy coding is in non-bypass mode. The absolute value of a transform skip coefficient is predicted by selecting the maximum of the absolute values of its top and left neighbors. In this proposal, the level prediction is removed.

- The same Rice parameter derivation as for transform coefficient is used. The difference is the normalization of the sum of the coefficients based on number of available coefficients which was studied in CE3-1.1.

- The coding of abs_level_gt2_flag is moved to the 1nd pass while coeff_sign_flag is moved to the 2nd pass. In the first pass of the residual coding, sig_coeff_flag, abs_level_gt1_flag, abs_level_gt2_flag and par_level_flag are coded. In the 2nd pass, coeff_sign_flag and abs_level_gtX_flags (X=3,4,5) are coded.

- The context derivation of sig_coeff_flag, abs_level_gt1_flag and the abs_level_gt2_flag are modified using the method presented in [1]. Luma and chroma use separate contexts with exception of the sign context.

## Rice parameter derivation

First the sum of absolute coefficient values (locSumAbs) in the local template is calculated. The local template includes up to 5 neighboring coefficients as shown in Figure 1. The template is the same as used for regular transform coefficients.

Figure 1 Template used for TS Rice parameter derivation

The value of locSumAbs is normalized using the number of coefficients available in this template (noPos):

locSumAbs = locSumAbs << ((5 – noPos) >>1).

As in the case of the transform coefficients, the final value of the Rice parameter depends also on the base level (baseLevel) of the current coefficient:

cRiceParam = riceParTable[max(min(31, baseLevel == 0 ? locSumAbs : locSumAbs - 20)), 0)],

where the Rice table riceParTable[] is the same as the one defined in VVC draft 7 for the regular transform coefficient coding.

## Modifications to the context derivation

The method presented in [1] to calculate the offset for the context increment for a significant flag (sig_coeff_flag) is extended to calculate the context for abs_level_gt1_flag and abs_level_gt2_flag flags. The context offset of a given flag (sig_coeff_flag, abs_level_gt1_flag or abs_level_gt2_flag) is derived based on the left, top and top-left neighbor values of this flag as shown in the table below. If a neighboring coefficient doesn’t exist, the value is inferred to be 0.

|  |  | topLeftFlag |  |
| --- | --- | --- | --- |
| topFlag | lefFlag | 0 | 1 |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 3 |
| 1 | 0 | 1 | 3 |
| 1 | 1 | 2 | 2 |

Table 1: Context offset derivation.

# Simulation Results

## Lossless results

Summary of the simulation results according to the CE3 test conditions for lossless coding are provided below. The following additional results are provided as well:

- Anchor is using RRC instead of TSRC for residual coding.

- Anchor has BDPCM enabled for luma. TSRC is used for residual coding.

- Anchor has BDPCM enabled for luma. RRC is used for residual coding.

Proposed method has BDPCM enabled for luma whenever anchor has BDPCM enabled.

### Simulation results with BDPCM=0

#### Using VTM-7.0 as anchor

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -6.43% | -7.08% |  |
| Class A2 | -10.35% | -9.40% |  |
| Class B | -4.83% | -4.01% | -4.03% |
| Class C | -6.20% | -5.43% | -5.34% |
| Class D | -6.23% | -5.53% | -5.49% |
| Class E | -5.76% |  | -7.10% |
| Class F | -4.37% | -4.68% | -4.52% |
| TGM | -2.34% | -3.36% | -2.19% |
| Overall | -6.48% | -6.08% | -5.23% |
| Enc Time[%] | 97% | 102% | 100% |
| Dec Time[%] | 97% | 99% | 107% |

#### Using VTM-7.0 + RRC for residual coding as anchor

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -1.08% | -0.90% |  |
| Class A2 | -1.65% | -1.39% |  |
| Class B | -0.86% | -0.20% | -0.21% |
| Class C | -1.30% | -0.06% | -0.05% |
| Class D | -1.67% | -0.28% | -0.26% |
| Class E | -2.02% |  | -0.73% |
| Class F | -3.00% | -2.28% | -2.39% |
| TGM | -3.81% | -3.92% | -3.32% |
| Overall | -1.32% | -0.54% | -0.29% |
| Enc Time[%] | 105% | 104% | 103% |
| Dec Time[%] | 91% | 99% | 98% |

### Simulation results with BDPCM=1

#### Using VTM-7.0 as anchor

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -6.24% | -6.94% |  |
| Class A2 | -10.16% | -9.33% |  |
| Class B | -4.30% | -3.83% | -3.83% |
| Class C | -5.60% | -5.28% | -5.20% |
| Class D | -5.52% | -5.42% | -5.39% |
| Class E | -4.97% |  | -6.59% |
| Class F | -4.22% | -4.62% | -4.50% |
| TGM | -2.89% | -3.61% | -2.45% |
| Overall | -6.00% | -5.94% | -4.98% |
| Enc Time[%] | 99% | 102% | 99% |
| Dec Time[%] | 97% | 101% | 108% |

#### Using VTM-7.0 + RRC for residual coding as anchor

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -1.01% | -0.86% |  |
| Class A2 | -1.64% | -1.37% |  |
| Class B | -0.56% | -0.07% | -0.07% |
| Class C | -1.03% | 0.02% | 0.03% |
| Class D | -1.22% | -0.21% | -0.19% |
| Class E | -1.41% |  | -0.49% |
| Class F | -2.33% | -1.83% | -2.09% |
| TGM | -3.49% | -3.84% | -3.40% |
| Overall | -1.06% | -0.46% | -0.14% |
| Enc Time[%] | 105% | 104% | 104% |
| Dec Time[%] | 95% | 99% | 99% |

## Lossy results

Results according to the JVET CTC are provided in the below tables, separately for normal CTC QP range (22, 27, 32, 37) and the low QP range (2, 7, 12, 17) as specified in the CE3 description.

### CTC standard QP range (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.17% | 0.06% | 99% | 97% |
| Class A2 | 0.00% | -0.01% | -0.07% | 101% | 99% |
| Class B | -0.01% | -0.02% | -0.06% | 101% | 102% |
| Class C | -0.06% | -0.02% | 0.01% | 100% | 97% |
| Class E | -0.05% | -0.05% | 0.06% | 100% | 97% |
| Overall | -0.03% | -0.05% | -0.01% | 100% | 99% |
| Class D | -0.07% | -0.13% | -0.02% | 99% | 93% |
| Class F | 0.03% | 0.16% | 0.11% | 102% | 99% |
| TGM | 0.03% | -0.02% | -0.11% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.03% | 100% | 99% |
| Class A2 | -0.03% | -0.11% | 0.00% | 102% | 99% |
| Class B | 0.01% | -0.09% | -0.05% | 99% | 101% |
| Class C | -0.03% | -0.15% | -0.05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.11% | -0.02% | 100% | 100% |
| Class D | -0.08% | -0.18% | -0.33% | 101% | 101% |
| Class F | 0.02% | 0.14% | 0.24% | 102% | 101% |
| TGM | -0.05% | -0.22% | -0.17% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.26% | -0.13% | 100% | 95% |
| Class C | -0.07% | -0.45% | -0.33% | 100% | 99% |
| Class E | -0.16% | 0.35% | 0.14% | 101% | 98% |
| Overall | -0.06% | -0.17% | -0.13% | 100% | 97% |
| Class D | -0.12% | -0.20% | -0.35% | 100% | 102% |
| Class F | -0.22% | -0.01% | 0.43% | 102% | 100% |
| TGM | -0.32% | -0.41% | -0.45% | 101% | 100% |

### CTC low QP range (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.07% | -0.09% | 100% | 97% |
| Class A2 | 0.00% | -0.03% | -0.03% | 101% | 102% |
| Class B | -0.06% | -0.09% | -0.15% | 100% | 98% |
| Class C | -0.19% | -0.24% | -0.27% | 100% | 101% |
| Class E | -0.30% | -0.45% | -0.57% | 98% | 95% |
| Overall | -0.13% | -0.17% | -0.22% | 100% | 99% |
| Class D | -0.21% | -0.31% | -0.27% | 100% | 86% |
| Class F | -1.59% | -0.82% | -0.81% | 101% | 97% |
| TGM | -0.36% | -0.56% | -0.49% | 101% | 96% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | -0.10% | -0.14% | 101% | 100% |
| Class C | -0.10% | -0.19% | -0.21% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.21% | -0.28% | -0.52% | 100% | 97% |
| Class F | -2.17% | -0.96% | -1.01% | 101% | 98% |
| TGM | -1.27% | -0.62% | -0.59% | 101% | 95% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.08% | -0.12% | -0.18% | 101% | 103% |
| Class E | -0.10% | -0.01% | -0.03% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.21% | -0.16% | -0.38% | 101% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | -0.57% | -0.37% | -0.38% | 103% | 98% |

# Reference

- C. Auyeung, X. Li, X. Zhao, S. Liu , “CE7-related: Modifications to transform skip significant flag coding”, JVET-P0435, 16th meeting, Geneva, Oct. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).