| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0490-v2 |
| --- | --- |

| Title: | CE3-related: Simplification of rice parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Marta Karczewicz Muhammed Coban | Tel: Email: | +1 858-658-4535 hongtaow@qti.qualcomm.com martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a rice parameter derivation method without look-up table for regular transform residual coding. Instead of using the value locSumAbs (sum of absolute values with in the same local template) as a table index, the proposed method derives the rice parameter by performing arithmetic operations on locSumAbs. The reported psnr-Y, U, V BD-rate variations compared to VTM7.0 are 0.01%, -0.02%, -0.03% for the AI configuration, -0.01%, -0.01%, 0.02% for the RA configuration, 0.01%, -0.17%, -0.24% for the LDB configuration.

# Introduction

In VVC draft 7[1], rice parameters used for the coding of abs_remainder for regular transform residual coding (RRC) are derived as follows:

Figure 1 local template used for RRC rice parameter derivation

As shown in Figure 1, a local template of up to 5 neighbors is used to calculate the value locSumAbs:

locSumAbs = abs(coeff(x+1,y))   + abs(coeff(x+2,y)) + abs(coeff(x,y+1)) +

abs(coeff(x+1,y+1)) + abs(coeff(x,y+2)).

If the coeff(i, j) is outside of a TU, its value is inferred to be 0 during calculation.  Then locSumAbs is further adjusted according to baseLevel:

locSumAbs = locSumAbs - 5 * baseLevel.

The rice parameter (cRiceparam) is derived by using the value locSumAbs as an index of a look-up table:

cRiceParam = riceParTable[max(min(locSumAbs, 31), 0)],

riceParTable[32] = { 0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3 }.

# Proposed method

In the proposed method, instead of using locSumAbs as a table index, the rice parameter used for RRC is calculated as follows:

cRiceParam = max(0, min(3, (locSumAbs + 1) >> 3))

# Simulation Results

Results of the proposed method according to the JVET CTC are provided as follows.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.06% | 99% | 99% |
| Class A2 | 0.01% | 0.01% | -0.03% | 100% | 98% |
| Class B | 0.01% | 0.02% | -0.11% | 100% | 99% |
| Class C | 0.01% | 0.03% | 0.00% | 100% | 101% |
| Class E | 0.00% | -0.03% | 0.00% | 100% | 102% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.07% | 99% | 102% |
| Class F | -0.02% | 0.03% | 0.06% | 102% | 99% |
| TGM | 0.01% | 0.01% | 0.03% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.04% | 97% | 99% |
| Class A2 | -0.04% | -0.06% | -0.04% | 100% | 93% |
| Class B | 0.00% | 0.12% | 0.05% | 100% | 103% |
| Class C | 0.01% | -0.11% | 0.02% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.02% | 100% | 99% |
| Class D | -0.02% | -0.06% | 0.03% | 101% | 99% |
| Class F | -0.04% | 0.09% | 0.03% | 101% | 101% |
| TGM | -0.03% | -0.02% | 0.02% | 102% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.19% | 0.16% | 100% | 98% |
| Class C | 0.01% | -0.51% | -0.16% | 100% | 98% |
| Class E | -0.01% | 0.31% | -1.00% | 103% | 98% |
| Overall | 0.01% | -0.17% | -0.24% | 101% | 98% |
| Class D | -0.05% | -0.07% | -0.51% | 100% | 97% |
| Class F | -0.03% | -0.67% | 0.36% | 103% | 104% |
| TGM | -0.01% | -0.07% | -0.11% | 100% | 102% |

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th meeting, Geneva, Oct. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)