| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0490-v3 |
| --- | --- |

| Title: | CE3-related: Simplification of rice parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Marta Karczewicz Muhammed Coban | Tel: Email: | +1 858-658-4535 hongtaow@qti.qualcomm.com martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes rice parameter derivation methods without look-up table for regular transform residual coding (RRC) and transform-skip residual coding (TSRC). Instead of using the value locSumAbs (sum of absolute values with in the same local template) as a table index, the proposed method derives the rice parameter by performing arithmetic operations on locSumAbs.

The coding results for the proposed method comparing to VTM-7.0 anchor (CTC) are summarized below:

|  | RRC change | RRC + TSRC change |
| --- | --- | --- |
| Natural | 0.01% (AI), XX% (RA), XX% (LB) | 0.00% (AI), XX% (RA), XX% (LB) |
| Class F | 0.01% (AI), 0.00% (RA), XX% (LB) | -0.15% (AI), -0.12% (RA), XX% (LB) |
| TGM | 0.02% (AI), -0.01% (RA), XX% (LB) | -0.36% (AI), -0.18% (RA), XX% (LB) |

The coding results for the proposed method comparing VTM-7.0 anchor (lossless) are summarized below:

|  | RRC change | TSRC change |
| --- | --- | --- |
| Natural | -5.11% (AI), XX% (RA), -4.97% (LB) | -5.12% (AI), XX% (RA), -2.41% (LB) |
| Class F | -1.41% (AI), -2.42% (RA), -2.14% (LB) | -4.11% (AI), -2.91% (RA), -2.65% (LB) |
| TGM | 1.64% (AI), 0.74% (RA), 1.32% (LB) | -4.77% (AI), -6.65% (RA), -6.68% (LB) |

# Introduction

In VVC draft 7[1], a fixed value of 1 is used as the rice parameter for transform-skip residual coding (TSRC).

For regular transform residual coding (RRC), rice parameters are derived in VVC draft 7 as follows:

Figure 1 local template used for RRC rice parameter derivation

As shown in Figure 1, a local template of up to 5 neighbors is used to calculate the value locSumAbs:

locSumAbs = abs(coeff(x+1,y))   + abs(coeff(x+2,y)) + abs(coeff(x,y+1)) +

abs(coeff(x+1,y+1)) + abs(coeff(x,y+2)).

If the coeff(i, j) is outside of a TU, its value is inferred to be 0 during calculation. Then locSumAbs is further adjusted according to baseLevel:

locSumAbs = locSumAbs - 5 * baseLevel.

The rice parameter (cRiceparam) is derived by using the value locSumAbs as an index of a look-up table:

cRiceParam = riceParTable[max(min(locSumAbs, 31), 0)],

riceParTable[32] = { 0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3 }.

# Proposed method

## Method 1: Proposed RRC change

In the proposed method, instead of using locSumAbs as a table index, the rice parameter used for RRC is calculated as follows:

cRiceParam = max(0, min(maxRiceParam, (locSumAbs + offset) >> 3))

The maximum allowed value of rice parameter (maxRiceParam) is set to 3 and the offset is set to 0 or 1. The impact of setting maxRiceParam to 4 for lossless coding is studied as well in this contribution.

## Method 2: Proposed RRC+TSRC change

In Method 2, the RRC change described in section 2.1 with offset=1 is used. In addition to that, the rice parameter derivation of TSRC is described as follows:

The value of locSumAbs is calculated based on the top and left neighbors:

locSumAbs = abs(coeff(x-1, y)) + abs(coeff(x,y-1)).

If coeff(i,j) doesn’t exist, its value is inferred to be 0. And locSumAbs is further adjusted according to the value of baseLevel:

locSumAbs = locSumAbs – 2 * baseLevel + !baseLevel.

Based on locSumAbs, value of cRiceParam is calculated as follows:

cRiceParam = max(0, min(maxRiceParam, (locSumAbs + 5) >> 3)).

Simulation results of setting maxRiceParam to both 3 and 4 are provided in this contribution.

# Simulation Results

## Simulation results of method 1

### Method 1 with offset=0

Results of the proposed method 1 with offset=0 according to the CTC are provided in Table 1:

Table 1 Method 1 with offset=0 CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | 0.07% | 105% | 99% |
| Class A2 | 0.00% | -0.02% | -0.03% | 98% | 98% |
| Class B | 0.01% | 0.03% | -0.01% | 96% | 93% |
| Class C | 0.01% | 0.01% | 0.08% | 98% | 92% |
| Class E | 0.01% | -0.09% | -0.02% | 98% | 89% |
| Overall | 0.01% | -0.03% | 0.02% | 99% | 94% |
| Class D | 0.02% | -0.01% | -0.01% | 96% | 85% |
| Class F | 0.01% | 0.03% | 0.03% | 98% | 90% |
| TGM | 0.02% | 0.02% | 0.00% | 92% | 94% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.03% | 99% | 98% |
| Class A2 | -0.01% | 0.07% | 0.09% | 95% | 97% |
| Class B | 0.01% | 0.06% | 0.10% | 94% | 92% |
| Class C | 0.01% | 0.09% | -0.09% | 97% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.03% | 96% | 94% |
| Class D | 0.03% | -0.21% | -0.05% | 96% | 91% |
| Class F | 0.00% | 0.10% | -0.08% | 96% | 93% |
| TGM | -0.01% | 0.00% | 0.03% | 95% | 93% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.39% | -0.08% | 99% | 96% |
| Class C | 0.03% | -0.20% | -0.05% | 95% | 94% |
| Class E | -0.02% | 0.52% | 0.24% | 96% | 90% |
| Overall | 0.00% | -0.10% | 0.01% | 97% | 94% |
| Class D | 0.02% | 0.31% | -0.39% | 96% | 94% |
| Class F | 0.03% | -0.30% | 0.53% | 94% | 90% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of the proposed method 1 with offset=0 according to the CE3 lossless test condition are provided in Table 2:

Table 2 Method 1 with offset=0 lossless results

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.16% | #DIV/0! |  |
| Class A2 | -8.56% | #DIV/0! |  |
| Class B | -3.88% | -3.71% | -3.71% |
| Class C | -4.93% | -5.43% | -5.36% |
| Class D | -4.59% | -5.29% | -5.27% |
| Class E | -3.93% |  | -6.57% |
| Class F | -1.41% | -2.42% | -2.14% |
| TGM | 1.64% | 0.74% | 1.32% |
| Overall | -5.11% | #DIV/0! | -4.97% |
| Enc Time[%] | 90% | #NUM! | 93% |
| Dec Time[%] | 96% | #NUM! | 102% |

Additionally, the simulation results of using maxRiceParam=4 are summarized in Table 3:

Table 3 Method 1 with offset=1&maxRiceParam=4 lossless results

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.30% | #DIV/0! |  |
| Class A2 | -7.62% | #DIV/0! |  |
| Class B | -3.45% | -3.17% | -3.16% |
| Class C | -4.59% | -5.24% | -5.17% |
| Class D | -4.33% | -5.11% | -5.11% |
| Class E | -3.81% |  | -6.56% |
| Class F | -1.47% | -2.46% | -2.16% |
| TGM | 0.89% | 0.83% | 1.76% |
| Overall | -4.60% | #DIV/0! | -4.68% |
| Enc Time[%] | 90% | #NUM! | 93% |
| Dec Time[%] | 99% | #NUM! | 104% |

### Method 1 with offset=1

Results of the proposed method 1 with offset=1 according to the JVET CTC are provided in Table 4.

Table 4 Method 1 with offset=1 CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.06% | 99% | 99% |
| Class A2 | 0.01% | 0.01% | -0.03% | 100% | 98% |
| Class B | 0.01% | 0.02% | -0.11% | 100% | 99% |
| Class C | 0.01% | 0.03% | 0.00% | 100% | 101% |
| Class E | 0.00% | -0.03% | 0.00% | 100% | 102% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.07% | 99% | 102% |
| Class F | -0.02% | 0.03% | 0.06% | 102% | 99% |
| TGM | 0.01% | 0.01% | 0.03% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.04% | 97% | 99% |
| Class A2 | -0.04% | -0.06% | -0.04% | 100% | 93% |
| Class B | 0.00% | 0.12% | 0.05% | 100% | 103% |
| Class C | 0.01% | -0.11% | 0.02% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.02% | 100% | 99% |
| Class D | -0.02% | -0.06% | 0.03% | 101% | 99% |
| Class F | -0.04% | 0.09% | 0.03% | 101% | 101% |
| TGM | -0.03% | -0.02% | 0.02% | 102% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.19% | 0.16% | 100% | 98% |
| Class C | 0.01% | -0.51% | -0.16% | 100% | 98% |
| Class E | -0.01% | 0.31% | -1.00% | 103% | 98% |
| Overall | 0.01% | -0.17% | -0.24% | 101% | 98% |
| Class D | -0.05% | -0.07% | -0.51% | 100% | 97% |
| Class F | -0.03% | -0.67% | 0.36% | 103% | 104% |
| TGM | -0.01% | -0.07% | -0.11% | 100% | 102% |

Results of the proposed method 1 with offset=1 according to the CE3 lossless test condition are provided in Table 5:

Table 5 Method 1 with offset=1 lossless results

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.22% | #DIV/0! |  |
| Class A2 | -8.68% | #DIV/0! |  |
| Class B | -3.89% | -3.68% | -3.68% |
| Class C | -4.89% | -5.32% | -5.24% |
| Class D | -4.55% | -5.22% | -5.21% |
| Class E | -3.78% |  | -6.42% |
| Class F | -1.35% | -2.36% | -2.09% |
| TGM | 1.55% | 0.69% | 1.35% |
| Overall | -5.11% | #DIV/0! | -4.89% |
| Enc Time[%] | 92% | #NUM! | 96% |
| Dec Time[%] | 102% | #NUM! | 108% |

## Simulation results of method 2

### Method 2 with maxRiceParam=3

Results of the proposed method 2 with maxRiceParam=3 according to the CTC are provided in Table 6:

Table 6 Method 2 with maxRiceParam=3 CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | 0.07% | 105% | 102% |
| Class A2 | 0.01% | 0.02% | -0.06% | 97% | 99% |
| Class B | 0.01% | 0.07% | -0.10% | 97% | 96% |
| Class C | -0.03% | 0.01% | 0.00% | 99% | 98% |
| Class E | 0.00% | -0.14% | -0.10% | 99% | 97% |
| Overall | 0.00% | -0.02% | -0.04% | 99% | 98% |
| Class D | -0.04% | 0.02% | -0.02% | 98% | 94% |
| Class F | -0.17% | -0.20% | -0.02% | 100% | 98% |
| TGM | -0.38% | -0.30% | -0.32% | 93% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.01% | 99% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.02% | 0.16% | 97% | 97% |
| Class C | -0.02% | -0.06% | -0.05% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.03% | -0.02% | 99% | 97% |
| Class F | -0.11% | 0.03% | -0.05% | 101% | 100% |
| TGM | -0.22% | -0.24% | -0.14% | 99% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.39% | -0.29% | 97% | 97% |
| Class E | 0.08% | 0.14% | -0.70% | 98% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.25% | -0.46% | 98% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of the proposed method 2 with maxRiceParam=3 according to the CE3 lossless test condition are provided in Table 7:

Table 7 Method 2 with maxRiceParam=3 lossless results

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.57% | #DIV/0! |  |
| Class A2 | -8.12% | #DIV/0! |  |
| Class B | -3.90% | -2.32% | -2.35% |
| Class C | -4.81% | -2.44% | -2.35% |
| Class D | -5.28% | -2.55% | -2.43% |
| Class E | -3.93% |  | -2.62% |
| Class F | -3.77% | -2.71% | -2.45% |
| TGM | -3.76% | -5.98% | -6.14% |
| Overall | -4.92% | #DIV/0! | -2.42% |
| Enc Time[%] | 100% | #NUM! | 100% |
| Dec Time[%] | 100% | #NUM! | 102% |

### Method 2 with maxRiceParam=4

Results of the proposed method 2 with maxRiceParam=4 according to the CTC are provided in Table 8:

Table 8 Method 2 with maxRiceParam=4 CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | 0.07% | 105% | 101% |
| Class A2 | 0.01% | 0.03% | -0.03% | 97% | 98% |
| Class B | 0.01% | 0.06% | -0.09% | 97% | 97% |
| Class C | -0.02% | 0.03% | -0.05% | 98% | 97% |
| Class E | 0.00% | -0.08% | -0.12% | 99% | 99% |
| Overall | 0.00% | -0.01% | -0.05% | 99% | 98% |
| Class D | -0.03% | 0.07% | -0.02% | 98% | 97% |
| Class F | -0.15% | -0.16% | -0.06% | 100% | 97% |
| TGM | -0.36% | -0.30% | -0.31% | 94% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | -0.04% | 99% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.05% | 0.15% | 97% | 96% |
| Class C | -0.03% | -0.01% | 0.05% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.13% | -0.18% | 99% | 97% |
| Class F | -0.12% | -0.05% | -0.06% | 100% | 99% |
| TGM | -0.18% | -0.16% | -0.11% | 99% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.28% | -0.19% | 97% | 96% |
| Class E | -0.01% | -0.04% | -1.15% | 97% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.05% | -0.13% | 0.12% | 98% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of the proposed method 2 with maxRiceParam=4 according to the CE3 lossless test condition are provided in Table 9:

Table 9 Method 2 with maxRiceParam=4 lossless results

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.64% | #DIV/0! |  |
| Class A2 | -8.60% | #DIV/0! |  |
| Class B | -4.09% | -2.30% | -2.33% |
| Class C | -5.00% | -2.46% | -2.37% |
| Class D | -5.61% | -2.57% | -2.45% |
| Class E | -4.03% |  | -2.62% |
| Class F | -4.11% | -2.91% | -2.65% |
| TGM | -4.77% | -6.65% | -6.68% |
| Overall | -5.12% | #DIV/0! | -2.41% |
| Enc Time[%] | 99% | #NUM! | 100% |
| Dec Time[%] | 100% | #NUM! | 103% |

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th meeting, Geneva, Oct. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)