| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0493-v3 |
| --- | --- |

| Title: | Non-CE2: Palette encoder improvements for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Tsung-Chuan Ma, Yi-Wen Chen, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, tsung-chuanma, yiwenchen, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, the following encoder improvements are proposed to enhance the VTM7 Palette mode (PLT) coding for lossless case.

- Use correct precision of distortion based on input bit-depth during the derivation of the palette for lossless coding.

- Remove the unnecessary calculation of distortion during palette coding RD decision on lossless, because there is no distortion for lossless cases.

The proposed changes are implemented on VTM 7 and evaluated based on CE2 test conditions. Simulation results show that the coding performance is improved by -4.57% AI, -2.84% RA, -2.43% LB for dual tree on case, and -2.56% AI, -1.71% RA, -1.83% LB for dual tree off case.

# Introduction

In current VVC [1], it supports the palette mode to be used for screen content coding in 4:4:4 color format. If the palette mode is utilized, the sample values in the CU are represented by a small set of representative colour values. The set is referred to as the palette. For pixels with values close to the palette colors, the palette indices are signalled. For samples outside of the palette, they are signalled as escape values.

For the derivation of the palette for lossy coding, a modified k-means clustering algorithm is used. The first sample of the block is added to the palette. Then, for each subsequent sample from the block, the sum of absolute distances (SAD) in 8-bit precision between the sample and each of the current palette color is calculated. If the distortion for each of the components is less than a threshold value for the palette entry corresponding to the minimum SAD, the sample is added to the cluster belonging to the palette entry. Otherwise, the sample is added as a new palette entry. When the number of samples mapped to a cluster exceeds a threshold, a centroid for that cluster is updated and becomes the palette entry of that cluster.

In the next step, the clusters are sorted in a descending order of usage. Then, the palette entry corresponding to each entry is updated. Normally, the cluster centroid is used as the palette entry. But a rate-distortion analysis is performed to analyze whether any entry from the palette predictor may be more suitable to be used as the updated palette entry instead of the centroid when the cost of coding the palette entries is taken into account. Note the precision of the distortion part is 8-bit precision. This process is continued till all the clusters are processed or the maximum palette size is reached. Finally, if a cluster has only a single sample and the corresponding palette entry is not in the palette predictor, the sample is converted to an escape symbol. Additionally, duplicate palette entries are removed and their clusters are merged.

After palette derivation, each sample in the block is assigned the index of the minimal RD cost.

# Proposal

To further improve the palette coding efficiency on lossless, the following non-normative changes are proposed in the VTM-7.0:

- Firstly, it is proposed to fix precision of distortion during the derivation of the palette for lossless coding. In current VTM7, the palette distortion is scaled down to 8 bit scale. However, the precision should be the same as input bit depth for lossless coding.

- Secondly, it is proposed to remove the unnecessary calculation of distortion during palette coding RD decision on lossless. There is no distortion for lossless cases. It is reasonable to only consider no distortion cases used for rate-distortion analysis on lossless in encoder side.

# Simulation Results

The proposed methods were implemented based on the VTM-7.0 reference software [2] and tested using the lossless setting used in CE3 [3] and YUV444 sequences used in CE2 [4] are applied. The palette mode is turned on for all Classes in anchor.

Table 1: Results of proposed modification, DualITree = 1 for lossless VVC

|  | All Intra Main |  |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  |  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 20.1 | 21.3 | -6.13% |  | 42.5 | 44.6 | -3.35% | 62.1 | 64.7 | -2.69% |
| TGM 720p | 11.0 | 12.1 | -7.19% |  | 13.9 | 15.3 | -4.28% | 25.2 | 26.5 | -2.14% |
| Animation | 5.8 | 6.9 | -7.66% |  | 1.7 | 2.0 | -6.27% | 1.6 | 2.0 | -7.14% |
| Mixed content | 7.2 | 7.3 | -1.89% |  | 6.2 | 6.2 | -0.29% | 6.9 | 6.9 | -0.18% |
| Camera-Captured content | 1.9 | 1.9 | 0.00% |  | 0.3 | 0.3 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 9.2 | 9.9 | -4.57% |  | 12.9 | 13.7 | -2.84% | 19.2 | 20.1 | -2.43% |
| Enc Time[%] | 100% |  |  |  | 97% |  |  | 101% |  |  |
| Dec Time[%] | 93% |  |  |  | 97% |  |  | 93% |  |  |

Table 2: Results of proposed modification, DualITree = 0 for lossless VVC

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 26.8 | 27.4 | -2.46% | 54.1 | 55.0 | -1.39% | 75.9 | 76.9 | -1.11% |
| TGM 720p | 13.4 | 13.5 | -2.30% | 16.7 | 16.8 | -0.80% | 27.5 | 27.7 | -0.84% |
| Animation | 6.0 | 7.1 | -7.35% | 1.7 | 2.0 | -6.24% | 1.6 | 2.0 | -7.13% |
| Mixed content | 7.9 | 8.0 | -0.66% | 6.3 | 6.3 | -0.12% | 7.0 | 7.0 | -0.07% |
| Camera-Captured content | 1.8 | 1.8 | 0.00% | 0.3 | 0.3 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 11.2 | 11.6 | -2.56% | 15.8 | 16.1 | -1.71% | 22.5 | 22.8 | -1.83% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 101% |  |  | 95% |  |  |

# Conclusions

In this contribution, two encoder changes are proposed to enhance the VTM7 Palette mode (PLT) coding for lossless case. Simulation results show that the proposed changes do provide coding efficiency improvement for lossless coding. It is recommended that the proposed changes be adopted into VTM8.

# References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- T. Nguyen, T. C. Ma and A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, 16th meeting, Geneva, CH, 1–11 October 2019

- X. Xu, Y.-H. Chao, Y.-C. Sun and J. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).