| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0495 |
| --- | --- |

| Title: | AHG16: Simplified clip ranges for NL-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC, clipping operations are performed on each pixel in the non-linear ALF (NL-ALF) filtering process, with clipping bounary values derived in a form of 2f where f is a non-integer value. In this document, it is proposed to align the clipping boundary values to a form of 2N to facilitate the implementaion of such clipping operations. The results reportedly show that the proposed method has 0.00% (AI), 0.00%(RA) and 0.01%(LDB) BR-rate impact under CTC; 0.00% (AI), 0.00%(RA) and 0.01%(LDB) with IBDI set to 8, and -0.01% (AI), 0.00%(RA) and -0.02%(LDB) with IBDI set to 12.

# Introduction

In VVC, there are clipping operations involved in the ALF filtering process as shown in the filtering equation. Each sample R(i,j) within the CU is filtered, resulting in sample value R'(i,j) as shown below. Kx,y is the clipping function and c(k, l) denotes the decoded clipping parameters.

R'i,j=Ri,j+k≠0l≠0fk, l×KRi+k,j+l-Ri,j,ck, l+64≫7

And in current VVC, these clipping values are dependent of the internal bitdepth. More precisely, the Luma table of clipping values and Chroma table of clipping values are obtained by the following formula:

AlfClip=round2B-*n  for n∈0..N-1,

where AlfClip is clipping value, B refers to internal bit depth (IBDI) or the bit depth of input samples for ALF,  is a pre-defined constant value equal to 2.35.

Table 1 The clipping table in VVC

| IBDI | clip value index |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 255 | 50 | 10 | 2 |
| 9 | 511 | 100 | 20 | 4 |
| 10 | 1023 | 201 | 39 | 8 |
| 11 | 2047 | 402 | 79 | 15 |
| 12 | 4095 | 803 | 158 | 31 |
| 13 | 8191 | 1607 | 315 | 62 |
| 14 | 16383 | 3214 | 630 | 124 |
| 15 | 32767 | 6427 | 1261 | 247 |
| 16 | 65535 | 12855 | 2521 | 495 |

# Proposal

It is proposed to use 2N as the clipping upper-bound and lower-bound values where N is an integer value. With the proposed modification, the clipping can be implemented as logical or/and the most significant bits from the (N+1) bit to the most significant bit.

Table 2 The proposed clipping table

| IBDI | clip value index |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28 | 25 | 23 | 21 |
| 9 | 29 | 26 | 24 | 22 |
| 10 | 210 | 27 | 25 | 23 |
| 11 | 211 | 28 | 26 | 24 |
| 12 | 212 | 29 | 27 | 25 |
| 13 | 213 | 210 | 28 | 26 |
| 14 | 214 | 211 | 29 | 27 |
| 15 | 215 | 212 | 210 | 28 |
| 16 | 215 | 213 | 211 | 29 |

# Simulation Results

As shown in Table 3, the proposed method is implemented based on the VTM-7.0 reference software [1] and tested using the common test condition defined in [2]. As shown in Table 4 and Tabl5, additional tests are also conducted to see the coding performance of the proposed method when IBDI is 8 and 12, respectively.

Table 3 Results of the proposed method under CTC (IBDI=10)

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.01% | 99% | 101% |
| Class A2 | -0.01% | -0.01% | -0.02% | 95% | 100% |
| Class B | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.01% | -0.02% | 100% | 96% |
| Class E | 0.00% | 0.03% | 0.00% | 95% | 92% |
| Overall | 0.00% | 0.01% | -0.01% | 98% | 98% |
| Class D | 0.01% | -0.03% | -0.02% | 95% | 94% |
| Class F | 0.02% | -0.04% | -0.04% | 100% | 96% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.06% | 99% | 100% |
| Class A2 | -0.02% | 0.00% | 0.00% | 99% | 101% |
| Class B | -0.01% | -0.02% | -0.02% | 98% | 97% |
| Class C | 0.02% | -0.10% | 0.01% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.01% | 98% | 98% |
| Class D | -0.03% | -0.04% | -0.06% | 98% | 95% |
| Class F | -0.01% | -0.06% | -0.11% | 98% | 96% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.31% | -0.13% | 101% | 98% |
| Class C | -0.04% | -0.22% | -0.15% | 95% | 99% |
| Class E | 0.14% | 0.97% | 0.05% | 99% | 100% |
| Overall | 0.01% | 0.04% | -0.09% | 98% | 99% |
| Class D | -0.05% | 0.42% | 0.07% | 98% | 97% |
| Class F | 0.09% | -0.62% | 0.26% | 99% | 99% |

Table 4 Results of the proposed method when IBDI=8

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.06% | 100% | 97% |
| Class A2 | -0.01% | 0.00% | -0.01% | 104% | 102% |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 101% |
| Class C | -0.01% | 0.01% | -0.02% | 99% | 104% |
| Class E | 0.00% | 0.03% | -0.01% | 96% | 95% |
| Overall | 0.00% | 0.02% | -0.02% | 99% | 100% |
| Class D | 0.00% | -0.03% | -0.02% | 106% | 106% |
| Class F | 0.01% | -0.10% | -0.14% | 99% | 97% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.17% | 0.09% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | -0.06% | 100% | 101% |
| Class B | 0.00% | 0.17% | 0.07% | 101% | 99% |
| Class C | 0.00% | 0.09% | 0.05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.12% | 0.04% | 101% | 100% |
| Class D | 0.05% | 0.04% | 0.09% | 100% | 98% |
| Class F | 0.01% | -0.06% | -0.10% | 99% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.14% | -0.18% | 102% | 106% |
| Class C | -0.05% | 0.04% | -0.18% | 101% | 101% |
| Class E | 0.09% | -0.01% | 0.33% | 101% | 99% |
| Overall | 0.01% | 0.07% | -0.05% | 101% | 102% |
| Class D | -0.03% | -0.24% | 0.04% | 100% | 101% |
| Class F | 0.07% | -0.56% | -0.61% | 98% | 97% |

Table 5 Results of the proposed method when IBDI=12

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.03% | 103% | 102% |
| Class A2 | -0.02% | -0.02% | -0.02% | 101% | 103% |
| Class B | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Class C | -0.01% | 0.00% | -0.03% | 102% | 103% |
| Class E | 0.00% | 0.03% | -0.01% | 101% | 101% |
| Overall | -0.01% | 0.00% | -0.02% | 101% | 102% |
| Class D | 0.00% | -0.03% | -0.03% | 101% | 101% |
| Class F | 0.02% | -0.05% | -0.04% | 105% | 108% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.17% | 99% | 97% |
| Class A2 | -0.02% | 0.00% | 0.06% | 99% | 100% |
| Class B | 0.00% | -0.07% | -0.13% | 99% | 99% |
| Class C | 0.00% | -0.07% | 0.05% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.05% | 99% | 99% |
| Class D | -0.02% | 0.04% | -0.13% | 101% | 102% |
| Class F | 0.02% | -0.02% | -0.18% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.09% | -0.15% | 103% | 102% |
| Class C | -0.07% | -0.06% | -0.09% | 101% | 98% |
| Class E | 0.00% | -0.02% | -0.93% | 99% | 99% |
| Overall | -0.02% | -0.06% | -0.33% | 101% | 100% |
| Class D | 0.09% | -0.18% | -0.57% | 96% | 100% |
| Class F | -0.02% | -0.41% | -0.14% | 98% | 99% |

# References

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, 16th meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).