| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0499 |
| --- | --- |

| Title: | On LFNST signaling with Transform-skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Lane, Suite 400, San Diego, CA 92122, USA | Tel: Email: | 1-310-567-2937 {tsung-chuanma, xiaoyuxiu, yiwenchen, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC [1], LFNST index is not signaled if luma CU is Transform-Skip mode. Two issues are addressed in this contribution. First, in dual tree case, even chroma CU is TS mode, LFNST index for chroma CU is still signaled if its corresponding luma CU is non-TS mode. Second, in single tree case, LFNST can not be applied with chroma once its luma block is TS mode.

In this contribution, it is proposed to modify LFNST signaling with TS mode. First, in dual tree case, chroma LFNST index is not signaled if it is applied with chroma TS mode. Second, in single tree case, LFNST index is not signaled if all Y/Cb/Cr blocks are TS mode.

Simulation results show that the proposed changes result in a BD-rate change (YUV) of 0.00%, 0.00%, 0.00% for AI, XXX%, XXX%, XXX% for RA, XXX%, XXX%, XXX% for LDB.

# Introduction

In current VVC [1], LFNST index is not signaled if luma block is TS mode. There are two issues asserted in current design. First, if in dual tree case, chroma LFNST index is still signaled with chroma TS mode if its corresponding luma CU is non-TS mode. In such case, it is a redundant signaling. In a correct design, chroma LFNST index should not be signaled with chroma TS mode. Second, if in single tree case, LFNST can not be applied with chroma if its corresponding luma block is TS mode. Since LFNST index is not signaled if luma is TS mode, LFNST for chroma block is prohibited in current VVC design if its corresponding luma block is TS mode. In a correct design, LFNST should be applied with chroma non-TS mode even its corresponding luma block is TS mode.

In this contribution, it is proposed to modify LFNST index signaling with TS mode. First, in dual tree case, chroma LFNST index is not signaled if it is applied with chroma TS mode. Second, in single tree case, LFNST index is not signaled if all Y/Cb/Cr blocks are TS mode. Therefore, LFNST can be applied for chroma even if its corresponding luma block is TS mode.

# Proposal

In this contribution, it is proposed following change to WD.

Table 2. Proposed change for WD

isTSMode = ( treeType != SINGLE_TREE ? ( treeType == DUAL_TREE_CHROMA ? transform_skip_flag[ x0 ][ y0 ][ 1 ] && transform_skip_flag[ x0 ][ y0 ][ 2 ] : transform_skip_flag[ x0 ][ y0 ][ 0 ]) : transform_skip_flag[ x0 ][ y0 ][ 0 ] && transform_skip_flag[ x0 ][ y0 ][ 1 ] && transform_skip_flag[ x0 ][ y0 ][ 2 ]

if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && isTSMode = = 0 transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) {

if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 )

lfnst_idx

# Simulation results

The proposed changes were implemented on top of VTM-7.0 [2] and tested following the test conditions used in [3]. The highlight part will be updated.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.02% | 101% | 102% |
| Class A2 | xxx% | xxx% | xxx% | xxx% | xxx% |
| Class B | 0.00% | 0.00% | 0.05% | 100% | 100% |
| Class C | 0.02% | -0.11% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | xxx% | xxx% | xxx% | xxx% | xxx% |
| Class D | 0.01% | -0.19% | -0.10% | 99% | 99% |
| Class F | 0.01% | 0.02% | 0.03% | 100% | 98% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | xxx% | xxx% | xxx% | xxx% | xxx% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | xxx% | xxx% | xxx% | xxx% | xxx% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class F | xxx% | xxx% | xxx% | xxx% | xxx% |

# Conclusion

In this contribution, changes are proposed to address two issues in signaling LFNST index with TS mode. The proposed changes result in minor impact on coding performance. It is recommended to adopt proposed change into VVC.

# Reference

- [1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- [2] VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- [3] F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).