| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0501-v2 |
| --- | --- |

| Title: | Non-CE2: On palette predictor initialization in WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 7, palette predictor will be reset to 0 at the beginning of each CTU row if Wavefront Parallel Processing (WPP) is enabled. This proposal proposes to unify the initialization scheme of palette predictors used in WPP with CABAC synchronization:  The palette predictor at the beginning of each CTU row will be initialized to the predictor derived from the first CTU in the previous CTU row.

The results on YUV4:4:4 sequences compared to WPP (–WaveFrontSynchro=1) in VTM7.0 are as follows:

Dual tree OFF:  -0.33% AI, -0.18 % RA, -0.15% LB

Dual tree ON: -0.06% AI, -0.08 % RA, -0.02 % LB

# Introduction

Figure 1: Palette predictor initialization for WPP

Figure 1 shows the proposed palette predictor initialization for WPP. P0 ~ P3 are palette predictors for each CTU rows. P0 will be updated after finishing processing the CTU C(0, 0).The updated P0 will be used to initialize predictor P1 in the next CTU row.

# Simulation Results

The proposed method is evaluated according to the CE2 description [1] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM (=2), PLT are turned on for all Classes. The anchor is VTM7.0 with –WaveFrontSynchro=1.

## Dual Tree OFF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.62% | -0.73% | -0.67% | 100% | 100% |
| TGM 720p | -0.28% | -0.50% | -0.48% | 100% | 101% |
| Animation | -0.40% | -0.85% | -0.98% | 101% | 100% |
| Mixed content | -0.17% | -0.06% | -0.17% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.33% | -0.48% | -0.50% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.25% | -0.41% | -0.30% | 100% | 100% |
| TGM 720p | -0.20% | -0.43% | -0.49% | 100% | 101% |
| Animation | -0.21% | -0.54% | -0.65% | 99% | 100% |
| Mixed content | -0.13% | -0.29% | -0.12% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.18% | -0.37% | -0.34% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | -0.24% | -0.20% | 100% | 100% |
| TGM 720p | -0.42% | -0.49% | -0.76% | 101% | 100% |
| Animation | -0.07% | -0.22% | -0.03% | 97% | 100% |
| Mixed content | -0.06% | 0.19% | -0.05% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.15% | -0.19% | -0.26% | 99% | 100% |

## Dual Tree ON

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.10% | -0.06% | 100% | 100% |
| TGM 720p | -0.12% | 0.00% | -0.01% | 100% | 101% |
| Animation | -0.07% | -0.05% | 0.04% | 100% | 101% |
| Mixed content | -0.03% | -0.06% | 0.02% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Overall | -0.06% | -0.05% | -0.01% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.14% | -0.20% | -0.10% | 100% | 99% |
| TGM 720p | -0.05% | -0.07% | -0.07% | 100% | 98% |
| Animation | -0.08% | 0.01% | 0.10% | 101% | 100% |
| Mixed content | -0.09% | -0.03% | -0.04% | 100% | 100% |
| Camera-Captured content | 0.00% | -0.02% | 0.02% | 100% | 102% |
| Overall | -0.08% | -0.07% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.15% | -0.10% | 100% | 100% |
| TGM 720p | 0.13% | 0.04% | -0.13% | 101% | 101% |
| Animation | -0.02% | -0.13% | -0.11% | 102% | 100% |
| Mixed content | -0.10% | -0.10% | 0.02% | 99% | 100% |
| Camera-Captured content | -0.01% | 0.03% | -0.05% | 100% | 100% |
| Overall | -0.02% | -0.06% | -0.08% | 100% | 100% |

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, October 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).