| - Joint Video Experts Team (JVET) - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0501-WD |
| --- | --- |

| Title: | Non-CE2: On palette predictor initialization in WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

9.3	CABAC parsing process for slice data

9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.9.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

- When ending the parsing of the CTU syntax in clause 7.3.9.2, entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in clause 9.3.2.3 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, and TableMpsValWpp as outputs.

The storage process for palette mode predictor is applied as follows:

- When ending the parsing of the CTU syntax in clause 7.3.9.2, entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette mode predictor as specified in clause 9.3.2.6 is invoked with arrays tablePaletteSizeWPP and tablePaletteEntriesWPP as ouput.

The whole CABAC parsing process for a syntax element synEl is illustrated in Figure 11.

9.3.2	Initialization process

9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ compIdx ], with compIdx = 0, 1, is initialized to 0.

–	Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1488)

–	The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, TableMpsValWpp as inputs and the synchronization process for palette predictor as specified in clause 9.3.2.7 is invoked with array tablePaletteSizeWPP and tablePaletteEntriesWPP as inputs.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ compIdx ], with compIdx = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the array PredictorPaletteSize[ compIdx ], with compIdx = 0, 1, is initialized to 0. [Ed. (YK): Check whether this can be merged with the condition "If the CTU is the first CTU in a brick", as the operation is the same. This is also the case in HEVC.]

The decoding engine registers ivlCurrRange and ivlOffset both in 16 bit register precision are initialized by invoking the initialization process for the arithmetic decoding engine as specified in subclause 9.3.2.5.

9.3.2.6	Storage process for palette predictor

Inputs to this process are:

–	palette predictor size in array PredictorPaletteSize[ Idx ], with Idx = 0, 1

–	palette predictor in array PredictorPaletteEntries[ Idx ], with Idx = 0, 1, 2

Outputs of this process are:

–	The arrays tablePaletteSizeWPP, and tablePaletteEntriesWPP containing the values in array PredictorPaletteSize and PredictorPaletteEntries in clause 7.4.10.6 and 8.4.5.3

The storage process for context variables is illustrated as follows:

- If the current CTU is the first CTU in tile row and if entropy_coding_sync_enabled_flag equal to 1, involke the storage of palette predictor in arrays tablePaletteSizeWPP and tablePaletteEntriesWPP.

- Otherwise, arrays tablePaletteSizeWPP and tablePaletteEntriesWPP are not updated.

9.3.2.7	Synchronization process for palette predictor

Inputs to this process are:

–	The arrays tablePaletteSizeWPP, and tablePaletteEntriesWPP containing the values of the palette predictor size and entries used in the storage process of palette predictor.

Outputs of this process are:

–	The initialized palette predictor size and entries in array PredictorPaletteSize and PredictorPaletteEntries.

For each values in array PredictorPaletteSize and PredictorPaletteEntries are initialized to the corresponding entries in arrays tablePaletteSizeWPP and tablePaletteEntriesWPP.

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th meeting, Geneva, CH, October 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).