| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0504-v1 |
| --- | --- |

| Title: | CE2-related: Palette mode for non 4:4:4 color format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Mohammed Golam Sarwer, Jie Chen, Yan Ye, Jiancong Luo (Alibaba) Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | m.sarwer@alibaba-inc.com ruling.lrl@alibaba-inc.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Inc, Alibaba group |  |  |

_____________________________

- Abstract

This contribution proposes to enable the palette mode for non 4:4:4 color format including 4:2:0, 4:2:2 and monochrome color formats. To support the palette mode for non 4:4:4 color format, two modifications are made to VVC draft 7. Firstly, for a sample only containing luma component, only the luma value is signaled if the sample is coded using escape mode. This is the same as in the HEVC SCC extension. Secondly, for a local dual tree block, a default value, defined as (1 << (component bit depth - 1)), is set to the new palette entry before the palette predictor update. It is reported that, for class SCC in 4:2:0 color format, the overall performance impact for {Y, U, V} is {-6.35%, -6.21%, -5.88%} for AI, {-3.96%,-3.68%, -3.53%} for RA and {%, %, %} for LB.

Introduction

In VVC draft 7, the palette mode is only allowed for 4:4:4 color format. However, in current market of video industries, almost all mass-market consumer video content uses the 4:2:0 chroma sub-sampling format, including video conferencing of which screen sharing is a critical functionality. In order to have full benefits of palette mode, the extension of the palette mode for 4:2:0 color format is necessary. This allows VVC to have the same functionality as the HEVC screen content extension, where the palette mode is already allowed for the 4:2:0 color format. Therefore, this contribution proposes to allow the palette mode for the 4:2:0 color format. Besides, it is also proposed to enable the palette mode for 4:2:2 and monochrome color format.

Proposed methods

In order to apply the palette mode in VVC draft 7 for non 4:4:4 color format, two parts need to be fixed. Firstly, when signaling the escape value, only the luma value is signaled for a sample only containing luma component. For example, in the 4:2:0 color format, only luma value is signaled for a sample location not located at top-left position of each 2x2 luma sample area. Secondly, for a local dual tree block, the palette mode is applied to the block in the same way as the palette mode applied to a single tee block except that the process of palette predictor update is slightly modified. Since the local dual tree block may only contain luma(chroma) component, the value of luma(chroma) component is signaled and a default value is set to the chroma(luma) component for the new palette entry before reordering the palette predictor. The default value is set to be (1 << (component bit depth - 1)) in this contribution.

For palette mode in monochrome format, a bugfix is required which sets the number of colour components in a palette coded block to 1 for monochrome format (instead of always setting it to 3 as in VVC draft 7).

Simulation results

Table 1 shows the simulation results of the palette mode for 4:2:0 color format.

Table 1. Simulation results of the palette mode

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.24% | -0.68% | -0.66% | 106% | 98% |
| Class SCC | -6.35% | -6.21% | -5.88% | 102% | 92% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.35% | -0.55% | -0.56% | 106% | 100% |
| Class SCC | -3.96% | -3.68% | -3.53% | 104% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class SCC | -1.42% | -1.28% | -1.27% | 107% | 100% |

Conclusion

In order to improve the coding efficiency for the screen content in non 4:4:4 color format, this contribution proposes applying the palette mode to non 4:4:4 color format. The results show that 6.35% BD rate reduction can be achieved with reasonable encoding time increase. Considering the interesting coding performance, and the importance of 4:2:0 screen content coding in real-world video applications, we suggest to adopt this contribution to the next version of VVC draft.

Specification changes

To apply the palette mode to non 4:4:4 color format, the changes to the VVC draft 7 are highlighted in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_format_idc = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| if( chroma_format_idc = = 3 ) { |  |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| palettePredictionFinished = 0 |  |
| … |  |
| if( palette_escape_val_present_flag ) { |  |
| for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) |  |
| for( sPos = minSubPos; sPos < maxSubPos; sPos++ ) { |  |
| xC = x0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 0 ] |  |
| yC = y0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 1 ] |  |
| if( !( treeType = = SINGLE_TREE && cIdx != 0 && xC % SubWidthC != 0 && yC % SubHeightC != 0 ) ) { |  |
| if( PaletteIndexMap[ cIdx ][ xC ][ yC ] = = MaxPaletteIndex ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| … |  |

8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location ( xCbComp, yCbComp ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current coding block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of treeType, the variables startComp and numComps are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(444)

numComps = ChromaArrayType  = =  0 ? 1 : 3		(445)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(446)

numComps = 1		(447)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(448)

numComps = 2		(449)

…

The variable localDualtree is derived as follows:

localDualtree = treeType != SINGLE_TREE && ( slice_type != I | | ( slice_type = = I &&

qtbtt_dual_tree_intra_flag = = 0 ) ) ? 1 : 0

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 2;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

If localDualtree is equal to 1, the following applies:

  - If treeType is equal to DUAL_TREE_LUMA

for( i = 0; i < num_signalled_palette_entries[ startComp ]; i++ ) {
        CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
        CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
}

  - Otherwise, if treeType is equal to DUAL_TREE_CHROMA

for( i = 0; i < num_signalled_palette_entries[ startComp ]; i++ ) {
	         CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
        }

  - CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]

  - startComp = 0

  - numComps = 3

The array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ )
	for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )
		newPredictorPaletteEntries[ compIdx ][ i ] = CurrentPaletteEntries[ compIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize[ startComp ]
for( i = 0; i < PredictorPaletteSize[ startComp ]  &&  newPredictorPaletteSize < 63; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )	(460)
			newPredictorPaletteEntries[ compIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ compIdx ][ i ]
		newPredictorPaletteSize++
	}
for( compIdx = startComp; compIdx < ( startComp + numComps ); compIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ compIdx ][ i ] = newPredictorPaletteEntries[ compIdx ][ i ]
PredictorPaletteSize[ startComp ] = newPredictorPaletteSize

If qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies:

PredictorPaletteSize[ 1 ] = newPredictorPaletteSize

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to 63, inclusive.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).