| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0506 |
| --- | --- |

| Title: | Interaction between ACT and cross-component coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Zhang, Xiang Li, Ling Li, Guichun Li, Shan Liu | Tel: Email: | hobingzhang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

Abstract

This contribution reports the interactions between the Adaptive Color Transform (ACT) and JCCR. Specifically, there is an overlap between JCCR and ACT in terms of reducing the inter-component dependency among residuals. It is proposed to disable JCCR when the ACT is applied at CU/SPS level. Experimental results reportedly show that on 4:4:4 YUV and RGB sequences the BD-rate changes are as follows:

- YUV: AI (0.00%/0.01%/0.00%), RA (0.03%/0.03%/0.02%) for Y/Cb/Cr, respectively;

- RGB: AI (0.11%0.15%/0.08%), RA (0.18%/0.17%/0.03%) for G/B/R, respectively.

In addition, the encoder and decoder complexity reduction on RGB is also reported as 6%/3%, 5%/0% for encoding/decoding time under AI and RA configurations, respectively.

- Introduction

In VVC draft 7, the newly adopted adaptive color transform (ACT) is used to enhance the coding efficiency of 444 RGB videos. The coding gain of ACT is provided by that the color space conversion (YCgCo) is carried out in the residue domain and a CU-level flag is signalled to indicate the usage of the color-space transform. For inter and IBC CUs, the ACT is only enabled when there is at least one non-zero coefficient in the CU. For intra CUs, the ACT is only enabled when chroma components select the same intra prediction mode of luma component, i.e., DM mode.

The residual signals in YCgCo domain are verified to be good in decorrelating RGB into luminance, offset orange and offset green [1]. This may degrade the performance of cross-component coding tools. A typical example is the Joint Cb Cr Residual coding (JCCR) mode where a flag at TU level is signaled to indicate whether the chroma residuals are coded jointly in single residual_coding() or not.

- Proposed methods

This contribution proposes that ACT and JCCR cannot be used simultaneously for a block. Solution is proposed as follow:

- Solution: skip the signaling of JCCR flag but referred it as off at TU level if ACT is enabled at CU level.

The spec texts of the two solutions may be found in section 5.

- Experimental results

The proposed solution has been implemented into VTM7.1 platform. Tests were conducted on 4:4:4 RGB and YUV videos. VTM7.1 is used as anchor for performance evaluation. The test condition follows AI and RA configurations [2], and additional parameters are listed as follows:

- DualITree = 0: Dual tree is off for I slices.

- ColorTransform = 1: Adaptive color transform is on.

Table 1 reports the SCC results of proposed solution in AI and RA configurations. It can be observed that the proposed solution has minor impact on YUV, i.e., 0.01%/0.01%/0.00%, 0.04%/0.04%/-0.01% for Y/Cb/Cr under AI and RA, respectively. For RGB sequences, disabling JCCR when ACT is on at CU level shows the BD-rate loss is minor, i.e., 0.12%0.15%/0.08%, 0.19%/0.17%/0.04% for R/G/B under AI and RA, respectively.

Table 1. SCC test results of the proposed solution on top of VTM-7.1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | EncT | DecT |
| RGB, text & graphics with motion, 1080p & 720p | 0.24% | 0.21% | 0.15% | 95% | 97% |
| RGB, mixed content, 1440p & 1080p | 0.04% | 0.11% | 0.04% | 94% | 98% |
| RGB, Animation, 720p | 0.07% | 0.18% | 0.05% | 94% | 99% |
| RGB, camera captured, 1080p | 0.11% | 0.11% | 0.07% | 93% | 98% |
| YUV, text & graphics with motion, 1080p & 720p | 0.01% | 0.00% | 0.00% | 99% | 95% |
| YUV, mixed content, 1440p & 1080p | 0.01% | 0.03% | 0.01% | 99% | 100% |
| YUV, Animation, 720p | 0.00% | 0.01% | 0.00% | 99% | 98% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% | 99% | 97% |
| RGB, average | 0.11% | 0.15% | 0.08% | 94% | 97% |
| YUV, average | 0.00% | 0.01% | 0.00% | 99% | 97% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | EncT | DecT |
| RGB, text & graphics with motion, 1080p & 720p | 0.36% | 0.27% | 0.14% | 95% | 99% |
| RGB, mixed content, 1440p & 1080p | 0.04% | 0.17% | 0.04% | 95% | 100% |
| RGB, Animation, 720p | -0.02% | 0.12% | -0.03% | 96% | 99% |
| RGB, camera captured, 1080p | 0.36% | 0.12% | -0.01% | 95% | 100% |
| YUV, text & graphics with motion, 1080p & 720p | 0.02% | 0.01% | 0.03% | 100% | 101% |
| YUV, mixed content, 1440p & 1080p | 0.07% | 0.04% | 0.10% | 100% | 99% |
| YUV, Animation, 720p | 0.03% | 0.04% | -0.04% | 99% | 99% |
| YUV, camera captured, 1080p | 0.02% | 0.03% | 0.00% | 100% | 100% |
| RGB, average | 0.18% | 0.17% | 0.03% | 95% | 100% |
| YUV, average | 0.03% | 0.03% | 0.02% | 100% | 100% |

Additionally, Table 2 reports the SCC results of disabling JCCR while ACT is on under AI and RA configurations [2]. Compared with Table 1 and Table 2, it can be observed that most gain of JCCR is kept in our proposed solution. Importantly, it can be observed in Table 1 that the proposed solution can achieves 6% and 5% encoding time saving under AI and RA in RGB; 3% decoding time saving under AI for YUV and RGB sequences. These saving benefits from the skipping of JCCR checking when ACT is on at CU level. Disabling JCCR when ACT is on at CU level presents a good trade-off between BD-rate changes and complexity reduction in RGB videos.

Table 2. VTM7.1 SCC test results of disabling JCCR while ACT is on.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | Enc Time | Dec Time |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | 0.5% | 0.4% | 0.92 | 1.00 |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.0% | -0.1% | 0.92 | 0.98 |
| RGB, Animation, 720p | 0.8% | 0.0% | 0.0% | 0.91 | 1.01 |
| RGB, camera captured, 1080p | 2.5% | 0.4% | -1.5% | 0.91 | 1.01 |
| YUV, text & graphics with motion, 1080p & 720p | 0.7% | 0.0% | 0.3% | 0.94 | 1.00 |
| YUV, mixed content, 1440p & 1080p | 1.3% | 1.2% | 1.1% | 0.94 | 0.99 |
| YUV, Animation, 720p | 2.1% | 7.1% | 4.4% | 0.96 | 1.03 |
| YUV, camera captured, 1080p | 2.8% | 0.7% | -1.2% | 0.96 | 1.04 |
| RGB Average | 1.07% | 0.25% | -0.28% | 92% | 100% |
| YUV Average | 1.73% | 2.27% | 1.13% | 95% | 101% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | Enc Time | Dec Time |
| RGB, text & graphics with motion, 1080p & 720p | 0.8% | 0.8% | 0.7% | 0.92 | 1.00 |
| RGB, mixed content, 1440p & 1080p | 0.4% | -0.1% | -0.2% | 0.94 | 0.98 |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% | 0.93 | 1.01 |
| RGB, camera captured, 1080p | 2.0% | 1.2% | 0.0% | 0.94 | 1.01 |
| YUV, text & graphics with motion, 1080p & 720p | 0.5% | 0.1% | 0.3% | 0.95 | 1.00 |
| YUV, mixed content, 1440p & 1080p | 1.1% | 0.7% | 0.9% | 0.96 | 0.99 |
| YUV, Animation, 720p | 2.1% | 6.8% | 5.5% | 0.97 | 1.03 |
| YUV, camera captured, 1080p | 0.9% | 0.4% | -0.4% | 0.97 | 1.04 |
| RGB Average | 0.80% | 0.44% | 0.10% | 93% | 100% |
| YUV Average | 1.15% | 2.01% | 1.55% | 96% | 101% |

- Conclusions

From the results shown in Table 1 it can be concluded there is overlap between ACT and JCCR. Disallowing JCCR when ACT is on at block level shows better trade-off between coding efficiency and complexity. Therefore, it is proposed to adopt the proposed method.

- Spec text

The following proposed text modifications are based on JVET-O2001-vE. The added texts are highlighted in yellow, and deleted texts are marked by red strikethrough.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( ( CbWidth[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > 64 ¦ ¦ ( chromaAvailable && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ) && treeType != DUAL_TREE_LUMA ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ¦ ¦ ( tu_cbf_cb[ xC ][ yC ] && tu_cbf_cr[ xC ][ yC ] ) ) && chromaAvailable && !cu_act_enabled_flag) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| …… |  |
| } |  |

- References

- H. S. Malvar and G. J. Sullivan, “Transform, Scaling & Color Space Impact of Professional Extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-H031, Geneva, May 2003.

- X. Z. Xu, Y.-H. Chao, Y.-C. Sun and J. Z. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding ” JVET of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 Document: JVET-P2022-v3, Geneva, CH, 1–11 October 2019

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).