| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0509-v1 |
| --- | --- |

| Title: | CE2-1.1-related: Encoder optimization with adjusted palette clustering steps |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Wang, JunruLi, Li Zhang, Jizheng Xu, Kai Zhang, Hongbin Liu, Shiqi Wang | Tel: Email: | 1-858-999-7093 wangmeng.kids@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents supplemental results of CE2-1.1 with modified palette clustering steps. More specifically, the clustering steps are refined on the premise of escape sample coding method proposed in CE2-1.1. Two methods are presented:

Method #1: One modified table is proposed for the relationship between clustering step and quantization parameters.

Method #2: Based on method #1, an additional table is proposed for chroma components when dual tree is enabled.

The proposed methods are implemented and evaluated with CE2-1.1 following the CE2 test conditions. When compared to VTM-7.0, experimental results are reported as follows with almost no running time changes:

Method #1 under low QPs = {2, 7, 12, 17}

Dual tree off: {-3.09%, -3.52%, -3.57%} {-2.86%, -2.55%, -2.58%}, {-1.91%, -2.31%, -2.36%} for AI, RA, and LDB, respectively.

Dual tree on: {-2.69%, -2.42%, -2.49%}, {-3.01%, -1.83%, -1.86%}, {-1.75%, -1.83%, -1.87%} for AI, RA, and LDB, respectively.

Method #1 under CTC QPs = {22, 27, 32, 37}

Dual tree off: { -0.14%, -0.47%, -0.49%}, {-0.31%, -1.08%, -1.02%}, {-0.31%, -0.44%, -0.44%} for AI, RA, and LDB, respectively.

Dual tree on: {0.24%, -0.69%, -0.66%}, {0.03%, -1.69%, -1.72%}, {-0.22%, -0.80%, -0.83%} for AI, RA, and LDB, respectively.

Method #2 under low QPs = {2, 7, 12, 17}

Dual tree on: {-2.69%, -2.42%, -2.49%}, {-3.01%, -1.83%, -1.86%}, {} for AI, RA, and LDB, respectively.

Method #2 under CTC QPs = {22, 27, 32, 37}

Dual tree on: {-0.32%, -0.15%, -0.20%}, {-0.23%, -1.04%, -1.13%}, {} for AI, RA, and LDB, respectively.

# Introduction

With Palette prediction mode, dominant colors are clustered as palette and coded with entry indices and color values. Escape samples are quantized, binarized and signaled. Palette generation is crucial to the coding performance. Similar colors can be grouped into one entry if the absolute difference between the sample color and the entry color is smaller than a preset step. In VTM-7.0 [1], palette clustering steps are represented with a look-up-table indexed by a base-QP. The original g_paletteQuant in VTM-7.0 with respect to different QPs is illustrated in Table 1-1.

Table 1-1: g_paletteQuant in VTM-7.0

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g_paletteQuant | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| g_paletteQuant | 5 | 6 | 6 | 7 | 7 | 8 | 9 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 19 | 21 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
| g_paletteQuant | 22 | 24 | 23 | 25 | 26 | 28 | 29 | 31 | 32 | 34 | 36 | 37 | 39 | 41 | 42 | 45 |  |  |

# Proposed methods

It is proposed to adjust the palette clustering steps according to the estimated cost of escape-coded samples. In CE2-1.1, fixed length binarization is investigated for escape sample coding, which reveals benefits in handling the uniform distribution signal [2]. This proposal refines palette clustering step sizes in accordance with CE2-1.1.

Two methods are proposed. In method #1, the proposed pltQstep is illustrated in Table 2-1. In method #2, an additional table is proposed for chroma components when dual tree is enabled. More specifically, the clustering steps for chroma components can be further adjusted according to the table for chroma components illustrated in Table 2-2 to balance the rate and distortion behavior in dual-tree cases. In other cases, i.e. for the component or when dual-tree is off, Table 2-1 is used as in method #1.

Table 2-1: The proposed g_paletteQuant.

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g_paletteQuant | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| g_paletteQuant | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 11 | 12 | 13 | 15 | 17 | 17 | 19 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
| g_paletteQuant | 21 | 24 | 27 | 30 | 27 | 31 | 35 | 39 | 44 | 49 | 39 | 44 | 49 | 55 | 62 | 70 |  |  |

Table 2-2: The proposed g_paletteQuant for chroma components, dual-tree on.

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g_paletteQuant | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| g_paletteQuant | 4 | 4 | 5 | 6 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 15 | 17 | 19 | 19 | 21 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
| g_paletteQuant | 24 | 27 | 30 | 34 | 31 | 35 | 39 | 44 | 49 | 55 | 44 | 49 | 55 | 62 | 70 | 78 |  |  |

# Simulation results

The proposed methods are implemented and evaluated with CE2-1.1 [2] over VTM-7.0 following the CE2 test conditions [3]. Simulation results are illustrated from Table 3-1 to Table 3-6. More detailed results could be found in the attached excel sheets.

Table 3-1. Coding performance of the proposed Method#1 with dual tree off, CTC QPs.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.10% | -0.57% | -0.70% | 100% | 101% |
| TGM 720p | -0.44% | -1.02% | -0.94% | 101% | 100% |
| Animation | 0.07% | 0.02% | 0.05% | 98% | 102% |
| Mixed content | -0.09% | -0.39% | -0.48% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.14% | -0.47% | -0.49% | 100% | 101% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.11% | -1.75% | -1.38% | 98% | 98% |
| TGM 720p | -0.78% | -1.68% | -1.73% | 98% | 101% |
| Animation | 0.05% | 0.05% | 0.04% | 100% | 98% |
| Mixed content | -0.51% | -1.21% | -1.30% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Overall | -0.31% | -1.08% | -1.02% | 99% | 99% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.18% | -0.53% | -0.53% | 96% | 97% |
| TGM 720p | -0.70% | -0.75% | -1.00% | 100% | 97% |
| Animation | -0.02% | 0.14% | 0.16% | 100% | 100% |
| Mixed content | -0.44% | -0.79% | -0.46% | 97% | 98% |
| Camera-Captured content | 0.00% | -0.03% | 0.00% | 98% | 98% |
| Overall | -0.31% | -0.44% | -0.44% | 98% | 98% |

(c) under LDB configurations

Table 3-2. Coding performance of the proposed Method#1 with dual tree on, CTC QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.43% | -0.79% | -0.78% | 103% | 102% |
| TGM 720p | 0.21% | -1.06% | -1.14% | 100% | 101% |
| Animation | 0.05% | -0.14% | -0.08% | 102% | 102% |
| Mixed content | 0.39% | -1.04% | -0.87% | 102% | 99% |
| Camera-Captured content | 0.00% | -0.01% | -0.01% | 93% | 99% |
| Overall | 0.24% | -0.69% | -0.66% | 101% | 101% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.04% | -2.14% | -1.94% | 96% | 97% |
| TGM 720p | -0.06% | -2.00% | -2.61% | 98% | 97% |
| Animation | 0.05% | -0.02% | -0.07% | 98% | 98% |
| Mixed content | 0.26% | -3.45% | -3.02% | 98% | 98% |
| Camera-Captured content | -0.01% | 0.01% | 0.00% | 98% | 99% |
| Overall | 0.03% | -1.69% | -1.72% | 97% | 98% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.22% | -0.83% | -0.77% | 95% | 97% |
| TGM 720p | -0.57% | -0.87% | -1.04% | 99% | 100% |
| Animation | 0.01% | 0.04% | -0.09% | 99% | 98% |
| Mixed content | -0.12% | -1.96% | -1.87% | 99% | 100% |
| Camera-Captured content | 0.02% | -0.09% | -0.05% | 99% | 101% |
| Overall | -0.22% | -0.80% | -0.83% | 98% | 99% |

(c) under LDB configurations

Table 3-3. Coding performance of the proposed Method#1 with dual tree off, low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -5.25% | -5.90% | -5.45% | 100% | 96% |
| TGM 720p | -4.20% | -3.66% | -4.15% | 103% | 98% |
| Animation | -1.48% | -3.09% | -3.19% | 99% | 98% |
| Mixed content | -2.42% | -2.94% | -3.07% | 100% | 97% |
| Camera-Captured content | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Overall | -3.09% | -3.52% | -3.57% | 100% | 97% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -4.50% | -4.16% | -3.72% | 99% | 97% |
| TGM 720p | -4.31% | -3.20% | -3.37% | 97% | 95% |
| Animation | -1.48% | -2.27% | -2.05% | 99% | 99% |
| Mixed content | -2.05% | -1.50% | -2.23% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.01% | -0.01% | 98% | 97% |
| Overall | -2.86% | -2.55% | -2.58% | 98% | 97% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.81% | -3.32% | -3.05% | 99% | 96% |
| TGM 720p | -3.17% | -2.85% | -3.14% | 99% | 99% |
| Animation | -0.86% | -2.16% | -2.03% | 100% | 96% |
| Mixed content | -1.36% | -1.95% | -2.30% | 99% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | -1.91% | -2.31% | -2.36% | 99% | 98% |

(c) under LDB configurations

Table 3-4. The performance of the proposed Method#1 with dual tree on, low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -4.13% | -4.54% | -4.61% | 101% | 97% |
| TGM 720p | -4.15% | -2.97% | -2.92% | 103% | 93% |
| Animation | -0.76% | -0.65% | -0.80% | 100% | 99% |
| Mixed content | -2.55% | -2.27% | -2.41% | 102% | 96% |
| Camera-Captured content | -0.01% | -0.01% | 0.00% | 99% | 99% |
| Overall | -2.69% | -2.42% | -2.49% | 101% | 96% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -3.97% | -3.80% | -3.47% | 99% | 98% |
| TGM 720p | -5.36% | -1.49% | -1.87% | 98% | 100% |
| Animation | -1.26% | -1.87% | -1.67% | 102% | 100% |
| Mixed content | -2.31% | -0.85% | -1.11% | 100% | 99% |
| Camera-Captured content | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Overall | -3.01% | -1.83% | -1.86% | 100% | 100% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.49% | -3.03% | -2.87% | 100% | 98% |
| TGM 720p | -2.87% | -2.36% | -2.59% | 101% | 102% |
| Animation | -1.14% | -1.53% | -1.54% | 98% | 99% |
| Mixed content | -1.03% | -1.07% | -1.16% | 103% | 100% |
| Camera-Captured content | 0.01% | 0.01% | 0.00% | 103% | 103% |
| Overall | -1.75% | -1.83% | -1.87% | 101% | 100% |

(c) under LDB configurations

Table 3-5. The performance of the proposed Method#2 with dual tree on, CTC QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.61% | -0.25% | -0.42% | 103% | 102% |
| TGM 720p | -0.54% | -0.10% | -0.28% | 101% | 102% |
| Animation | -0.05% | -0.02% | 0.02% | 101% | 100% |
| Mixed content | -0.13% | -0.33% | -0.16% | 103% | 101% |
| Camera-Captured content | 0.00% | -0.01% | -0.01% | 94% | 98% |
| Overall | -0.32% | -0.15% | -0.20% | 101% | 101% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.49% | -1.12% | -1.00% | 99% | 100% |
| TGM 720p | -0.44% | -1.29% | -2.05% | 98% | 101% |
| Animation | 0.06% | 0.00% | 0.03% | 101% | 101% |
| Mixed content | -0.05% | -2.31% | -2.03% | 102% | 101% |
| Camera-Captured content | 0.00% | -0.01% | 0.01% | 100% | 102% |
| Overall | -0.23% | -1.04% | -1.13% | 100% | 101% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.43% | -0.57% | -0.50% | 100% | 98% |
| TGM 720p | -0.52% | -0.15% | -0.56% | 102% | 100% |
| Animation |  |  |  |  |  |
| Mixed content | -0.43% | -0.74% | -0.52% | 102% | 105% |
| Camera-Captured content | 0.02% | -0.09% | -0.05% | 100% | 100% |
| Overall |  |  |  |  |  |

(c) under LDB configurations

Table 3-6. The performance of the proposed Method#2 with dual tree on under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -4.13% | -4.54% | -4.61% | 100% | 97% |
| TGM 720p | -4.15% | -2.97% | -2.92% | 101% | 91% |
| Animation | -0.76% | -0.65% | -0.80% | 102% | 103% |
| Mixed content | -2.55% | -2.27% | -2.41% | 103% | 103% |
| Camera-Captured content | -0.01% | -0.01% | 0.00% | 101% | 103% |
| Overall | -2.69% | -2.42% | -2.49% | 101% | 98% |

(a) under AI configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -3.97% | -3.80% | -3.47% | 98% | 96% |
| TGM 720p | -5.36% | -1.49% | -1.87% | 97% | 99% |
| Animation | -1.26% | -1.87% | -1.67% | 102% | 100% |
| Mixed content | -2.31% | -0.85% | -1.11% | 101% | 101% |
| Camera-Captured content | -0.01% | 0.00% | 0.00% | 101% | 100% |
| Overall | -3.01% | -1.83% | -1.86% | 100% | 99% |

(b) under RA configurations

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.41% | -2.59% | -2.48% | 101% | 101% |
| TGM 720p | -2.77% | -1.54% | -1.88% | 101% | 100% |
| Animation |  |  |  |  |  |
| Mixed content | -1.03% | -0.69% | -0.76% | 102% | 101% |
| Camera-Captured content |  |  |  |  |  |
| Overall |  |  |  |  |  |

(c) under LDB configurations

# Conclusions

It is proposed to adjust palette clustering step sizes in accordance with the escape coding method in CE2-1.1. The performance gains are desirable and encoding/decoding time variations are negligible. It is suggested to adopt the proposed algorithms to next version of VVC and VTM.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019.

- W. Zhu, J. Xu, L. Zhang, “CE2-1.1: QP dependent fixed-length binarization for escape coding”, JVET-Q0075, Dec. 2019.

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding,” JVET-P2022, Oct. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).