Proposed spec changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_act_qp_offsets_present_flag | u(1) |
| if (pps_act_qp_offsets_present_flag) { |  |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_minus1 | se(v) |
| pps_act_cr_qp_offset_minus3 | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_act_cbcr_qp_offset_minu1 | se(v) |
| } |  |
| …… |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

……

pps_act_qp_offsets_present_flag equal to 1 specifies that pps_act_y_qp_offset_plus5  and pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 are present in the PPS syntax structure. pps_act_qp_offsets_present_flag equal to 0 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, and pps_act_cr_qp_offset_minus3 are not present in the PPS RBSP syntax structure. When sps_act_enabled_flag is equal to 0, the value of pps_act_qp_offsets_present_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are used to determine the offsets that are applied to the quantization parameter values, when cu_act_enabled_flag is equal to 1. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1 pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are inferred to be equal to 0.

The variable ppsActQpOffsetY is set equal to pps_act_y_qp_offset_plus5 - 5.

The variable ppsActQpOffsetCb is set equal to pps_act_cb_qp_offset_minus1 + 1.

The variable ppsActQpOffsetCr is set equal to pps_act_cr_qp_offset_minus3 + 3.

The variable ppsActQpOffsetCbCr is set equal to pps_act_cbcr_qp_offset_minus1 + 1.

ppsActQpOffsetY, ppsActQpOffsetCb ppsActQpOffsetCr and ppsActQpOffsetCbCr shall be in the range of -12 to 12, inclusive.

……

### 8.7.3 Scaling process for transform coefficients

……

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Clip3(0, 63+QpBdOffset, Qp′Y + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetY:0))		(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Clip3(0, 63+QpBdOffset, Qp′CbCr + (cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCbCr:0 )) 		(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Clip3(0, 63+QpBdOffset, Qp′Cb + (cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCb:0 ))		(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Clip3(0, 63+QpBdOffset, Qp′Cr + (cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCr:0 ))		(1132)

……

### 8.7.4.6 Residual modification process for blocks using colour space conversion

……

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ]	(1191)

rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − rCr[ x ][ y ]	(1193)

rCr[ x ][ y ] = tmp + rCr[ x ][ y ]	(1194)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )	(1191)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ]  >>  1 )	(1193)

rCr[ x ][ y ]  +=  rCb[ x ][ y ]	(1194)

……