| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0510_r1 |
| --- | --- |

| Title: | ACT colour conversion for both lossless and lossy coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu1 Yi-Wen Chen1 Tsung-Chuan Ma1 Hong-Jheng Jhu1 Xianglin Wang1 Jie Zhao2 Hendry2 Seethal Paluri2 Seung Hwan Kim2 Weijia Zhu3 Jizheng Xu3 Li Zhang3 | Email: | xiaoyu.xiu@kwai.com jie.zhao@lge.com weijia.zhu@bytedance.com |
| Source: | Kwai Inc.1, LG Electronics2 and Bytedance Inc.3 |  |  |

_____________________________

# Abstract

Adaptive colour transform (ACT) in the VVC draft 7 is not reversible so that it does not support lossless coding. This contribution proposes to enable the ACT mode in lossless coding by using reversible YCgCo colour transform (also known as YCgCo-R). To achieve one unified design, it is also proposed to use the YCgCo-R transform for lossy coding. Finally, instead of using fixed offset values, it is proposed to signal ACT QP offsets in bitstream. The proposed method provides coding performance improvement for lossless coding while maintaining similar results for CTC QP.

For lossless coding, comparing to the VTM-7.1 lossless anchors, the proposed scheme provides bitrate differences of -2.35% for AI, -4.86% for RA, and -6.04% for LD for RGB test sequences, average encoding and decoding runtime impacts are 102% and 100%.

A side benefit of enabling the ACT lossless coding is that transform skip (TS) mode is enabled for coding chroma components for the ACT. Comparing to VTM-7.1 lossy anchors, using YCgCo-R transform and enabling chroma TS together provides {G/Y, B/Cb, R/Cr} BD-rate differences of {-0.16%, -0.08%, -0.10%} for AI, {-0.02%, 0.01%, -0.09%} for RA, and {-0.29%,0.01%, -0.31%} for LD for RGB test sequences. Impacts on encoding and decoding runtime are negligible.

Without chroma TS, using YCgCo-R transform alone has almost no impact on coding performance in lossy coding. Specifically, comparing to VTM-7.1 lossy anchors, the coding performance impacts on RGB test sequences is {-0.01%, 0.03%, 0.00%} for AI, {0.05%, 0.06%, -0.06%} for RA and {0.01%%, 0.18%, -0.10%} for LD. Impacts on encoding and decoding runtime are negligible.

# Introduction

Adaptive colour transform (ACT) was adopted into VVC working draft 7 for 444 content. Similar to the ACT in HEVC screen content coding (SCC) extension, it adaptively converts prediction residuals from one colour space to YCgCo space. Two colour spaces are adaptively selected by signaling one ACT flag for each coding unit (CU). When the flag is equal to one, it indicates the residuals of the CU are coded in the YCgCo space; otherwise (i.e., the flag is equal to 0), it indicates that the residuals of the CU is coded in the original colour space.  An illustration of the decoding process with ACT is shown in Figure 1.

Figure 1: Illustration of decoding process with the ACT

The core transforms (transform kernels) used for the colour space conversions are irreversible ACT transform matrices. Specifically, the following forward and inverse YCgCo colour transform matrices, as described as follows, are applied.

| C0'C1'C2'=2112-1-10-22C0C1C2/4YCoCg=12120-2-12-1RGBYCoCg=12120-2-12-1RGB C0C1C2=1101-1-11-11C0'C1'C2' | (1) |
| --- | --- |

Here C0, C1, C1 correspond to G, B, R, and C0’, C1’ and C2’ correspond to Y, Cg, Co. This conversion is known as YCgCo conversion. To compensate the dynamic range change of residuals signals before and after colour transform, the QP adjustments of (-5, -5, -3) are applied to the transform residuals for Y, Cg, Co components.

The above conversion is not fully reversible. Therefore, it cannot be used in applications that require lossless coding. In the current VTM-7.1, ACT is turned off for lossless coding. The drawback is that lossless coding applications cannot benefit from the gains of ACT. This proposal provides solutions that enable ACT in both lossy and lossless coding applications.

# Proposed method

## Reversible ACT transform for both lossless and lossy coding

In HEVC SCC extension, the ACT supports both lossless and lossy coding. Based on cu_transquant_bypass_flag, (indication of. lossless or lossy coding), ACT conversions are performed differently. It is equivalent to that in the case of lossy coding, YCgCo conversion (which is the one adopted to VVC) is used, and in the case of lossless coding, a reversible conversion (known as YCgCo-R conversion) is used.

However, according to the VCC draft 7, there is no cu_transquant_bypass_flag signaled in the bitstream to indicate whether lossy or lossless coding is applied. Based on such design, in this contribution, it is proposed to use the YCgCo-R transform for both lossy and lossless cases.  The YCgCo-R reversible colour transform is shown as below.

| Forward Conversion: GBR to YCgCo | Backward Conversion: YCgCo to GBR |
| --- | --- |
| Co = R-B; t = B + (Co>>1); Cg = G – t; Y = t + (Cg>>1); | t = Y – (Cg>>1) G = Cg + t B = t – (Co>>1) R = Co + B |

Since the YCgCo-R transform leads to different norm change compared to the existing YCgCo colour conversion in VVC, ACT QP offsets for Y, Cg, Co need to be adjusted to compensate the dynamic range change due to colour space conversion. In the VVC draft 7, ACT QP offsets (-5, -5, -3) are used for Y, Cg, Co components. When the YCgCo-R transform is applied, given that the bit-depth of Cg and Co is increased by 1-bit, the corresponding ACT QP offsets (-5, 1, 3) are used when lossy coding is applied.

Although in the VVC draft 7 transform skip (TS) mode is enabled for coding chroma residuals of one ACT-coded CU, the checking of chroma TS mode is not enabled in the current ACT implementation in VTM-7.1. Therefore, to enable ACT lossless coding, encoder-only modifications are made to appropriately enable the chroma TS mode for the ACT mode.

## Signalling of ACT QP offsets

For both lossless and lossy coding, different ACT QP offsets need to be applied. Therefore, it becomes necessary to signal different ACT QP offsets from encoder to decoder.  Besides lossless support, it is asserted that signalling ACT QP offsets can provide more flexibilities to encoder for adjusting the QPs that are applied to ACT CUs. For example, for different sequences or different QP ranges, varying ACT QP offsets can have better coding performance compared to fixed ACT QP offset values.

Table 1. below shows a simple scheme of signalling ACT QP offsets at PPS level. It should be noted that ACT QP offsets may also be signalled at other granularities (such as picture header, or slice level) to allow more flexibilities.

Table 1. Proposed signaling of ACT QP offsets in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_act_qp_offsets_present_flag | u(1) |
| if (pps_act_qp_offsets_present_flag) { |  |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_minus1 | se(v) |
| pps_act_cr_qp_offset_minus3 | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_act_cbcr_qp_offset_minus1 | se(v) |
| } |  |
| … |  |
| } |  |

pps_act_qp_offsets_present_flag equal to 1 specifies that pps_act_y_qp_offset_plus5  and pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 are present in the PPS syntax structure. pps_act_qp_offsets_present_flag equal to 0 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, and pps_act_cr_qp_offset_minus3 are not present in the PPS RBSP syntax structure. When sps_act_enabled_flag is equal to 0, the value of pps_act_qp_offsets_present_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are used to determine the offsets that are applied to the quantization parameter values, when cu_act_enabled_flag is equal to 1. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1 pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are inferred to be equal to 0, 0, 0 and 0.

The variable ppsActQpOffsetY is set equal to pps_act_y_qp_offset_plus5 - 5.

The variable ppsActQpOffsetCb is set equal to pps_act_cb_qp_offset_minus1 + 1.

The variable ppsActQpOffsetCr is set equal to pps_act_cr_qp_offset_minus3 + 3.

The variable ppsActQpOffsetCbCr is set equal to pps_act_cbcr_qp_offset_minus1 + 1.

ppsActQpOffsetY, ppsActQpOffsetCb ppsActQpOffsetCr and ppsActQpOffsetCbCr shall be in the range of -12 to 12, inclusive.

For the ACT QP offset of joint chroma residual coding (JCCR) mode, same as the existing ACT design in VVC draft 7, it is set equal to ACT QP offset of the chroma component whose residuals are coded. Specifically, when the JCCR mode (i.e. TuCResMode[ xTbY ][ yTbY ] ) is equal to 1 and 2, the Cb QP offset is used; when the JCCR mode is equal to 3, the Cr QP offset is used.

# Experimental Result

## Enabling ACT in Lossless coding

The proposed method is implemented on the top of VTM-7.1, and tested on RGB and YUV444 test sequences by following the current common test conditions as specified in CE3. Additionally, palette mode is enabled in both the anchor and the test. In the simulations, the dual-tree partition is disabled. An example command line is given as

EncoderApp.exe -c Console_RGB.cfg -c encoder_randomaccess_vtm.cfg -c lossless.cfg -c classF.cfg -c yuv444.cfg --BDPCM=0 --PLT=1 --ColorTransform=1 --DualITree=0 –q 0 –f 100

Comparing to VTM7.1 where the ACT is disabled, the lossless coding performance of the proposed method is summarized as below. More detailed results can be found in the attached spreadsheets.

Table 2. Result of enabling ACT in lossless coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.1 | VTM7.1 + ACT |  | VTM7.1 | VTM7.1 + ACT |  | VTM7.1 | VTM7.1 + ACT |  |
| RGB, TGM 1080p | 25.6 | 26.0 | -2.96% | 408.9 | 417.4 | -5.50% | 573.3 | 587.5 | -6.73% |
| RGB, TGM 720p | 10.6 | 10.9 | -2.47% | 102.8 | 106.8 | -5.05% | 166.4 | 175.5 | -6.82% |
| RGB, Animation | 2.3 | 2.3 | -2.62% | 3.8 | 4.0 | -4.28% | 3.8 | 4.0 | -5.03% |
| RGB, Mixed | 6.0 | 6.2 | -2.49% | 34.9 | 37.3 | -6.47% | 38.0 | 41.2 | -7.73% |
| RGB, Camera-Captured | 1.5 | 1.5 | -0.56% | 1.3 | 1.3 | -1.07% | 1.3 | 1.3 | -1.06% |
| YUV, TGM 1080p | 26.8 | 26.7 | 0.02% | 432.5 | 432.4 | 0.01% | 606.3 | 606.1 | 0.02% |
| YUV, TGM 720p | 13.4 | 13.4 | 0.01% | 133.6 | 133.5 | 0.01% | 220.3 | 220.2 | 0.02% |
| YUV, Animation | 6.0 | 6.0 | 0.01% | 13.6 | 13.6 | 0.00% | 13.1 | 13.1 | 0.02% |
| YUV, Mixed content | 7.9 | 7.9 | 0.01% | 50.7 | 50.7 | 0.02% | 55.9 | 55.9 | 0.01% |
| YUV, Camera-Captured | 1.8 | 1.8 | 0.00% | 2.0 | 2.0 | 0.00% | 2.0 | 2.0 | 0.00% |
| All (RGB) | 12.0 | 12.2 | -2.35% | 154.2 | 158.2 | -4.86% | 220.0 | 227.3 | -6.04% |
| All (YUV) | 12.9 | 12.9 | 0.01% | 153.8 | 153.8 | 0.01% | 219.8 | 219.8 | 0.02% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |

Result show that for RGB test sequences, there are rate savings of 2.35% for AI, 4.86% for RA and 6.04% for LD.

## Impact of proposed change in Lossy coding

Tests are also done at CTC QPs to check the impact of proposed change on lossy coding.  An example command line is given as follows.

EncoderApp.exe -c Console_RGB.cfg -c encoder_randomaccess_vtm.cfg -c classF.cfg -c yuv444.cfg --PLT=1 --ColorTransform=1 --DualITree=0 –q 22

Table 3. shows the impact of replacing YCgCo transform with YCgCo-R transform only. Results show that changing YCgCo to YCgCo-R has almost no RD performance impact.

A side benefit of lossless coding support is that chroma TS mode is now also implemented in the ACT functions (Note: TS chroma has already been adopted in VVC draft 7, but was not implemented in ACT functions in VTM-7.1). Table 4. shows the impact of replacing YCgCo transform with YCgCo-R transform and additionally adding chroma TS.

Table 3. Result of using reversible ACT in lossy coding

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT |
| RGB, TGM 1080p | 0.02% | 0.00% | 0.01% |  | 100% | 100% |
| RGB, TGM 720p | 0.02% | 0.06% | 0.03% |  | 101% | 99% |
| RGB, Animation | -0.01% | 0.05% | -0.03% |  | 100% | 100% |
| RGB, Mixed content | 0.03% | 0.01% | 0.03% |  | 100% | 101% |
| RGB, Camera-Captured content | -0.15% | 0.03% | -0.08% |  | 100% | 99% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% |  | 100% | 100% |
| YUV, TGM 720p | -0.01% | 0.00% | -0.02% |  | 100% | 99% |
| YUV, Animation | 0.00% | -0.01% | -0.01% |  | 100% | 101% |
| YUV, Mixed content | 0.00% | 0.04% | 0.01% |  | 100% | 100% |
| YUV, Camera-Captured content | 0.01% | 0.00% | 0.00% |  | 100% | 100% |
| Overall (RGB) | -0.01% | 0.03% | 0.00% |  | 100% | 100% |
| Overall (YUV) | 0.00% | 0.01% | 0.00% |  | 100% | 100% |
|  |  |  |  | Random access Main10 |  |  |
|  | Over VTM7.1 |  |  |  |  |  |
|  | Y | U |  | V | EncT | DecT |
| RGB, TGM 1080p | 0.06% | 0.02% |  | 0.01% | 100% | 100% |
| RGB, TGM 720p | 0.07% | 0.11% |  | -0.07% | 99% | 101% |
| RGB, Animation | -0.01% | 0.05% |  | -0.04% | 100% | 100% |
| RGB, Mixed content | -0.02% | 0.02% |  | -0.02% | 99% | 100% |
| RGB, Camera-Captured content | 0.13% | 0.06% |  | -0.27% | 100% | 99% |
| YUV, TGM 1080p | 0.01% | -0.02% |  | -0.01% | 100% | 100% |
| YUV, TGM 720p | 0.00% | -0.03% |  | 0.02% | 100% | 101% |
| YUV, Animation | -0.01% | -0.03% |  | -0.04% | 100% | 100% |
| YUV, Mixed content | 0.01% | 0.01% |  | -0.02% | 100% | 100% |
| YUV, Camera-Captured content | 0.03% | -0.01% |  | -0.06% | 100% | 102% |
| Overall (RGB) | 0.05% | 0.06% |  | -0.06% | 100% | 100% |
| Overall (YUV) | 0.00% | -0.02% |  | -0.02% | 100% | 101% |
|  |  |  |  | Low delay B Main10 |  |  |
|  | Over VTM7.1 |  |  |  |  |  |
|  | Y | U |  | V | EncT | DecT |
| RGB, TGM 1080p | 0.11% | 0.09% |  | 0.06% | 100% | 99% |
| RGB, TGM 720p | 0.14% | 0.34% |  | -0.13% | 100% | 100% |
| RGB, Animation | -0.04% | 0.10% |  | -0.08% | 100% | 100% |
| RGB, Mixed content | -0.07% | 0.05% |  | 0.11% | 99% | 100% |
| RGB, Camera-Captured content | -0.28% | 0.32% |  | -0.69% | 101% | 101% |
| YUV, TGM 1080p | -0.03% | -0.07% |  | -0.11% | 100% | 99% |
| YUV, TGM 720p | 0.02% | -0.13% |  | -0.33% | 101% | 101% |
| YUV, Animation | 0.00% | 0.07% |  | -0.01% | 100% | 99% |
| YUV, Mixed content | -0.11% | 0.15% |  | -0.13% | 100% | 100% |
| YUV, Camera-Captured content | 0.05% | 0.04% |  | 0.23% | 100% | 100% |
| Overall (RGB) | 0.01% | 0.18% |  | -0.10% | 100% | 100% |
| Overall (YUV) | -0.02% | 0.00% |  | -0.11% | 100% | 100% |

Table 4. Result of using reversible ACT and adding ACT TS Chroma into ACT functions

|  |  |  |  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V | EncT | DecT |
| RGB, TGM 1080p | -0.09% |  | 0.00% |  |  | 0.00% | 101% | 100% |
| RGB, TGM 720p | -0.18% |  | -0.06% |  |  | -0.15% | 101% | 99% |
| RGB, Animation | -0.23% |  | -0.41% |  |  | -0.28% | 102% | 99% |
| RGB, Mixed content | -0.15% |  | -0.13% |  |  | -0.12% | 100% | 101% |
| RGB, Camera-Captured content | -0.20% |  | -0.05% |  |  | -0.09% | 103% | 100% |
| YUV, TGM 1080p | 0.00% |  | 0.00% |  |  | 0.01% | 100% | 100% |
| YUV, TGM 720p | -0.02% |  | 0.01% |  |  | -0.01% | 100% | 100% |
| YUV, Animation | 0.00% |  | -0.02% |  |  | -0.02% | 100% | 100% |
| YUV, Mixed content | 0.00% |  | 0.04% |  |  | 0.01% | 100% | 100% |
| YUV, Camera-Captured content | 0.01% |  | 0.00% |  |  | 0.00% | 99% | 99% |
| Overall (RGB) | -0.16% |  | -0.08% |  |  | -0.10% | 101% | 100% |
| Overall (YUV) | 0.00% |  | 0.01% |  |  | 0.00% | 100% | 100% |
|  |  |  |  |  |  | Random access Main10 |  |  |
|  | Over VTM7.1 |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V | EncT | DecT |
| RGB, TGM 1080p | 0.01% |  | 0.02% |  |  | 0.04% | 102% | 101% |
| RGB, TGM 720p | -0.01% |  | 0.05% |  |  | -0.18% | 100% | 100% |
| RGB, Animation | -0.01% |  | -0.08% |  |  | -0.05% | 100% | 101% |
| RGB, Mixed content | -0.19% |  | -0.05% |  |  | -0.10% | 100% | 101% |
| RGB, Camera-Captured content | 0.16% |  | 0.05% |  |  | -0.19% | 101% | 100% |
| YUV, TGM 1080p | 0.01% |  | 0.03% |  |  | 0.01% | 100% | 100% |
| YUV, TGM 720p | 0.00% |  | -0.02% |  |  | 0.05% | 100% | 100% |
| YUV, Animation | -0.06% |  | -0.07% |  |  | -0.03% | 100% | 100% |
| YUV, Mixed content | 0.02% |  | 0.05% |  |  | 0.01% | 100% | 100% |
| YUV, Camera-Captured content | 0.05% |  | -0.01% |  |  | -0.07% | 100% | 101% |
| Overall (RGB) | -0.02% |  | 0.01% |  |  | -0.09% | 101% | 101% |
| Overall (YUV) | 0.00% |  | 0.00% |  |  | 0.00% | 100% | 100% |
|  |  |  |  |  |  | Low delay B Main10 |  |  |
|  | Over VTM7.1 |  |  |  |  |  |  |  |
|  | Y |  |  | U |  | V | EncT | DecT |
| RGB, TGM 1080p | -0.01% |  |  | 0.02% |  | -0.03% | 100% | 101% |
| RGB, TGM 720p | -0.18% |  |  | 0.12% |  | -0.45% | 100% | 101% |
| RGB, Animation | -0.42% |  |  | -0.20% |  | -0.24% | 100% | 101% |
| RGB, Mixed content | -0.75% |  |  | -0.27% |  | -0.24% | 99% | 101% |
| RGB, Camera-Captured content | -0.33% |  |  | 0.29% |  | -0.74% | 99% | 100% |
| YUV, TGM 1080p | 0.03% |  |  | 0.00% |  | -0.03% | 100% | 99% |
| YUV, TGM 720p | -0.11% |  |  | -0.17% |  | -0.08% | 101% | 102% |
| YUV, Animation | 0.04% |  |  | 0.01% |  | -0.01% | 101% | 101% |
| YUV, Mixed content | -0.05% |  |  | 0.03% |  | -0.27% | 100% | 100% |
| YUV, Camera-Captured content | 0.04% |  |  | -0.02% |  | 0.14% | 100% | 100% |
| Overall (RGB) | -0.29% |  |  | 0.01% |  | -0.31% | 100% | 101% |
| Overall (YUV) | -0.02% |  |  | -0.04% |  | -0.06% | 100% | 100% |

# Conclusion

This contribution proposed to enable ACT in lossless coding by using YCgCo-R transform for the ACT mode. This YCgCo-R transform is used for both lossy and lossless configuration. Additionally, ACT QP offsets are signalled in PPS. Experimental results show the proposed scheme has 2.35%, 4.86% and 6.04% bitrate savings for AI, RA and LD configurations on RGB test sequences; and little impact for the lossy configuration. It is proposed to adopt this proposal into the next VVC draft.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-O0010, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).