Proposed spec changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_act_qp_offsets_present_flag | u(1) |
| if (pps_act_qp_offsets_present_flag) { |  |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_minus1 | se(v) |
| pps_act_cr_qp_offset_minus3 | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_act_cbcr_qp_offset_minus1 | se(v) |
| } |  |
| …… |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

……

pps_act_qp_offsets_present_flag equal to 1 specifies that pps_act_y_qp_offset_plus5  and pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 are present in the PPS syntax structure. pps_act_qp_offsets_present_flag equal to 0 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, and pps_act_cr_qp_offset_minus3 are not present in the PPS RBSP syntax structure. When sps_act_enabled_flag is equal to 0, the value of pps_act_qp_offsets_present_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are used to determine the offsets that are applied to the quantization parameter values, when cu_act_enabled_flag is equal to 1. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1 pps_act_cr_qp_offset_minus3 and pps_act_cbcr_qp_offset_minus1 are inferred to be equal to 0.

The variable ppsActQpOffsetY is set equal to pps_act_y_qp_offset_plus5 - 5.

The variable ppsActQpOffsetCb is set equal to pps_act_cb_qp_offset_minus1 + 1.

The variable ppsActQpOffsetCr is set equal to pps_act_cr_qp_offset_minus3 + 3.

The variable ppsActQpOffsetCbCr is set equal to pps_act_cbcr_qp_offset_minus1 + 1.

ppsActQpOffsetY, ppsActQpOffsetCb ppsActQpOffsetCr and ppsActQpOffsetCbCr shall be in the range of -12 to 12, inclusive.

……

### 8.7.3 Scaling process for transform coefficients

……

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetY : 0 ) 		(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCbCr : 0 ) 		(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCb : 0 )		(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCr : 0 ) )		(1132)

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 )	(1133)

qP = Clip3( 0, 63 + QpBdOffset, qP )		(1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 )  ?  1 : 0	(1134)

bdShift = BitDepth + rectNonTsFlag +		(1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ?  5  :  0 )	(1136)

qP = Clip3( QpPrimeTsMin, 63 + QpBdOffset, qP )	(1136)

rectNonTsFlag = 0		(1137)

bdShift = 10			(1138)

……

### 8.7.4.6 Residual modification process for blocks using colour space conversion

……

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ]	(1191)

rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − rCr[ x ][ y ]	(1193)

rCr[ x ][ y ] = tmp + rCr[ x ][ y ]	(1194)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )	(1191)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ]  >>  1 )	(1193)

rCr[ x ][ y ]  +=  rCb[ x ][ y ]	(1194)

……