| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET- Q0511 |
| --- | --- |

| Title: | On ACT QP clipping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao1 Hendry1 Seung Hwan Kim1 Xiaoyu Xiu2 Yi-Wen Chen2 Tsung-Chuan Ma2 Hong-Jheng Jhu2 Xianglin Wang2 Weijia Zhu3 Jizheng Xu3 Li Zhang3 | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com xiaoyu.xiu@kwai.com weijia.zhu@bytedance.com |
| Source: | LG Electronics.1, Kwai2 and Bytedance Inc.3 |  |  |

_____________________________

# Abstract

In VVC draft 7, Adaptive Colour Transform (ACT) is used to reduce statistical redundancy among different colour components. Since ACT forward transform is not normalized, predefined ACT QP offsets are applied to compensate the dynamic range change between colour components. However, in the existing ACT design, the values of the modified QPs may go beyond the range of the allowed QP values.

To resolve such QP value overflow problem, this contribution proposes to clip the QP values that are applied to ACT coding units (CUs) to valid QP range. At low QPs, the proposed method reportedly provides {G, B, R} BD-rate differences of {-0.32%, -0.22%,-0.21%} in AI, {-0.1%,-0.03%,-0.02%} in RA and {-0.07%, -0.05%, -0.05%} in LDB, respectively, for RGB test sequences.

# Introduction

In VVC draft 7, when adaptive colour transform (ACT) is applied, fixed ACT QP offset (-5, -5, -3) is used for Y, Cb, Cr (a.k.a. Y, Cg, Co), and joint CbCr. ACT QP offset modifies the qP used in scaling process for transform coefficients. The related texts are as below (note the text below include fix of spec reported by ticket #758 [3] on the top of VVC draft 7 [1]).

8.7.3 Scaling process for transform coefficients …… The quantization parameter qP is derived as follows: - If cIdx is equal to 0, the following applies: qP = Qp′Y (1129) actQpOffset= -5 - Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies: qP = Qp′CbCr (1130) actQpOffset= -5 - Otherwise, if cIdx is equal to 1, the following applies: qP = Qp′Cb (1131) actQpOffset= -5 - Otherwise (cIdx is equal to 2), the following applies: qP = Qp′Cr (1132) actQpOffset= -3 The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows: - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies: qP = qP + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpOffset : 0 ) (1133) … - Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies: qP = Max( QpPrimeTsMin, qP ) + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpOffset : 0 ) (1136)

The modified QP listed in Eq. 1133 and Eq. 1136 may exceed the range of valid QP values, specifically in the following cases:

- At very low QPs, it can result in negative qP values. qP will be used in a modulus operation (e.g. levelScale[ qP%6 ]). The modulus operation ‘%’ is not defined for negative numbers in VVC. So negative qP will result in undefined behavior.

- In transform skip mode, the modified qP after Eq. 1136 now may become smaller than the QpPrimeTsMin which is the minimum allowed QP for transform skip mode.

- In the case that positive ACT QP offset values are used, the modified qP may also go beyond the maximum allowed QP range.

# Proposed solutions

To prevent the QP overflow problem in the existing ACT design, it is proposed to clip the modified qP. The corresponding text specification changes to the VVC draft 7 [1] with ticket#758 fix [3] as shown in the introduction section are described as below:

8.7.3 Scaling process for transform coefficients …… The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows: - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies: qP = qP + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpOffset : 0 ) (1133) qP = Clip3(0, 63+ QpBdOffset, qP + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpOffset : 0 ) ) (1133) rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 ) = = 1 ) ? 1 : 0 (1134) bdShift = BitDepth + rectNonTsFlag + (1135) ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag - Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies: qP = Max( QpPrimeTsMin, qP ) + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpoffset : 0 ) (1136) qP = Clip3( QpPrimeTsMin, 63+ QpBdOffset, qP + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? actQpOffset : 0 ) ) (1136) …

# Experimental result

The proposed method is implemented on top of VTM7.1. Tests are done using RGB and YUV444 test sequences. The proposed ACT QP clipping has no impact on CTC QPs but affects the coding performance at lower QP values.

Table 1-3 shows that BD-rate performance of the proposed method compared to the VTM-7.1 anchors at low QPs (i.e, 2, 7, 12 and 17) test conditions. It is asserted that the gain comes from clipping the minimum Qp to QpPrimeTsMin.

Table 1. AI coding performance of the proposed scheme compared to VTM7.1

|  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 |  |  |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| RGB, TGM 1080p | -0.61% | -0.41% |  |  | -0.41% | 100% | 100% |
| RGB, TGM 720p | -0.26% | -0.19% |  |  | -0.18% | 100% | 100% |
| RGB, Animation | 0.00% | 0.00% |  |  | 0.01% | 100% | 99% |
| RGB, Mixed content | -0.32% | -0.24% |  |  | -0.19% | 100% | 100% |
| RGB, Camera-Captured content | 0.00% | 0.00% |  |  | 0.00% | 101% | 100% |
| YUV, TGM 1080p | 0.00% | 0.00% |  |  | 0.00% | 101% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| YUV, Animation | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| YUV, Mixed content | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| Overall (RGB) | -0.32% | -0.22% |  |  | -0.21% | 100% | 100% |
| Overall (YUV) | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| Table 2. RA coding performance of the proposed scheme compared to VTM7.1 |  |  |  |  |  |  |  |
|  |  |  | Random access Main10 |  |  |  |  |
|  | Over VTM7.1 |  |  |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| RGB, TGM 1080p | -0.17% | -0.01% |  |  | -0.02% | 100% | 100% |
| RGB, TGM 720p | -0.05% | -0.09% |  |  | 0.00% | 101% | 100% |
| RGB, Animation | 0.00% | -0.05% |  |  | -0.07% | 100% | 99% |
| RGB, Mixed content | -0.17% | -0.01% |  |  | -0.04% | 100% | 100% |
| RGB, Camera-Captured content | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| YUV, TGM 1080p | 0.00% | 0.02% |  |  | -0.01% | 100% | 100% |
| YUV, TGM 720p | 0.00% | -0.02% |  |  | 0.00% | 100% | 100% |
| YUV, Animation | 0.00% | 0.00% |  |  | -0.01% | 100% | 100% |
| YUV, Mixed content | 0.01% | 0.01% |  |  | 0.01% | 100% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| Overall (RGB) | -0.10% | -0.03% |  |  | -0.02% | 100% | 100% |
| Overall (YUV) | 0.00% | 0.00% |  |  | 0.00% | 100% | 100% |
| Table 3. LDB coding performance of the proposed scheme compared to VTM7.1 |  |  |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.1 |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| RGB, TGM 1080p | -0.21% | -0.15% |  | -0.11% |  | 100% | 100% |
| RGB, TGM 720p | -0.04% | -0.07% |  | -0.09% |  | 99% | 99% |
| RGB, Animation | 0.05% | -0.03% |  | -0.04% |  | 100% | 100% |
| RGB, Mixed content | -0.03% | 0.07% |  | 0.06% |  | 100% | 100% |
| RGB, Camera-Captured content | 0.00% | 0.00% |  | 0.00% |  | 100% | 100% |
| YUV, TGM 1080p | -0.02% | -0.01% |  | 0.00% |  | 100% | 100% |
| YUV, TGM 720p | -0.03% | -0.03% |  | -0.01% |  | 101% | 101% |
| YUV, Animation | -0.01% | 0.01% |  | -0.01% |  | 100% | 100% |
| YUV, Mixed content | -0.01% | 0.04% |  | 0.01% |  | 101% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% |  | 0.00% |  | 101% | 100% |
| Overall (RGB) | -0.07% | -0.05% |  | -0.05% |  | 100% | 100% |
| Overall (YUV) | -0.01% | 0.00% |  | 0.00% |  | 100% | 100% |

# Conclusion

This contribution proposes to fix on qP modified with ACT QP offsets to prevent it exceeding the valid QP range. Simulation results show that the proposed change has BD-rate difference of -0.32%, -0.1% and -0.07% at low QP test condition for RGB sequences in AI, RA and LDB test conditions respectively.

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team (JVET), JVET-P2001, v14, Geneva, CH, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-O0010, Gothenburg, SE, July 2019.

- JVET bug tracker Ticket#758,  https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/758

- X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, J. Zhao, Hendry, S. Paluri, S.-H. Kim, W. Zhu, J. Xu, L. Zhang “ACT colour conversion for both lossless and lossy coding”, JVET-Q0510, Brussels, BE, Jan. 2020.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).