# Note: this text is on the top of VVC7 draf text JVET-Q0041-v2.docx, it also includes fix reported by ticket #758 regarding of ACT QP offset value (-5,-5,-3), where the spec fix is based on the JVET-P0517-WD.docx from the adopted JVET-P0517.

## Scaling, transformation and array construction process

### Derivation process for quantization parameters

…

### Scaling and transformation process

…

### Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y  + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? -5 : 0 )	(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? -5 : 0 )	(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? -5 : 0 )	(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? -3 : 0 )		(1132)

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP -- ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 )	(1133)

qP = Max(0, qP)                                                                                                                                             (1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 )  ?  1 : 0	(1134)

bdShift = BitDepth + rectNonTsFlag +		(1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) - ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ? 5:  0 )	(1136)

qP = Max( QpPrimeTsMin, qP )                                                                                                                   (1136)

rectNonTsFlag = 0		(1137)

bdShift = 10			(1138)

…

