| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_act_qp_offsets_present_flag | u(1) |
| if (pps_act_qp_offsets_present_flag) { |  |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_plus5 | se(v) |
| pps_act_cr_qp_offset_plus3 | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_act_cbcr_qp_offset_plus5 | se(v) |
| } |  |
| …… |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

……

pps_act_qp_offsets_present_flag equal to 1 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5, pps_act_cr_qp_offset_plus3 and/or pps_act_cbcr_qp_offset_plus5 are present in the PPS syntax structure. pps_act_qp_offsets_present_flag equal to 0 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5, pps_act_cr_qp_offset_plus3 and pps_act_cbcr_qp_offset_plus5 are not present in the PPS RBSP syntax structure. When sps_act_enabled_flag is equal to 0, the value of pps_act_qp_offsets_present_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5, pps_act_cr_qp_offset_plus3 and pps_act_cbcr_qp_offset_plus5 are used to determine the offsets that are applied to the quantization parameter values, when cu_act_enabled_flag is equal to 1. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5 pps_act_cr_qp_offset_plus3 and pps_act_cbcr_qp_offset_plus5 are inferred to be equal to 0.

The variable ppsActQpOffsetY is set equal to pps_act_y_qp_offset_plus5 - 5.

The variable ppsActQpOffsetCb is set equal to pps_act_cb_qp_offset_ plus5 - 5.

The variable ppsActQpOffsetCr is set equal to pps_act_cr_qp_offset_ plus3 - 3.

The variable ppsActQpOffsetCbCr is set equal to pps_act_cbcr_qp_offset_plus5 + 5.

ppsActQpOffsetY, ppsActQpOffsetCb ppsActQpOffsetCr and ppsActQpOffsetCbCr shall be in the range of -12 to 12, inclusive.

## Scaling, transformation and array construction process

### Derivation process for quantization parameters

…

### Scaling and transformation process

…

### Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetY : 0 )	(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCbCr : 0 )	(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCb : 0 )	(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr  + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? ppsActQpOffsetCr : 0 )	(1132)

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP +- ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 5 : 0 )	(1133)

qP = Clip3(0, 63+ QpBdOffset, qP )                                                                                                             (1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 )  ?  1 : 0	(1134)

bdShift = BitDepth + rectNonTsFlag +		(1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) + - ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ? 5  :  0 )	(1136)

qP = Clip3( QpPrimeTsMin, 63+ QpBdOffset, qP ) )                                                                             (1136)

rectNonTsFlag = 0		(1137)

bdShift = 10			(1138)

…

