| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0513 |
| --- | --- |

| Title: | AHG16: Clipping residual samples for JCCR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add one clipping at joint coding of chroma residuals (JCCR) module at decoder such that the reconstructed residuals of the JCCR do not exceed the dynamic range of 16-bit sign integers. Simulation results reportedly show that the proposed clipping operation has no coding performance impact under common test condition (CTC).

# Problem statement

Same to the HEVC, in the current VVC design, all the residuals that are maintained at intermediate decoding process are intended to be within the dynamic range of 16-bit signed integers. Figure 1 summarizes residual decoding workflow of one VVC decoder. As illustrated in Figure 1, in order to maintain the 16-bit dynamic range of residual information, multiple clipping operations are applied after de-quantization, inverse transform and inverse CRS. Meanwhile, according to VVC draft 7 [1], one bitstream conformance constraint is applied to ensure that the parsed residuals (i.e., the residuals fed into the de-quantization) are within the 16-bit dynamic range.

Figure 1 Flowchart of residual decoding process in the VVC

In Figure 1, ClipA and ClipB represent two different clipping functions, as defined as

| ClipA=Clip(-215, 215-1,M) ClipB=Clip(-2B, 2B-1,M) |  |
| --- | --- |

where B is internal coding bit-depth.

As depicted in Figure 1, because there is no clipping operation applied to the inverse JCCR, this could cause the decoded residuals of the JCCR go beyond the 16-bit dynamic range. Such overflow problem happens when the JCCR mode is 2 with signaled negative sign (i.e., Cr = -Cb) and the decoded Cb residual sample is equal to the minimum of 16-bit sign integer (i.e., -215). In such case, the calculated Cr residual sample is equal to 215 which is outside the 16-bit range [-215, 215-1].

# Proposal

To resolve the above residual overflow problem, it is proposed to add clipping operation (i.e., ClipA) to the decoded residuals of the JCCR at decoder, as illustrated in Figure 2.

Figure 2 Flowchart of decoding process with proposed clipping to JCCR residuals

It was noticed that ticket #744 was submitted to VTM bug tracker for reporting the same issue and merge request !1168, which is equivalent to the proposed method, was merged into VTM7.1 [2] as software-only fix. This contribution proposes to align the specification to the VTM software by adding the same clipping into the next release of VVC working draft.

# Simulation results

The proposed clipping was tested on top of VTM-7.0 [3] by following the common test conditions (CTCs) as specified in [4]. Simulation results show that the change has no BD-rate impacts.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 96% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 106% | 110% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 96% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 106% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 107% |
| Class E | 0.00% | 0.00% | 0.00% | 94% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 94% | 92% |
| Class F | 0.00% | 0.00% | 0.00% | 97% | 100% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).