| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0513_r2 |
| --- | --- |

| Title: | AHG16: Clipping residual samples for ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add one clipping to the input residual samples to inverse adaptive color transform (IACT) module at decoder such that the reconstructed residuals along the VVC decoding path do not exceed the dynamic range of 16-bit sign integers. Simulation results reportedly show that the proposed clipping operation has minor performance impact under common test condition (CTC).

# Problem statement

Same to the HEVC, in the current VVC design, all the residuals that are maintained at intermediate decoding process are intended to be within the dynamic range of 16-bit signed integers. Figure 1 summarizes residual decoding workflow of one VVC decoder. As illustrated in Figure 1, in order to maintain the 16-bit dynamic range of residual information, multiple clipping operations are applied after de-quantization, and inverse CRS. Meanwhile, according to VVC draft 7 [1], one bitstream conformance constraint is applied to ensure that the parsed residuals (i.e., the residuals fed into the de-quantization) are within the 16-bit dynamic range.

Figure 1 Flowchart of residual decoding process in the VVC

In Figure 1, ClipA and ClipB represent two different clipping functions, as defined as

| ClipA=Clip(-215, 215-1,M) ClipB=Clip(-2B, 2B-1,M) |  |
| --- | --- |

where B is internal coding bit-depth.

As depicted in Figure 1, because there is no clipping operation applied to the inverse JCCR, this could cause the decoded residuals of the JCCR go beyond the 16-bit dynamic range. Such case happens when the JCCR mode is 2 with signaled negative sign (i.e., Cr = -Cb) and the decoded Cb residual sample is equal to the minimum of 16-bit sign integer (i.e., -215). In such case, the calculated Cr residual sample is equal to 215 which is outside the 16-bit range [-215, 215-1]. Given that the reconstructed residuals of the JCCR are the input of IACT, the bit-depth increase results in higher implementation cost of the IACT, which is undesirable for hardware implementations.

# Proposal

To resolve the above residual overflow problem while ensuring 16-bit implementation of IACT, it is proposed to add clipping operation  to the input residuals of the IACT at decoder. In the first method, it is proposed to apply the clipping ClipB to the input residuals of the IACT as illustrated in Figure 2.

Figure 2 Flowchart of decoding process with proposed clipping to input IACT residuals (Method One)

In the second method, it is proposed to apply the clipping ClipA to the input residuals of the IACT, as illustrated in Figure 3.

Figure 3 Flowchart of decoding process with proposed clipping to input IACT residuals (Method Two)

# Proposed specification changes

- Text changes for Method One

#### 8.7.4.6 Residual modification process for blocks using colour space conversion

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

rY[ x ][ y ]= Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, rY[ x ][ y ])	(xxxx)

rCb[ x ][ y ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, rCb[ x ][ y ])	(xxxx)

rCr[ x ][ y ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, rCr[ x ][ y ])	(xxxx)

tmp = rY[ x ][ y ] − rCb[ x ][ y ]	                                                                                                               (1191)

rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]	                                                                                                        (1192)

rCb[ x ][ y ] = tmp − rCr[ x ][ y ]                                        	(1193)

rCr[ x ][ y ] = tmp + rCr[ x ][ y ]                                         	(1194)

- Text changes for Method Two

rY[ x ][ y ]= Clip3( CoeffMin, CoeffMax, rY[ x ][ y ])	(xxxx)

rCb[ x ][ y ] = Clip3( CoeffMin, CoeffMax, rCb[ x ][ y ])	(xxxx)

rCr[ x ][ y ] = Clip3( CoeffMin, CoeffMax, rCr[ x ][ y ])	(xxxx)

tmp = rY[ x ][ y ] − rCb[ x ][ y ]	                                                                                                               (1191)

rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]	                                                                                                        (1192)

rCb[ x ][ y ] = tmp − rCr[ x ][ y ]                                        	(1193)

rCr[ x ][ y ] = tmp + rCr[ x ][ y ]                                         	(1194)

# Simulation results

The proposed clipping was tested on top of VTM-7.1 [2] by following the common test conditions (CTCs) as specified in [4].

- Simulation results of Method One

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.01% | -0.01% | 0.00% | 95% | 97% |
| RGB, TGM 720p | 0.00% | -0.01% | 0.00% | 98% | 96% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 94% | 96% |
| RGB, Mixed content | 0.00% | 0.00% | 0.00% | 98% | 95% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 96% | 96% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 98% | 97% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 99% | 99% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 99% | 103% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 97% | 96% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall (RGB) | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 99% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | 0.00% | 0.02% | 0.01% | 101% | 99% |
| RGB, TGM 720p | 0.00% | 0.01% | 0.01% | 100% | 100% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 102% | 101% |
| RGB, Mixed content | -0.02% | -0.01% | -0.01% | 97% | 96% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 97% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 102% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 101% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 99% | 99% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 100% | 97% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Overall (RGB) | 0.00% | 0.01% | 0.00% | 100% | 99% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 100% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.04% | -0.04% | -0.05% | 100% | 101% |
| RGB, TGM 720p | -0.02% | -0.03% | -0.04% | 97% | 95% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 106% | 105% |
| RGB, Mixed content | -0.04% | 0.01% | 0.06% | 98% | 98% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 97% | 103% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 97% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 100% | 99% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 95% | 98% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall (RGB) | -0.03% | -0.02% | -0.01% | 98% | 99% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 99% | 99% |

- Simulation results of Method Two

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | 0.00% | 0.00% | 0.00% | 95% | 95% |
| RGB, TGM 720p | 0.00% | 0.00% | 0.00% | 98% | 97% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 92% | 99% |
| RGB, Mixed content | 0.00% | 0.00% | 0.00% | 103% | 103% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 96% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 101% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 100% | 102% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 98% | 98% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall (RGB) | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 98% |
| RGB, TGM 720p | 0.00% | 0.00% | 0.00% | 99% | 98% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 103% | 104% |
| RGB, Mixed content | 0.00% | 0.00% | 0.00% | 99% | 97% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 99% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 100% | 98% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall (RGB) | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 100% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| RGB, TGM 720p | 0.00% | 0.00% | 0.00% | 97% | 97% |
| RGB, Animation | 0.00% | 0.00% | 0.00% | 106% | 106% |
| RGB, Mixed content | 0.00% | 0.00% | 0.00% | 98% | 99% |
| RGB, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 100% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 101% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 98% | 98% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 98% | 100% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 96% | 98% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Overall (RGB) | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 98% | 99% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode”, JVET-P2022, Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).