| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0516 |
| --- | --- |

| Title: | Constrained MTS signaling based on the position of last significant coefficient |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC draft 7, multiple transform selection (MTS) can be enabled when there is non-zero transform coefficient in luma transform block (TB). This is done by checking whether the luma coded-block-flag (CBF) is equal to one. In this contribution, it is proposed to replace such CBF based condition with one based on the position of last significant coefficient. More specifically, the signaling of the MTS index (i.e., mts_idx) is conditioned on whether the position of last significant coefficient is less than one. Since the same condition is also used to determine the signaling of LFNST index lfnst_idx, the proposed change incurs no extra complexity. Compared to VTM-7.0 anchors, the proposed change reportedly leads to BD-rate differences of -0.10% and -0.03% for AI and RA configurations, respectively, with no impacts on encoding and decoding runtimes.

# Introduction

In VVC draft 7 [1], multiple transform selection (MTS) and low-frequency non-separable transform (LFNST) are two transform coding tools that are used to achieve better energy compaction of transform coefficients.

The MTS consists of 5 primary transform candidates selected from DCT-2, DST-7 and DCT-8 core transforms. The selection of optimal transforms is specified by the syntax mts_idx. The LFNST is applied to intra coded coding units (CUs) where the top-left coefficients obtained from primary transforms are further transformed using non-separable transform kernels. In the existing LFNST design, the syntax lfnst_idx is signaled to choose the non-separable transform from 2 transform candidate sets and the set is selected implicitly based on CU size and intra prediction mode.

Additionally, in VVC draft 7 [1], the position information of transform coefficients is used to condition the signaling of mts_idx and lfnst_idx. Specifically, for the MTS, mts_idx is not signaled and inferred to be 0 when either of the following two conditions is applied:

- There is no significant coefficient in luma transform block (TB). i.e., tu_luma_cbf is equal to zero.

- The last significant coefficient of the luma TB is located inside the MTS zero-out region.

For the LFNST, lfnst_idx is not signaled and inferred to be 0 when either one of two condition is satisfied:

- All the positions of the last significant coefficients within the CU are smaller than 1 (i.e., DC only).

- There is one component of the CU whose last significant coefficient’s position is within the LFNST zero-out region.

# Proposal

In this contribution, the existing MTS signaling condition on luma CBF is replaced with checking the position of last significant luma coefficient. Additionally, to not introduce any new checking operation at parsing stage, the same threshold of last significant coefficient used for the signaling of LFNST index (which is equal to one, i.e., DC only) is reused in the proposal. When the position of last significant coefficient for luma is less than 1, the MTS index is not signaled and inferred to be 0 (i.e., DCT-2 is applied); otherwise, the MTS index is signaled. Specifically, with the proposed change, the MTS signaling is skipped when one of the below conditions is applied:

- There is no significant coefficient in luma transform block (TB). i.e., tu_luma_cbf is equal to zero.

- The position of the last significant coefficient for the luma TB is less than 1 (i.e., DC only).

- The last significant coefficient of the luma TB is located inside the MTS zero-out region.

Except for the modified signaling condition, the other existing MTS design, e.g., syntax elements, binarization, context modeling, and the encoder logics that are applied in the contribution are kept the same as that used in the VVC draft 7 [1] and the VTM-7.0 [2].

As will be shown later, the proposed change not only makes the signaling conditions of the MTS and LFNST indices more consistent but also brings some coding gains due to reduced signaling overhead of MTS index.

# Simulation results

The proposed change was implemented on top of VTM-7.0 [1] and tested following the test conditions (CTCs) as specified in [3].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.13% | 0.15% | 97% | 96% |
| Class A2 | -0.09% | 0.07% | -0.02% | 99% | 98% |
| Class B | -0.08% | 0.08% | -0.02% | 102% | 100% |
| Class C | -0.14% | 0.06% | 0.01% | 97% | 97% |
| Class E | -0.13% | -0.03% | 0.18% | 102% | 102% |
| Overall | -0.10% | 0.02% | 0.05% | 100% | 99% |
| Class D | -0.09% | 0.11% | -0.09% | 101% | 100% |
| Class F | -0.11% | 0.01% | 0.15% | 97% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | -0.09% | 0.10% | 99% | 99% |
| Class B | -0.04% | 0.11% | 0.09% | 100% | 100% |
| Class C | -0.05% | 0.06% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.03% | 0.06% | 100% | 100% |
| Class D | -0.05% | -0.04% | 0.23% | 100% | 100% |
| Class F | -0.05% | 0.11% | 0.14% | 100% | 100% |

# Proposed spec change

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsLastSigCoeffPosMetThresholdFlag = 0 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| …… |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsLastSigCoeffPosMetThresholdFlag == 1 tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| …… |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( lastSubBlock > 0 ¦¦ lastScanPos > Tpos) |  |
| MtsLastSigCoeffPosMetThresholdFlag = 1 |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| …… |  |
| } |  |

# Conclusions

- A new MTS index signaling condition is proposed in this contribution, which has no extra complexity but provides some BD-rate saving. It is recommended the proposed method be adopted into VVC.

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).