| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0517 |
| --- | --- |

| Title: | On RPR down-sampling filters for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC draft 7, for affine mode, 6-tap and 4-tap motion interpolation filters are always applied to interpolation process regardless of scaling factor between one current picture and its reference picture. Because those motion interpolation filters are not strong low-pass filters, affine prediction samples may present severe aliasing artifacts when the reference picture is of higher resolution than the current picture (i.e. down-sampling). In this contribution, down-sampling filters are proposed to replace the existing motion interpolation filters for affine mode. Similar to the derivation of luma motion interpolation filters for affine mode, the luma down-sampling filters for the affine mode are 6-tap filters and are directly derived from the existing 8-tap luma down-sampling filters used for non-affine mode. Meanwhile, for chorma components, the same 4-tap chroma down-sampling filters are used.

For common test condition (CTC) sequences, simulation results reportedly show that average luma BD-rate differences of PSNR1 are -0.19% and -0.14% for 2X and 1.5X scaling ratios, respectively. The corresponding average luma BD-rate differences of PSNR2 are -0.16% and -0.12% for 2X and 1.5X, respectively.

For affine sequences, simulation results reportedly show that average luma BD-rate differences of PSNR1 are -1.88% and -1.06 %% for 2X and 1.5X scaling ratios, respectively. The corresponding average luma BD-rate differences of PSNR2 are -1.57% and -0.84% for 2X and 1.5X, respectively.

# Introduction

In VVC draft 7, reference picture resampling (RPR) is supported, which allows fast spatial resolution switching with one bitstream. At the 16th JVET meeting, 8-tap/4-tap Lanczos filters and cosine windowed sinc filter were adopted to replace the existing motion interpolation filters for the down-sampling of luma/chroma reference samples for 1.5X and 2X scaling ratios, respectively. Table 1 to Table 4 illustrate the down-sampling filter coefficients that are applied in the VVC draft 7.

Table 1 Luma down-sampling filters for 1.5X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 | P4 | P5 | P6 | P7 |
| 0 | −1 | −5 | 17 | 42 | 17 | −5 | −1 | 0 |
| 1 | 0 | −5 | 15 | 41 | 19 | −5 | −1 | 0 |
| 2 | 0 | −5 | 13 | 40 | 21 | −4 | −1 | 0 |
| 3 | 0 | −5 | 11 | 39 | 24 | −4 | −2 | 1 |
| 4 | 0 | −5 | 9 | 38 | 26 | −3 | −2 | 1 |
| 5 | 0 | −5 | 7 | 38 | 28 | −2 | −3 | 1 |
| 6 | 1 | −5 | 5 | 36 | 30 | −1 | −3 | 1 |
| 7 | 1 | −4 | 3 | 35 | 32 | 0 | −4 | 1 |
| 8 | 1 | −4 | 2 | 33 | 33 | 2 | −4 | 1 |
| 9 | 1 | −4 | 0 | 32 | 35 | 3 | −4 | 1 |
| 10 | 1 | −3 | −1 | 30 | 36 | 5 | −5 | 1 |
| 11 | 1 | −3 | −2 | 28 | 38 | 7 | −5 | 0 |
| 12 | 1 | −2 | −3 | 26 | 38 | 9 | −5 | 0 |
| 13 | 1 | −2 | −4 | 24 | 39 | 11 | −5 | 0 |
| 14 | 0 | −1 | −4 | 21 | 40 | 13 | −5 | 0 |
| 15 | 0 | −1 | −5 | 19 | 41 | 15 | −5 | 0 |

Table 2 Luma down-sampling filters for 2X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 | P4 | P5 | P6 | P7 |
| 0 | −4 | 2 | 20 | 28 | 20 | 2 | −4 | 0 |
| 1 | −4 | 0 | 19 | 29 | 21 | 5 | −4 | −2 |
| 2 | −4 | −1 | 18 | 29 | 22 | 6 | −4 | −2 |
| 3 | −4 | −1 | 16 | 29 | 23 | 7 | −4 | −2 |
| 4 | −4 | −1 | 16 | 28 | 24 | 7 | −4 | −2 |
| 5 | −4 | −1 | 14 | 28 | 25 | 8 | −4 | −2 |
| 6 | −3 | −3 | 14 | 27 | 26 | 9 | −3 | −3 |
| 7 | −3 | −1 | 12 | 28 | 25 | 10 | −4 | −3 |
| 8 | −3 | −3 | 11 | 27 | 27 | 11 | −3 | −3 |
| 9 | −3 | −4 | 10 | 25 | 28 | 12 | −1 | −3 |
| 10 | −3 | −3 | 9 | 26 | 27 | 14 | −3 | −3 |
| 11 | −2 | −4 | 8 | 25 | 28 | 14 | −1 | −4 |
| 12 | −2 | −4 | 7 | 24 | 28 | 16 | −1 | −4 |
| 13 | −2 | −4 | 7 | 23 | 29 | 16 | −1 | −4 |
| 14 | −2 | −4 | 6 | 22 | 29 | 18 | −1 | −4 |
| 15 | −2 | −4 | 5 | 21 | 29 | 19 | 0 | −4 |

Table 3 Chroma down-sampling filters for 1.5X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 |
| 0 | 12 | 40 | 12 | 0 |
| 1 | 11 | 40 | 13 | 0 |
| 2 | 10 | 40 | 15 | −1 |
| 3 | 9 | 40 | 16 | −1 |
| 4 | 8 | 40 | 17 | −1 |
| 5 | 8 | 39 | 18 | −1 |
| 6 | 7 | 39 | 19 | −1 |
| 7 | 6 | 38 | 21 | −1 |
| 8 | 5 | 38 | 22 | −1 |
| 9 | 4 | 38 | 23 | −1 |
| 10 | 4 | 37 | 24 | −1 |
| 11 | 3 | 36 | 25 | 0 |
| 12 | 3 | 35 | 26 | 0 |
| 13 | 2 | 34 | 28 | 0 |
| 14 | 2 | 33 | 29 | 0 |
| 15 | 1 | 33 | 30 | 0 |
| 16 | 1 | 31 | 31 | 1 |
| 17 | 0 | 30 | 33 | 1 |
| 18 | 0 | 29 | 33 | 2 |
| 19 | 0 | 28 | 34 | 2 |
| 20 | 0 | 26 | 35 | 3 |
| 21 | 0 | 25 | 36 | 3 |
| 22 | −1 | 24 | 37 | 4 |
| 23 | −1 | 23 | 38 | 4 |
| 24 | −1 | 22 | 38 | 5 |
| 25 | −1 | 21 | 38 | 6 |
| 26 | −1 | 19 | 39 | 7 |
| 27 | −1 | 18 | 39 | 8 |
| 28 | −1 | 17 | 40 | 8 |
| 29 | −1 | 16 | 40 | 9 |
| 30 | −1 | 15 | 40 | 10 |
| 31 | 0 | 13 | 40 | 11 |

Table 4 Chroma down-sampling filters for 2X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 |
| 0 | 17 | 30 | 17 | 0 |
| 1 | 17 | 30 | 18 | −1 |
| 2 | 16 | 30 | 18 | 0 |
| 3 | 16 | 30 | 18 | 0 |
| 4 | 15 | 30 | 18 | 1 |
| 5 | 14 | 30 | 18 | 2 |
| 6 | 13 | 29 | 19 | 3 |
| 7 | 13 | 29 | 19 | 3 |
| 8 | 12 | 29 | 20 | 3 |
| 9 | 11 | 28 | 21 | 4 |
| 10 | 10 | 28 | 22 | 4 |
| 11 | 10 | 27 | 22 | 5 |
| 12 | 9 | 27 | 23 | 5 |
| 13 | 9 | 26 | 24 | 5 |
| 14 | 8 | 26 | 24 | 6 |
| 15 | 7 | 26 | 25 | 6 |
| 16 | 7 | 25 | 25 | 7 |
| 17 | 6 | 25 | 26 | 7 |
| 18 | 6 | 24 | 26 | 8 |
| 19 | 5 | 24 | 26 | 9 |
| 20 | 5 | 23 | 27 | 9 |
| 21 | 5 | 22 | 27 | 10 |
| 22 | 4 | 22 | 28 | 10 |
| 23 | 4 | 21 | 28 | 11 |
| 24 | 3 | 20 | 29 | 12 |
| 25 | 3 | 19 | 29 | 13 |
| 26 | 3 | 19 | 29 | 13 |
| 27 | 2 | 18 | 30 | 14 |
| 28 | 1 | 18 | 30 | 15 |
| 29 | 0 | 18 | 30 | 16 |
| 30 | 0 | 18 | 30 | 16 |
| 31 | −1 | 18 | 30 | 17 |

In the existing RPR design, the above down-sampling filters are only applied to the motion compensation of non-affine mode. For affine mode, the existing 6-tap/4-tap motion interpolation filters are still applied to the interpolation of luma/chroma prediction samples when the resolution of the reference picture is larger than that of the current picture. Because the motion interpolation filters do not show strong low-pass characteristics, compared to non-affine mode, affine prediction samples may present serious aliasing due to the well-known Nyquist-Shannon sampling theorem.

# Proposal

To improve the RPR coding efficiency of the affine mode, additional down-sampling filters are proposed to replace the existing 6-tap/4-tap motion interpolation filters for the affine mode when the reference picture is in higher resolution than the current picture. The down-sampling filters have the same length as the existing motion interpolation filters for affine mode, i.e., 6-tap for luma and 4-tap for chroma. Specifically, for luma components, the 6-tap luma down-sampling filters are derived from the current 8-tap down-sampling filters in Table 1 (for scaling ratio 1.5X) and Table 2 (for scaling ratio 2X) by adding two left-most/right-most filter coefficients into one single filter coefficient. It is noted that in the VVC draft 7 the same method is also applied for deriving the 6-tap affine motion interpolation filters from 8-tap default motion interpolation filters for regular inter blocks when the RPR is not applied. Table 5 and Table 6 illustrate the corresponding luma down-sampling filter coefficients that are used for the affine mode for 1.5X and 2X scaling ratios.

Table 5 Luma down-sampling filters for affine mode for 1.5X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 | P4 | P5 |
| 0 | −4 | 17 | 42 | 17 | −5 | −1 |
| 1 | −5 | 15 | 41 | 19 | −5 | −1 |
| 2 | −5 | 13 | 40 | 21 | −4 | −1 |
| 3 | −5 | 11 | 39 | 24 | −4 | −1 |
| 4 | −5 | 9 | 38 | 26 | −3 | −1 |
| 5 | −5 | 7 | 38 | 28 | −2 | −2 |
| 6 | −4 | 5 | 36 | 30 | −1 | −2 |
| 7 | −3 | 3 | 35 | 32 | 0 | −3 |
| 8 | −3 | 2 | 33 | 33 | 2 | −3 |
| 9 | −3 | 0 | 32 | 35 | 3 | −3 |
| 10 | −2 | −1 | 30 | 36 | 5 | −4 |
| 11 | −2 | −2 | 28 | 38 | 7 | −5 |
| 12 | −1 | −3 | 26 | 38 | 9 | −5 |
| 13 | −1 | −4 | 24 | 39 | 11 | −5 |
| 14 | −1 | −4 | 21 | 40 | 13 | −5 |
| 15 | −1 | −5 | 19 | 41 | 15 | −5 |

Table 6 Luma down-sampling filters for affine mode for 2X scaling ratio

| Fractional sample | interpolation filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P3 | P4 | P5 |
| 0 | -2 | 20 | 28 | 20 | 2 | −4 |
| 1 | -4 | 19 | 29 | 21 | 5 | −6 |
| 2 | −5 | 18 | 29 | 22 | 6 | −6 |
| 3 | −5 | 16 | 29 | 23 | 7 | −6 |
| 4 | −5 | 16 | 28 | 24 | 7 | −6 |
| 5 | −5 | 14 | 28 | 25 | 8 | −6 |
| 6 | −6 | 14 | 27 | 26 | 9 | −6 |
| 7 | −4 | 12 | 28 | 25 | 10 | −7 |
| 8 | −6 | 11 | 27 | 27 | 11 | −6 |
| 9 | −7 | 10 | 25 | 28 | 12 | −4 |
| 10 | −6 | 9 | 26 | 27 | 14 | −6 |
| 11 | −6 | 8 | 25 | 28 | 14 | −5 |
| 12 | −6 | 7 | 24 | 28 | 16 | −5 |
| 13 | −6 | 7 | 23 | 29 | 16 | −5 |
| 14 | −6 | 6 | 22 | 29 | 18 | −5 |
| 15 | −6 | 5 | 21 | 29 | 19 | -4 |

For the down-sampling of chroma components, the same 4-tap chroma down-sampling filters in Table 3 (for 1.5X scaling ratio) and Table 4 (for 2X scaling ratio) are used.

# Simulation results

The proposed affine down-sampling filters were implemented on top of VTM-7.0 [4] and tested by following the RPR common test conditions as specified in [5]. Both PSNR1 (i.e., codec PSNR) and PSNR2 (i.e., end-to-end PSNR between original pictures and up-sampled pictures) are measured for the BD-rate calculations. Additionally, simulation results for affine sequences are also provided.

- Simulation results on CTC sequences

Table 7 Simulation results of PSNR1 metric for 2X scaling ratio

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.18% | -0.34% | 102% | 103% |
| Class C | -0.38% | -0.12% | -0.21% | 96% | 99% |
| Class E | -0.05% | 0.01% | -0.51% | 96% | 93% |
| Overall | -0.19% | -0.11% | -0.34% | 98% | 99% |
| Class D | -0.41% | 0.06% | -0.54% | 96% | 95% |
| Class F | -0.21% | 0.01% | 0.09% | 96% | 98% |

Table 8 Simulation results of PSNR2 metric for 2X scaling ratio

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.17% | -0.34% | 102% | 103% |
| Class C | -0.27% | -0.18% | -0.32% | 96% | 99% |
| Class E | -0.07% | -0.15% | -0.54% | 96% | 93% |
| Overall | -0.16% | -0.17% | -0.38% | 98% | 99% |
| Class D | -0.28% | 0.18% | -0.43% | 96% | 95% |
| Class F | -0.15% | -0.08% | 0.18% | 96% | 98% |

Table 9 Simulation results of PSNR1 metric for 1.5X scaling ratio

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.10% | -0.30% | 97% | 99% |
| Class C | -0.33% | -0.02% | -0.14% | 97% | 95% |
| Class E | 0.06% | 0.03% | 0.01% | 99% | 105% |
| Overall | -0.14% | -0.04% | -0.17% | 97% | 99% |
| Class D | -0.64% | -0.09% | -0.03% | 95% | 95% |
| Class F | -0.09% | -0.04% | 0.01% | 99% | 97% |

Table 10 Simulation results of PSNR2 metric for 1.5X scaling ratio

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.10% | -0.33% | 97% | 99% |
| Class C | -0.31% | -0.07% | -0.16% | 97% | 95% |
| Class E | 0.12% | -0.04% | -0.08% | 99% | 105% |
| Overall | -0.12% | -0.07% | -0.21% | 97% | 99% |
| Class D | -0.55% | -0.12% | -0.14% | 95% | 95% |
| Class F | -0.05% | -0.22% | -0.02% | 99% | 97% |

- Simulation results on affine sequences

Table 11 Simulation results of PSNR1 metric for 2X scaling ratio for affine sequences

|  |  | Low delay B Main10 |  |
| --- | --- | --- | --- |
|  |  | Over VTM-7.0 |  |
|  | Y | U | V |
| BlueSkyPart | -0.40% | -0.05% | -0.29% |
| JetParts | -1.49% | -2.39% | -0.76% |
| ShieldsPart | -1.26% | -1.03% | -1.54% |
| SpincalendarPart | -4.82% | -2.92% | -5.25% |
| StationKtaPart | -1.47% | -0.65% | -1.35% |
| TratorPart | -1.86% | -0.99% | -0.70% |
| Overall | -1.88% | -1.34% | -1.65% |

Table 12 Simulation results of PSNR2 metric for 2X scaling ratio for affine sequences

|  |  | Low delay B Main10 |  |
| --- | --- | --- | --- |
|  |  | Over VTM-7.0 |  |
|  | Y | U | V |
| BlueSkyPart | -0.34% | -0.12% | -0.27% |
| JetParts | -2.44% | -2.49% | -1.09% |
| ShieldsPart | -0.93% | -0.74% | -1.16% |
| SpincalendarPart | -2.80% | -3.43% | -4.35% |
| StationKtaPart | -1.37% | -0.62% | -1.22% |
| TratorPart | -1.55% | -1.08% | -0.90% |
| Overall | -1.57% | -1.41% | -1.50% |

Table 13 Simulation results of PSNR1 metric for 1.5X scaling ratio for affine sequences

|  |  | Low delay B Main10 |  |
| --- | --- | --- | --- |
|  |  | Over VTM-7.0 |  |
|  | Y | U | V |
| BlueSkyPart | 0.21% | 0.17% | 0.33% |
| JetParts | -2.15% | -0.78% | -2.24% |
| ShieldsPart | -0.55% | -0.15% | -0.61% |
| SpincalendarPart | -3.79% | -1.61% | -2.80% |
| StationKtaPart | 0.18% | 0.21% | -0.95% |
| TratorPart | -0.30% | 0.30% | -0.20% |
| Overall | -1.06% | -0.31% | -1.08% |

Table 14 Simulation results of PSNR1 metric for 2X scaling ratio for affine sequences

|  |  | Low delay B Main10 |  |
| --- | --- | --- | --- |
|  |  | Over VTM-7.0 |  |
|  | Y | U | V |
| BlueSkyPart | 0.19% | 0.30% | 0.28% |
| JetParts | -2.14% | -0.79% | -2.01% |
| ShieldsPart | -0.41% | -0.36% | -0.60% |
| SpincalendarPart | -2.67% | -2.29% | -3.14% |
| StationKtaPart | 0.27% | 0.26% | -0.79% |
| TratorPart | -0.26% | 0.33% | -0.14% |
| Overall | -0.84% | -0.42% | -1.06% |

# Proposed spec changes

##### 8.5.6.3.2 Luma sample interpolation filtering process

……

The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, and scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 8-1.

- Otherwise, if MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, and scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 8-2.

- Otherwise, iIf MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 29 depending on hpelIfIdx.

The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, and scalingRatio[ 0 ] is greater than 28672, the luma interpolation filter coefficients fLV[ p ] are specified in Table 8-1.

- Otherwise, if MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, and scalingRatio[ 0 ] is greater than 20480, the luma interpolation filter coefficients fLV[ p ] are specified inTable8-2.

- Otherwise, iIf MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 29 depending on hpelIfIdx.

……

Table 8-1 Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | −1 | −4 | 17 | 42 | 17 | −5 | −1 | 0 |
| 1 | 0 | −5 | 15 | 41 | 19 | −5 | −1 | 0 |
| 2 | 0 | −5 | 13 | 40 | 21 | −4 | −1 | 0 |
| 3 | 0 | −5 | 11 | 39 | 24 | −4 | −1 | 0 |
| 4 | 0 | −5 | 9 | 38 | 26 | −3 | −1 | 0 |
| 5 | 0 | −5 | 7 | 38 | 28 | −2 | −2 | 0 |
| 6 | 0 | −4 | 5 | 36 | 30 | −1 | −2 | 0 |
| 7 | 0 | −3 | 3 | 35 | 32 | 0 | −3 | 0 |
| 8 | 0 | −3 | 2 | 33 | 33 | 2 | −3 | 0 |
| 9 | 0 | −3 | 0 | 32 | 35 | 3 | −3 | 0 |
| 10 | 0 | −2 | −1 | 30 | 36 | 5 | −4 | 0 |
| 11 | 0 | −2 | −2 | 28 | 38 | 7 | −5 | 0 |
| 12 | 0 | −1 | −3 | 26 | 38 | 9 | −5 | 0 |
| 13 | 0 | −1 | −4 | 24 | 39 | 11 | −5 | 0 |
| 14 | 0 | −1 | −4 | 21 | 40 | 13 | −5 | 0 |
| 15 | 0 | −1 | −5 | 19 | 41 | 15 | −5 | 0 |

Table 8-2 Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | 0 | -2 | 20 | 28 | 20 | 2 | −4 | 0 |
| 1 | 0 | -4 | 19 | 29 | 21 | 5 | −6 | 0 |
| 2 | 0 | −5 | 18 | 29 | 22 | 6 | −6 | 0 |
| 3 | 0 | −5 | 16 | 29 | 23 | 7 | −6 | 0 |
| 4 | 0 | −5 | 16 | 28 | 24 | 7 | −6 | 0 |
| 5 | 0 | −5 | 14 | 28 | 25 | 8 | −6 | 0 |
| 6 | 0 | −6 | 14 | 27 | 26 | 9 | −6 | 0 |
| 7 | 0 | −4 | 12 | 28 | 25 | 10 | −7 | 0 |
| 8 | 0 | −6 | 11 | 27 | 27 | 11 | −6 | 0 |
| 9 | 0 | −7 | 10 | 25 | 28 | 12 | −4 | 0 |
| 10 | 0 | −6 | 9 | 26 | 27 | 14 | −6 | 0 |
| 11 | 0 | −6 | 8 | 25 | 28 | 14 | −5 | 0 |
| 12 | 0 | −6 | 7 | 24 | 28 | 16 | −5 | 0 |
| 13 | 0 | −6 | 7 | 23 | 29 | 16 | −5 | 0 |
| 14 | 0 | −6 | 6 | 22 | 29 | 18 | −5 | 0 |
| 15 | 0 | −6 | 5 | 21 | 29 | 19 | -4 | 0 |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

J. Samuelsson, S. Deshpande, A. Segall, “CE1-1: RPR downsampling filter”, JVET-P0088, Oct. 2019, Geneva, Switzerland.

J. Luo, Y. Ye, M. Sarwer, “CE1-related: Reference picture resampling filters”, JVET-P0353, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

D. Luo, V. Seregin, W. Wan, “Description of Core Experiment 1 (CE1): Reference picture resampling filters”, JVET-O2021, Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).