| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0518 |
| --- | --- |

| Title: | Mismatch between text specification and reference software on RPR chroma down-sampling for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

At the 16th JVET meeting, 8-tap/4-tap Lanczos down-sampling filters and cosine windowed sinc down-sampling filter were adopted for the motion interpolation process when the reference picture is of higher resolution than the current picture. It is reported that there is one mismatch between VVC draft 7 and VTM-7.0 reference software on the down-sampling filters that are applied to the chroma components. Specification fixes are provided in the contribution to align the specification with the software. The proposed change has no coding performance impacts.

# Introduction

In VVC draft 7 [1], reference picture resampling (RPR) is supported, which allows fast spatial resolution switching with one bitstream. At the 16th JVET meeting, 8-tap/4-tap Lanczos filters [2] and cosine windowed sinc filters [3] were adopted to replace the existing motion interpolation filters for the down-sampling of luma/chroma reference samples for 1.5X and 2X scaling ratios, respectively. In this contribution, one mismatch was identified between VVC draft 7 [1] and VTM-7.0 reference software [4].

In VTM-7.0, the additional down-sampling filters are only applied to the motion compensation of non-affine mode. For affine mode, the existing 6-tap/4-tap motion interpolation filters are still applied to the interpolation of luma/chroma prediction samples.

On the other hand, in VVC draft 7, the additional down-sampling filters are always disabled for the motion interpolation of luma component for affine mode, as indicated by the following conditions (as highlighted in bold italic fonts) applied in 8.6.5.3.2:

The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 29 depending on hpelIfIdx.

The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 29 depending on hpelIfIdx.

But for chroma components, the affine mode constraint is not considered when selecting motion interpolation filters such that the additional down-sampling filters are allowed to be applied, as indicted in 8.6.5.3.4:

The horizontal chroma interpolation filter coefficients fCH[ p ] for each 1/32 fractional sample position p equal to xFracC are derived as follows:

- If scalingRatio[ 0 ] is greater than 28672, chroma interpolation filter coefficients fCH[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, chroma interpolation filter coefficients fCH[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCH[ p ] are specified in Table 33.

The vertical chroma interpolation filter coefficients fCV[ p ] for each 1/32 fractional sample position p equal to yFracC are derived as follows:

- If scalingRatio[ 1 ] is greater than 28672, chroma interpolation filter coefficients fCV[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, chroma interpolation filter coefficients fCV[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCV[ p ] are specified in Table 33.

# Proposal

As per the decision in the last meeting, this contribution proposes to align the specification with VTM-7.0 reference software by disabling the additional down-sampling filters for the chroma motion interpolation of affine mode.

##### 8.5.6.3.4 Chroma sample interpolation process

……

The horizontal chroma interpolation filter coefficients fCH[ p ] for each 1/32 fractional sample position p equal to xFracC are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, chroma interpolation filter coefficients fCH[ p ] are specified in Table 33.

- Otherwise, iIf scalingRatio[ 0 ] is greater than 28672, chroma interpolation filter coefficients fCH[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, chroma interpolation filter coefficients fCH[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCH[ p ] are specified in Table 33.

The vertical chroma interpolation filter coefficients fCV[ p ] for each 1/32 fractional sample position p equal to yFracC are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, chroma interpolation filter coefficients fCH[ p ] are specified in Table 33.

- Otherwise, iIf scalingRatio[ 1 ] is greater than 28672, chroma interpolation filter coefficients fCV[ p ] are specified in Table 35.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, chroma interpolation filter coefficients fCV[ p ] are specified in Table 60.

- Otherwise, chroma interpolation filter coefficients fCV[ p ] are specified in Table 33.

……

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

J. Samuelsson, S. Deshpande, A. Segall, “CE1-1: RPR downsampling filter”, JVET-P0088, Oct. 2019, Geneva, Switzerland.

J. Luo, Y. Ye, M. Sarwer, “CE1-related: Reference picture resampling filters”, JVET-P0353, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).