| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0520-v1 |
| --- | --- |

| Title: | AHG9: Cleanups on signaling for CC-ALF, BDPCM, ACT and Palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Li Zhang, Kai Zhang, Weijia Zhu Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | 1-858-999-7093 wangyang.cs@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC and the suggested specification for cross-component adaptive loop filter (CC-ALF), certain coding tools, including CC-ALF, chroma BDPCM, ACT, and Palette modes are only allowed under certain conditions. More specifically, CC-ALF should be enabled for non-4:0:0 colour format, chroma BDPCM, ACT and Palette mode are only enabled for 4:4:4 colour format without separate plane coding. However, the corresponding syntax elements and semantics are not well defined to reflect these restrictions. In this contribution, two aspects are proposed to cleanup signalling for CC-ALF, and a third aspect is proposed to cleanup signalling for BDPCM, ACT, and Palette.

Aspect #1: Syntax elements related to CC-ALF are not signalled if ChromaArrayType is equal to 0.

Aspect #2: ALF and CC-ALF are controlled by two separate flags in SPS.

Aspect #3: BDPCM, ACT, and Palette mode for chroma are signalled under the condition of “ChromaArrayType is equal to 3”

# Introduction

With Cross Component Adaptive Loop Filtering (CC-ALF) as the CE5 common base [1], a reconstructed chroma sample is further refined by the result of an adaptive linear filtering process applied on corresponding luma samples. The signalling of CC-ALF related syntax elements in picture header are defined as follows:

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

There are two problems on signalling for CC-ALF.

Problem #1: Syntax elements related to CC-ALF are signalled in a picture header without considering the colour format. However, CC-ALF is not applicable for 4:0:0 colour format.

Problem #2: ALF and CC-ALF are controlled by a single flag in SPS. However, ALF and CC-ALF are two different coding tools which can be processed independently. It is unreasonable to control them as a whole.

While for chroma BDPCM, ACT, and Palette modes, the SPS controlling flags are signalled under the condition check of chroma_format_idc being equal to 3. It means that even separate plane coding is used for 4:4:4 colour format, those flags still need to be signalled even the picture is treated as monochrome pictures. The related syntax elements are defined as follows:

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … |  |
| if( chroma_format_idc = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposed methods

Two aspects are proposed to address the two problems of CC-ALF mentioned above.

Aspect #1: Syntax elements related to CC-ALF are not signalled if the colour format is 4:0:0.

Aspect #2: ALF and CC-ALF are controlled by two separate flags in SPS.

In addition, Aspect #3 is proposed to replace the condition check of “chroma_format_idc = = 3” by “ChromaArrayType == 3” for the signalling of chroma BDPCM, ACT, and Palette modes.

## Proposed changes for CC-ALF

The suggested specification changes on top of JVET-Q0058_CE5-Anchor_JVET-P2001-vE [1] is described as follows: deleted text is marked with red strikethrough, and newly added text are highlighted in green and grey for the first method and second aspect, respectively.

### Aspect #1

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| if(ChromaArrayType != 0 ) { |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |

### Aspect #2

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_ccalf_enabled_flag |  |
| } |  |
| sps_transform_skip_enabled_flag | u(1) |
| … |  |

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| if(sps_ccalf_enabled_flag != 0 ) { |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |

#### 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled.

sps_ccalf_enabled_flag equal to 0 specifies that the cross component adaptive loop filter is disabled. sps_ccalf_enabled_flag equal to 1 specifies that the cross component adaptive loop filter is enabled.

#### 7.4.4.2 General constraint information semantics

no_sao_constraint_flag equal to 1 specifies that sps_sao_enabled_flag shall be equal to 0. no_sao_constraint_flag equal to 0 does not impose such a constraint.

no_alf_constraint_flag equal to 1 specifies that sps_alf_enabled_flag shall be equal to 0. no_alf_constraint_flag equal to 0 does not impose such a constraint.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint.

## Proposed changes for chroma BDPCM, ACT and Palette modes

The suggested specification changes on top of JVET-P2001-vE [3] is described as follows: deleted text is marked with red strikethrough, and newly added text is highlighted in green.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idcChromaArrayType = = 3 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … |  |
| if(chroma_format_idcChromaArrayType = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Conclusion

Several aspects are proposed to cleanup signalling for CC-ALF, chroma BDPCM, ACT and Palette modes. It is recommended to adopt the proposed cleanups into the next version of VVC.

# Reference

- K. Misra, F. Bossen, A. Segall, P. Cowan, N. Hu, et al., “CE5 Common Base: Cross Component Adaptive Loop Filter,” JVET-Q0058, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 17th Meeting: Brussels, BE, 7–17 January 2020.

- H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, N. Hu, et al., “Non-CE5: Suggested text changes for ALF padding process,” JVET-P1038, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).