| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0522-v1 |
| --- | --- |

| Title: | Non-CE4: Early termination of DMVR and study of its pixel coverage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Han Huang, Wei-Jung Chien and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {chunchic, hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

- Abstract

This contribution proposes to early-terminate DMVR at CU level when the respective lengths of the given bi-directional motion vectors (MVs) in a coding unit are both small than a given threshold value. Experimental results were conducted on top of VTM-7.0. The report shows that when the proposed method is applied to DMVR with SAD-based early termination disabled additionally, the average BD-rate change observed is less ±0.01%, with 2.56% (when threshold value is 1) to 7.27% (when threshold value is 8) reduction of DMVR pixel coverage on average and less than ±1% of decoding-time fluctuation. In addition, it is also reported that there is no synergy when the sub-CU SAD-based early termination and the proposed one are both applied to DMVR. The average pixel coverage of DMVR for each configuration is summarized as follows:

- 39.32% (SAD-based method enabled);

- 39.27% (SAD-based method disabled);

- 36.76% (SAD-based method disabled + MV-based method with TH=1);

- 32.05% (SAD-based method disabled + MV-based method with TH=8);

- 32.05% (SAD-based method enabled + MV-based method with TH=8).

Introduction

In VVC working draft v7 (JVET-P2001), the decoding process of DMVR in a coding unit can be early-terminated if the sum of absolute difference (SAD) between the initial L0 and L1 prediction signals produced prior to motion vector refinement is smaller than a given threshold value during the motion compensation stage. This SAD-based early termination method was proven to be effectual in the early-stage of the development of DMVR to reduce the decoding complexity on average case. However, the decoding process has been polished and optimized for many meeting cycle. Therefore, this proposal begins with studying the decoding complexity, in terms of pixel coverage of DMVR and overall decoding time, followed by non-pixel-based method as a replacement or an addition to the current SAD-based method.

Proposed method

An CU-level early-termination method based on the length of bi-directional motion vectors (or MV-based method in short) is proposed. With it, the MV refinement process of DMVR is bypassed completely when the respective lengths of the given bi-directional MVs in a CU are both small than a given threshold value. For example, when the threshold value is 1, it means all the x and y components of MVL0 and MVL1 in a CU must be exactly zero (that is, “MVXL0 | MVYL0 | MVXL1 | MVYL1” being equal to 0). This proposal tested various threshold values (e.g. from 1 to 8) to optimize the configuration of the threshold value to reduce the use of DMVR while keeping the same coding performance as VTM-7.0 does. For example, when the threshold value is N (where N > 1), it means all the x and y components of MVL0 and MVL1 in a CU must not exceed N (that is, “abs(MVXL0) | abs(MVYL0) | abs(MVXL1) | abs(MVYL1)” being smaller than N). As motion vector field is reconstructed prior to the motion compensation stage, the decoder could make the decision solely at CU level to bypass hardware/software modules that are required to perform decoder-side MV refinement (e.g. loading reference samples to perform bi-linear interpolation and generating search-area samples to compute initial SAD value).

Experimental results

The experiments for the proposed methods were conducted on top of the VTM-7.0 software with the common test conditions (JVET-N1010) to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-7.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

The respective performance results are summarized in the table below. Threshold values tested start from the minimal value 1 and stop at 8 as we have started observed slight coding performance drops (e.g. 0.01%).

(VTM-7.0) SAD-based method enabled.

(Test A) SAD-based method disabled.

(Test B1,…,8) SAD-based method disabled + MV-based method with TH=1,…,8.

(Test C1) SAD-based method enabled + MV-based method with TH=1.

(Test C8) SAD-based method enabled + MV-based method with TH=8.

- Pixel coverage of DMVR.

- VTM-7.0 Anchor: SAD-based method enabled.

|  | VTM-7.0 Anchor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | - | - | - | - | - | 35.10% |
| Class A2 | - | - | - | - | - | 46.41% |
| Class B | - | - | - | - | - | 38.51% |
| Class C | - | - | - | - | - | 38.17% |
| Overall | - | - | - | - | - | 39.32% |
| Class D | - | - | - | - | - | 43.62% |
| Class F | - | - | - | - | - | 43.39% |

- Test A: SAD-based method disabled.

|  | Test A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.00% | -0.01% | -0.02% | 102% | 99% | 34.86% |
| Class A2 | -0.01% | 0.01% | 0.00% | 99% | 99% | 46.62% |
| Class B | 0.01% | -0.02% | 0.00% | 100% | 101% | 38.35% |
| Class C | 0.01% | -0.01% | 0.01% | 98% | 96% | 38.20% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 99% | 39.27% |
| Class D | 0.00% | 0.04% | 0.00% | 102% | 102% | 43.50% |
| Class F | 0.01% | -0.01% | -0.01% | 99% | 102% | 40.61% |

- Test B1: SAD-based method disabled + MV-based method with TH=1.

|  | Test B1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | -0.01% | 0.00% | 103% | 102% | 32.10% |
| Class A2 | -0.03% | -0.05% | 0.00% | 99% | 103% | 45.15% |
| Class B | -0.01% | 0.00% | 0.01% | 100% | 103% | 36.08% |
| Class C | -0.01% | 0.04% | 0.06% | 99% | 97% | 34.82% |
| Overall | -0.01% | 0.00% | 0.02% | 100% | 101% | 36.76% |
| Class D | 0.01% | 0.04% | 0.09% | 100% | 100% | 39.47% |
| Class F | -0.02% | -0.06% | -0.03% | 99% | 104% | 13.45% |

- Test B2: SAD-based method disabled + MV-based method with TH=2.

|  | Test B2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.01% | 0.01% | 103% | 101% | 31.98% |
| Class A2 | -0.03% | -0.05% | 0.01% | 99% | 103% | 44.78% |
| Class B | -0.01% | 0.00% | 0.01% | 100% | 103% | 35.50% |
| Class C | 0.01% | 0.04% | 0.04% | 99% | 98% | 32.61% |
| Overall | -0.01% | 0.00% | 0.02% | 100% | 101% | 35.88% |
| Class D | 0.00% | 0.06% | 0.06% | 99% | 99% | 36.76% |
| Class F | -0.02% | -0.01% | 0.01% | 98% | 103% | 12.59% |

- Test B3: SAD-based method disabled + MV-based method with TH=3.

|  | Test B3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.01% | 0.00% | 104% | 99% | 31.93% |
| Class A2 | -0.03% | -0.03% | 0.02% | 100% | 101% | 44.46% |
| Class B | 0.00% | 0.03% | 0.03% | 100% | 100% | 34.69% |
| Class C | 0.00% | 0.02% | 0.04% | 99% | 96% | 30.47% |
| Overall | 0.00% | 0.01% | 0.03% | 101% | 99% | 34.97% |
| Class D | 0.00% | 0.02% | 0.08% | 101% | 102% | 34.49% |
| Class F | -0.01% | 0.02% | 0.01% | 98% | 102% | 11.73% |

- Test B4: SAD-based method disabled + MV-based method with TH=4.

|  | Test B1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.02% | 0.03% | 103% | 101% | 31.91% |
| Class A2 | -0.02% | -0.04% | 0.00% | 100% | 103% | 43.98% |
| Class B | 0.00% | 0.01% | 0.03% | 100% | 103% | 33.97% |
| Class C | 0.01% | 0.04% | 0.05% | 100% | 98% | 27.95% |
| Overall | 0.00% | 0.01% | 0.03% | 101% | 101% | 33.95% |
| Class D | 0.03% | 0.04% | 0.07% | 103% | 104% | 31.33% |
| Class F | -0.01% | -0.02% | 0.00% | 95% | 101% | 11.15% |

- Test B5: SAD-based method disabled + MV-based method with TH=5.

|  | Test B1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.02% | 0.02% | 103% | 99% | 31.71% |
| Class A2 | -0.02% | -0.05% | 0.02% | 100% | 101% | 43.75% |
| Class B | -0.01% | 0.00% | 0.04% | 100% | 100% | 33.59% |
| Class C | 0.02% | 0.04% | 0.09% | 99% | 96% | 26.58% |
| Overall | 0.00% | 0.00% | 0.04% | 100% | 99% | 33.37% |
| Class D | 0.03% | 0.04% | 0.06% | 102% | 102% | 30.09% |
| Class F | -0.01% | 0.03% | 0.01% | 96% | 100% | 10.39% |

- Test B6: SAD-based method disabled + MV-based method with TH=6.

|  | Test B6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.02% | 0.02% | 103% | 101% | 31.66% |
| Class A2 | -0.01% | -0.08% | 0.02% | 101% | 101% | 43.57% |
| Class B | 0.00% | 0.01% | 0.04% | 100% | 99% | 33.37% |
| Class C | 0.01% | 0.05% | 0.09% | 99% | 98% | 25.92% |
| Overall | 0.00% | 0.00% | 0.04% | 101% | 99% | 33.08% |
| Class D | 0.02% | 0.03% | 0.05% | 101% | 101% | 29.25% |
| Class F | -0.01% | 0.01% | 0.00% | 94% | 99% | 10.27% |

- Test B7: SAD-based method disabled + MV-based method with TH=7.

|  | Test B7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | -0.01% | -0.01% | 103% | 99% | 31.65% |
| Class A2 | -0.02% | -0.01% | 0.01% | 101% | 101% | 43.27% |
| Class B | 0.00% | 0.02% | 0.05% | 100% | 100% | 33.02% |
| Class C | 0.01% | 0.06% | 0.07% | 100% | 97% | 25.16% |
| Overall | 0.00% | 0.02% | 0.03% | 101% | 99% | 32.70% |
| Class D | 0.04% | 0.02% | 0.01% | 101% | 101% | 28.45% |
| Class F | 0.00% | 0.02% | 0.03% | 94% | 99% | 10.05% |

- Test B8: SAD-based method disabled + MV-based method with TH=8.

|  | Test B8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.01% | -0.01% | 103% | 101% | 31.49% |
| Class A2 | -0.02% | 0.00% | 0.02% | 102% | 101% | 42.75% |
| Class B | 0.01% | 0.00% | 0.05% | 101% | 100% | 32.43% |
| Class C | 0.02% | 0.09% | 0.07% | 100% | 99% | 23.97% |
| Overall | 0.01% | 0.03% | 0.04% | 101% | 100% | 32.05% |
| Class D | 0.04% | 0.03% | 0.01% | 100% | 99% | 26.73% |
| Class F | -0.01% | 0.00% | 0.02% | 92% | 96% | 9.82% |

- Test C1: SAD-based method enabled + MV-based method with TH=1.

|  | Test C1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | 0.01% | 0.03% | -0.02% | 102% | 97% | 32.28% |
| Class A2 | -0.01% | -0.01% | 0.03% | 99% | 99% | 45.02% |
| Class B | 0.00% | -0.01% | -0.01% | 100% | 99% | 36.17% |
| Class C | 0.00% | 0.03% | 0.07% | 99% | 100% | 34.65% |
| Overall | 0.00% | 0.01% | 0.02% | 100% | 99% | 36.76% |
| Class D | 0.00% | 0.05% | 0.02% | 100% | 101% | 39.13% |
| Class F | 0.00% | -0.02% | -0.01% | 99% | 102% | 13.36% |

- Test C8: SAD-based method enabled + MV-based method with TH=8.

|  | Test C8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Coverage |
| Class A1 | -0.01% | 0.04% | -0.02% | 103% | 98% | 31.62% |
| Class A2 | 0.01% | 0.04% | 0.00% | 101% | 101% | 42.55% |
| Class B | 0.01% | 0.03% | 0.04% | 101% | 100% | 32.59% |
| Class C | 0.01% | 0.07% | 0.06% | 100% | 98% | 23.81% |
| Overall | 0.00% | 0.04% | 0.02% | 101% | 99% | 32.05% |
| Class D | 0.06% | 0.12% | 0.13% | 100% | 98% | 26.47% |
| Class F | 0.01% | -0.01% | 0.02% | 93% | 96% | 9.65% |

Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).