| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0524-v2 |
| --- | --- |

| Title: | Non-CE4: DC balancing in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Maryam Homayouni | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Decoder-side motion vector refinement (DMVR) refines the accuracy of motion vectors of merge mode by minimizing the sum of absolute difference (SAD) between the prediction blocks in reference picture lists L0 and L1. The refined MV might not be accurate in the case that two prediction blocks are not in the same level of brightness, for example in the case of fading scenes or lightening condition change in different frames. It is proposed to balance the DC component of two prediction blocks for SAD calculation. For this purpose, DC value of prediction blocks of L0 and L1 is calculated, and then delta DC value is subtracted from sample differences during SAD calculation. This modification stabilizes the functionality of DMVR in different situations. In the case of RA, the average BD-rate gain is reported to be 0.10% (0.17% gain for class A1 sequences). Additional non-CTC results shows that in the case of illumination change, DMVR not only provide gain, but also shows 0.07% loss (0.25% loss for Class A1), while the proposed method stabilize RD performance of DMVR and retains its original gain.

# Introduction/Problem statement

In the latest VVC draft, DMVR can be applied for the CUs which are coded in a specific situation, for example with following modes and features:

- CU level merge mode with bi-prediction MV

- One reference picture is in the past and another reference picture is in the future with respect to the current picture

- The distances (i.e. POC difference) from both reference pictures to the current picture are same

- CU has more than 128 luma samples

- Both CU height and CU width are larger than or equal to 8 luma samples

- BCW (Bi-prediction with CU weights) weight index indicates equal weight

- WP is not enabled for the current block

In DVMR, prediction block is partitioned to 16x16 subblocks, and MV refinement is performed for each 16x16 subblocks, separately. MV refinement (i.e. MVoffset) obeys the following rule, where MV0 and MV1 are the initial merge motion vectors, and MV0' and MV1' are the refined motion vectors.

MV0'=MV0+MVoffset

MV1'=MV1-MVoffset

MVoffset search is performed in two steps, including integer offset search, and fractional offset calculation. In integer offset search, 25 search points (i.e. 5 in horizontal and 5 in vertical directions) are searched. Search points in each direction are {-2, -1, 0, 1, +2} in full-pel Luma precision. For each search point, prediction blocks (i.e. Pred0' and Pred1') are generated using the refined MV (i.e. MV0' and MV1') and SAD is calculated (only even rows are used).

SAD(MVoffset)=j=0,j+=2H-1i=0W-1absPred0'i,j-Pred1'i,j

If the SAD of the initial MV pair (i.e. MVoffset=(0,0)) is smaller than a threshold, the integer sample stage of DMVR is terminated. Otherwise SADs of the 25 search points (i.e. the initial point and remaining 24 points) are calculated, and the point with the smallest SAD is selected as the output of integer sample offset search stage. The initial MV pair is prioritized by multiplying its SAD value to a factor of 0.75. The integer sample search is followed by fractional sample refinement, that calculates the fractional MVoffset based on the SAD values of the search point around the minimum integer point.

During integer sample search, as the MVoffset is an integer number in full-pel precision, most of the prediction samples which are involved in SAD calculation are shared. So, initial prediction blocks are generated with blocks size of (Width+2*DMVR_SR)*(Height+2*DMVR_SR) in each reference frame (DMVR_SR=2). During integer offset search, samples of prediction blocks (of size width*height) for each search point are directly extracted from this initial prediction blocks. To reduce computational complexity and memory access, these initial prediction blocks are generated using bi-linear interpolation filter.

It is noted that the DC level difference between two prediction blocks (Pred0 and Pred1) may cause DMVR not to be able to fine the optimum match of two prediction blocks. An example of one-dimensional signal is presented in XXX to demonstrate the problem. As shown in this simple example, DMVR process, which minimizes SAD between Pred0’ and Pred1’, but SAD is mainly affected bby the DC difference of two prediction block. As a results DMVR moves Pred0 to right (and Pred1 to left) to minimize SAD of these two prediction blocks. In this case, the part of the signal that has texture will be shifted away from each other, which results in and prediction and more residual.

| (a) Prediction blocks with original MV | (b) Prediction blocks with modified MV |
| --- | --- |

Figure 1 Value of weight for different pictures

# Proposal

The main idea of the proposal is to balance DC difference (i.e., DeltaDC) of two prediction blocks in the SAD calculation. So, the SAD value for each search point is calculated using the following equation:

SAD(MVoffset)=j=0,j+=2H-1i=0W-1absPred0'i,j-Pred1'i,j-DeltaDCMVoffset

To calculate DeltaDC values, two methods have been test as described below.

## Method1: DetlaDC calculation

In this method, DeltaDC is calcualtion for each search point separately. For this purpose, DC is calculated for both each prediction blocks (i.e., Pred0' and Pred1') using (only even rows are used)

DCL(MVoffset)=j=0, j+=2H-1i=0W-1PredL'i,j

Then DeltaDC is calculated as the difference between DC values of reference list L0 and L1.

DDC= DC0MVoffset-DC1MVoffset

num_shift=floorLog2W+floorLog2W-1

offset=1≪num_shift -1

DDC=DDC>0 ? DDC+offset≫num_shift:--DDC+offset≫num_shift

DeltaDCMVoffset= DDC

Method1 is more suitable to be implemented before SAD calculation.

Spec text for this variable:

#### Sum of absolute differences calculation process Inputs to this process are: - two variables nSbW and nSbH specifying the width and the height of the current subblock, - two ( nSbW + 4 ) x ( nSbH + 4 ) arrays pL0 and pL1 containing the predicted samples for L0 and L1 respectively, - an integer sample offset ( dX, dY ) in L0. Output of this process is: - the variable sad specifying the sum of absolute differences at the integer sample at the offset ( dX, dY ) in L0. - The variable DeltaDC is derived as follows: DeltaDC=x=0nSbW-1 y= 0nSbH/2-1 (pL0x+2+ dX2*y+ 2+ dY-pL1x+2-dX2*y+2-dY) (651) shift = Log2(SbW) + Log2 (sbH) − 1 (651) offset = (1<< (shift−1)) (651) DeltaDC = (DeltaDC>0) ? (DeltaDC+offset)>>shift : − ((−DeltaDC+offset)>>shift) (651) The variable sad is derived as follows: sad=x=0nSbW-1 y= 0nSbH/2-1 abs(pL0x+2+ dX2*y+ 2+ dY-pL1x+2-dX2*y+2-dY-DeltaDC) (651) When both dX and dY are equal to 0, the value of sad is modified as follows: sad = sad − ( sad >> 2 ) (652)

## Method2: DeltaDC interpolation

In this method, first DC values are calculated for four corner search points as mentioned above. Four corner search points are MVoffset = {(–2, –2), (+2, –2), (–2, +2), (+2 , +2)}.

And then DC values for other 21 remaining search points of MVoffset=(x,y) are estimated based on these four corner DC values using bi-linear interpolations as below:

DCLMVoffset=DC-2,-2×42-x×42-y+DC+2,-2×2+x×42-y+DC-2,+2×42-x×2+y+DC+2,+2×2+x×2+y≫4

Then DeltaDC values is calculated as described above for method 1.

Method2 is more suitable to be implemented after generating the initial prediction blocks (i.e., after initial motion compensation function). in VTM implementation, right after initial motion compensation, DC values are calculated for 4x4 subblocks, and corner DC values are calculating by accumulating DC values of the corresponding 4x4 blocks.

Spec text for this variation (Case 2):

### 8.5.3.2 Decoder side motion vector refinement process ### 8.5.3.2.1 General Inputs to this process are: - a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture, - a variable sbWidth specifying the width of the current coding subblock in luma samples, - a variable sbHeight specifying the height of the current coding subblock in luma samples, - the luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1, - the selected luma reference picture sample arrays refPicL0L and refPicL1L. Outputs of this process are: - delta luma motion vectors dMvL0 and dMvL1, - a variable dmvrSad specifying the mimimum sum of absolute differences. The variable subPelFlag is set equal to 0, the variable srRange is set equal to 2 and the integer sample offset ( intOffX, intOffY ) is set equal to ( 0, 0 ). Both components of the delta luma motion vectors dMvL0 and dMvL1 are set equal to zero and modified as follows: - For each X being 0 or 1, the ( sbWidth + 2 * srRange ) x ( sbHeight + 2 * srRange ) array predSamplesLXL of prediction luma sample values is derived by invoking the fractional sample bilinear interpolation process specified in 8.5.3.2.1 with the luma location ( xSb, ySb ), the prediction block width set equal to ( sbWidth + 2 * srRange ), the prediction block height set equal to ( sbHeight + 2 * srRange ), the reference picture sample array refPicLXL, the motion vector mvLX and the refinement search range srRange as inputs. - The 5x5 2-D array DeltaDC[ dX + 2 ][ dY + 2 ] with dX = −2..2 and dY = −2..2 is derived by invoking the DeltaDC derivation process specified in clause ‎8.5.3.6 with the width nSbW and height nSbH of the current coding subblock set equal to sbWidth and sbHeight, the prediction sample arrays pL0 and pL1 set equal to predSamplesL0L and predSamplesL1L, and the 5x5 2-D array DeltaDC [ dX + 2 ][ dY + 2 ] as output. - The variable minSad is derived by invoking the sum of absolute differences calculation process specified in clause 8.5.3.3 with the width sbW and height sbH of the current coding subblock set equal to sbWidth and sbHeight, the prediction sample arrays pL0 and pL1 set equal to predSamplesL0L and predSamplesL1L, and the offset ( dX, dY ) set equal to ( 0, 0 ), and DeltaDC [ 2 ][ 2 ] as inputs, and minSad as output. - The variable dmvrSad is set equal to minSad. - When minSad is greater than or equal to sbHeight * sbWidth, the following applies: - The 2-D array sadArray[ dX + 2 ][ dY + 2 ] with dX = −2..2 and dY = −2..2 is derived by invoking the sum of absolute differences calculation process specified in clause 8.5.3.3 with the width sbW and height sbH of the current coding subblock set equal to sbWidth and sbHeight, the prediction sample arrays pL0 and pL1 set equal to predSamplesL0L and predSamplesL1L, and the offset ( dX, dY ), and DeltaDC [ dX + 2 ][ dY + 2 ] as inputs, and sadArray[ dX + 2 ][ d Y + 2 ] as output. . . . . . . ### 8.5.3.3 Sum of absolute differences calculation process Inputs to this process are: - two variables nSbW and nSbH specifying the width and the height of the current subblock, - two ( nSbW + 4 ) x ( nSbH + 4 ) arrays pL0 and pL1 containing the predicted samples for L0 and L1 respectively, - an integer sample offset ( dX, dY ) in L0 - a variable DeltaDC. Output of this process is: - the variable sad specifying the sum of absolute differences at the integer sample at the offset ( dX, dY ) in L0. The variable sad is derived as follows: sad=x=0nSbW-1 y= 0nSbH/2-1 abs(pL0x+2+ dX2*y+ 2+ dY-pL1x+2-dX2*y+2-dY-DeltaDC) (651) When both dX and dY are equal to 0, the value of sad is modified as follows: sad = sad − ( sad >> 2 ) (652) . . . . . . #### 8.5.3.6 DeltaDC derivation process Inputs to this process are: - two variables nSbW and nSbH specifying the width and the height of the current subblock, - two ( nSbW + 4 ) x ( nSbH + 4 ) arrays pL0 and pL1 containing the predicted samples for L0 and L1 respectively, Output of this process is: - a 5x5 2-D array DeltaDC[ dX + 2 ][ dY + 2 ] with dX = −2..2 and dY = −2..2. Corner DCLX[ dX + 2 ][ dY + 2 ] variables with X being 0 or 1, dX = −2, 2 and dY = −2, 2 are derived as follows: DCLX dX + 2 [ dY+ 2 ]=x=0nSbW-1 y= 0nSbH/2-1 (pLXx+2+ dX2*y+ 2+ dY) (651) Other DCLX[ dX + 2 ][ dY + 2 ] variables with X being 0 or 1, dX = −2..2 and dY = −2..2 are derived as follows: DCLX dX + 2 dY+ 2 =DCLX00*2-dX*2-dy+ DCLX40*2+dX*2-dY+ DCLX04*2-dX*2+dY+ DCLX44*2+dX*2+dY≫4 (651) DeltaDC[ dX + 2 ][ dY + 2 ] variables with dX = −2..2 and dY = −2..2 are derived as follows: DeltaDC = DCL0[ dX + 2 ][ dY + 2 ] – DCL1[ 2 − dX][ 2 − dY ] (652) shift = Log2(nSbW) + Log2 (nSbH) − 1 (652) offset = (1<< (shift−1)) (652) DeltaDC = (DeltaDC>0) ? (DeltaDC + offset)>>shift : − ((−DeltaDC + offset)>>shift) (652) DeltaDC[ dX + 2 ][ dY + 2 ] = DeltaDC (652)

# Objective results

## CTC results

Results according to the JVET CTC are provided in the attached spreadsheet for two cases. Summary of the results is provided below. As shown in the following table, applying the proposal contributes to 0.10%  average BD-rate gain in RA test case, and 0.17% gain for class A1 sequences.

Table 1. Method 1: DC calculation

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15 % | -0.10 % | -0.08 % | 102 % | 100 % |
| Class A2 | -0.09 % | -0.12 % | -0.08 % | 102 % | 101 % |
| Class B | -0.09 % | -0.03 % | -0.06 % | 102 % | 101 % |
| Class C | -0.04 % | 0.05 % | 0.00 % | 101 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.09 % | -0.05 % | -0.05 % | 102 % | 100 % |
| Class D | -0.02 % | 0.07 % | 0.02 % | 100 % | 103 % |
| Class F | -0.07 % | -0.02 % | -0.03 % | 100 % | 101 % |

Table 2. Method 1: DC interpolation

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15 % | -0.07 % | -0.12 % | 102 % | 100 % |
| Class A2 | -0.09 % | -0.03 % | -0.08 % | 102 % | 101 % |
| Class B | -0.09 % | -0.07 % | -0.03 % | 102 % | 100 % |
| Class C | -0.05 % | -0.04 % | -0.01 % | 100 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.09 % | -0.05 % | -0.05 % | 102 % | 101 % |
| Class D | -0.02 % | -0.05 % | 0.02 % | 100 % | 103 % |
| Class F | -0.05 % | -0.05 % | -0.05 % | 100 % | 101 % |

## Non-CTC results (illumination change)

This test scenario has been designed to show that the performance of DMVR in the current VVC draft is sensitive to illumination difference of two prediction block. For this purpose, the sample values of input YUV is multiplied two a weight factor (w) which is changing periodically based on picture number (i.e. POC) as shown in Figure 2. It should be highlighted that this is only encoder side change.

Figure 2 Value of weight for different pictures

The values of input Y, U and V components are modified as follow:

Y = w*Y

U = w* (U-midValue) + midValue

V = w* (V-midValue) + midValue

midValue = 1 << (InputBitdepth -1)

Simulations are executed for three cases. Summary of results are provided in Table 3, Table 4, and Table 5. Table 3 shows that the proposed method should significant amount of gain in the case of illumination change. In this situation, DMVR-off (Table 4) indicates that DMVR tool bring loss in RD performance. DMVR-off results test for CTC is provided in Table 5 for the sake of completeness. These results suggest that the proposed method stabilize the performance of DMVR in different situations.

| Method | VTM Code | Input YUV |
| --- | --- | --- |
| Non-CTC anchor | VTM7.0 | Illumination change |
| Non-CTC test | VTM7.0+proposal | Illumination change |
| Non-CTC DMVR-Off | VTM7.0 + DMVR=Off | Illumination change |

Table 3. Non-CTC, Illumination change, w0 = 0.4, (Anchor: VTM7.0, Test: Method 2: DC interpolation)

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.12 % | -1.21 % | -1.21 % | 117 % | 95 % |
| Class A2 | -1.11 % | -1.75 % | -1.60 % | 116 % | 94 % |
| Class B | -0.80 % | -0.91 % | -0.80 % | 117 % | 96 % |
| Class C | -0.84 % | -1.05 % | -0.98 % | 99 % | 97 % |
| Class E |  |  |  |  |  |
| Overall | -0.94 % | -1.18 % | -1.09 % | 112 % | 96 % |
| Class D | -0.74 % | -0.74 % | -0.71 % | 99 % | 100 % |
| Class F | -0.51 % | -0.65 % | -0.58 % | 100 % | 99 % |

Table 4. Non-CTC, Illumination change, w0 = 0.4, (Anchor: VTM7.0, Test: VTm7.0 + DMVR-Off)

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25 % | -0.29 % | -0.08 % | 60 % | 95 % |
| Class A2 | -0.07 % | 0.02 % | -0.07 % | 60 % | 95 % |
| Class B | -0.05 % | 0.08 % | 0.16 % | 61 % | 95 % |
| Class C | 0.04 % | 0.08 % | 0.15 % | 98 % | 94 % |
| Class E |  |  |  |  |  |
| Overall | -0.07 % | -0.01 % | 0.07 % | 69 % | 95 % |
| Class D | -0.02 % | 0.20 % | 0.25 % | 99 % | 95 % |
| Class F | -0.01 % | 0.06 % | 0.15 % | 99 % | 97 % |

Table 5. CTC DMVR-off results

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.81 % | 1.06 % | 1.16 % | 99 % | 97 % |
| Class A2 | 1.38 % | 1.69 % | 1.69 % | 100 % | 97 % |
| Class B | 0.68 % | 0.90 % | 0.87 % | 100 % | 97 % |
| Class C | 0.63 % | 0.88 % | 0.88 % | 100 % | 96 % |
| Class E |  |  |  |  |  |
| Overall | 0.83 % | 1.08 % | 1.10 % | 100 % | 97 % |
| Class D | 0.68 % | 0.87 % | 0.87 % | 100 % | 95 % |
| Class F | 0.32 % | 0.40 % | 0.38 % | 101 % | 99 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).