| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0525-v2 |
| --- | --- |

| Title: | Non-CE4: Support large rotation and flipping in affine and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Deepa Naik | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Current syntax for affine motion vector supports signalling large rotation (e.g. close to 90/180/270 degree) and flipping, but engine of affine motion compensation and PROF are not working optimally for these cases. In these cases, there is a big discontinuity in borders of predicted subblocks, and PROF increases this discontinuity furthermore. This contribution proposes two solutions to fix these problems. At the first step, large rotation and flipping are detected based on affine motion vectors, and eight processing modes are defined for the current block. Then, based on the processing mode, 1) during PROF, motion vector refinements are modified, and/or 2) after performing subblock-based motion compensation, samples of the motion compensated subblocks are rotated or flipped. Under common test condition, the overall bdrate for RA and LDB is 0.00% and 0.03%, respectively. In point cloud coding (PCC) use case, where YUV sequences are generated from point cloud data, -2.81% and -3.12% overall bdrate is achieved for RA and LDB, respectively. The method in this proposal does not have noticeable affect on encoding and decoding time.

# Introduction/Problem statement

This proposal is based on JVET-P0600 which was marked as further study. It was also requested to provide evidence on more RD performance evidence, and to reduce bitdepth for calculating one of the parameters.

In the current working draft of VVC and in VTM7.0, there are 4-parameter and 6-parameter affine prediction modes. With 4- and 6-parameter afine, it is possible to signal a large rotation, for example around 90, 180, or 270 degree rotation. With 6-parameter affine, flipping (in four different direction) can also be indicated. This type of large rotation may happen in some of video content. For example, in point cloud coding (PCC), different patches in different frame may be packed with different orientations (e.g., rotation, flipping or their combinations) in a YUV picture, and the output YUV file is coded with a video codec. As another example, in 3x2 layout of cubemap projection, when a content goes from one face to another face, it may get rotated by 90 degree. But current subblock-based affine motion compensation and PROF method fails when affine parameters indicate large rotation or flipping.

## Subblock-based affine

In subblock-based motion compensation, subblocks have only translational motion vectors which are derived from affine motion vector of that block. So, in the case of large rotation or flipping, there is a big discontinuity in borders of motion compensated subblocks. Figure 1 illustrates an example where the object in the current frame is rotated by 90 degree clockwise with respect to the reference frame. This 90 degree rotation can be signalled using 4-parameter affine as shows in the figure. But the generated prediction block using subblock-based affine motion compensation has big discontinuity in subblock border, and is not a useful prediction to predict that block. Another example is illustrated in Figure 2 for the case of vertical flipping.

Figure 1. Subblock-based affine motion compensation in the case of 90-degree rotaion clockwise

Figure 2. Subblock-based affine motion compensation in the case of vertical flipping

## PROF

The assumption behind PROF method is that motion vector refinements are small (see Figure 3-(a)), so samples of motion compensated subblock can be refined using motion vector refinements and gradient of the samples of the subblocks. But in the case of large rotation and flipping, delta of affine motion vectors are large (see Figure 3-(b)), and motion vector refinements have large values. As a result, the above mentioned assumption does not hold, and applying PROF increase discontinuity in borders of subblocks.

| (a) | (b) |  |
| --- | --- | --- |

Figure 3. Motion vectore refinemts in PROF, (a) in the case of small rotation (e.g. 20 degree), (b) in the case of large rotation (e.g. 80 degree)

# Proposal

This contribution proposes two solutions to fix subblock-based motion compensation and PROF problems. In the following table, the proposed process for affine motion compensation is briefly compared to that of original ones in VTM7.0, and additional steps are highlighted. In the initialization step, first, delta motion vectors (DMV) are calculated based on affine motion vectors (i.e. motion vectors of different corner). Then a processing mode is determined based on DMV, and then a secondary DMV (DMV2) is calculated. Then for each subblock, a MV is calculated, motion compensation is executed, PROF is applied, and then predicted/refined subblock is rotated/flipped based on the processing mode.

Table 1. Origianl affine motion compensation versus the proposed method

| Step | VTM7.0 | Proposed method |
| --- | --- | --- |
| Initializatoin | Calculate DMV | Calculate DMV |
|  |  | Determine a processing mode based on DMV |
|  |  | Calculate DMV2 based on processing mode |
|  | Calculate MVR using DMV | Calculate MVR using DMV2 |
| For each subblock | Calculate subblock MV using DMV | Calculate subblock MV using DMV |
|  | Do motion compensation | Do motion compensation |
|  | Apply PROF using MVR | Apply PROF using MVR |
|  |  | Rotate/Flip predicted subblock-based on the processing mode |

## Flipping and rotation detection

At the first step, which is common for both solutions, large rotation and flipping are detected based on the affine motion vectors of the current block. For this purpose, as shown in Figure 4, based on affine motion vectors

- first, flipping is detected by examining the outer product of AB and AC vectors after affine transform, and

- second, rotation is detected by examining the location of (1,1) sample location with respect to the location of top-left corner after affine transformation.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 4. Detecting flipping and rotation, (a) current block, (b) prediction block after affine transform in the case of flipping (e.g. horizontal flipping), (c) prediction block after affine transform in the case of rotation (e.g. around 80 degree clockwise)

In this proposal, we have defined two modes of flipping (i.e. flipped or not flipped), and four modes of rotation (i.e. around 0/90/180/270 degree). Each mode of rotation covers +-45 degree around the main angle. For example, rotation mode of 90 degree cover rotation angle in the range of {45, 135} degree.

Outer product of AB and AC curve is calculated using the following equation. Compared to the previous proposal JVET-P0600, two right shift have been added to the equation to reduce the required number of bits for this operation

F=((((iDMvHorX+(1<<(iBit+4+iScaleX)))>>SH1)*((iDMvVerY+(1<<(iBit+4+iScaleX)))>>SV1))>>S2)-(((iDMvVerX>>SV1)*(iDMvHorY>>SH1))>>S2)

(1)

S2 = 16

SH1 = (iBit - floorLog2(W))

SH2 = (affineType == AFFINEMODEL_6PARAM) ? (iBit - floorLog2(H)) : SH1

iBit = MAX_CU_DEPTH = 7

and the location of (Ex, Ey) is calculated as below:

Ex = (iDMvHorX + iDMvVerX) + (1 << (iBit+4+iScaleX))

Ey = (iDMvHorY + iDMvVerY) + (1 << (iBit+4+iScaleY))

So, eight different processing modes are defined based on the combination of flipping and rotation as presented in table below:

Table 2. Different combinations of flipping and rotation

| F | Ex | Ey | Processing mode | W' | H' |
| --- | --- | --- | --- | --- | --- |
| ≥0 | >0 | >0 | No extra operation | W | H |
| ≥0 | >0 | ≤0 | Rotate 90 degrees | H | W |
| ≥0 | ≤0 | ≤0 | Rotate 180 degrees | W | H |
| ≥0 | ≤0 | >0 | Rotate 270 degrees | H | W |
| <0 | >0 | >0 | Flip Diagonal1 | H | W |
| <0 | >0 | ≤0 | Flip vertically | W | H |
| <0 | ≤0 | ≤0 | Flip Diagonal2 | H | W |
| <0 | ≤0 | >0 | Flip Horizontally | W | H |

## Calculation of motion vector refinement in PROF

In the next step, motion vector refinement for PROF is calculated according to the processing mode. The main idea here is to calculate the refined sample using the predicted sample which has the closest location. This will reduce the value of motion vector refinements.

| (a) | (b) |  |
| --- | --- | --- |

Figure 5. Modifying PROF process (by refining the sample values based on the closest sample)

In this proposal, based on the processing mode for the current block, a secondary delta motion vector is calculated using the original delta motion vector. Then motion vector refinements are derived based on the original PROF method using the secondary delta motion vector.

In current VVC draft (VTM7.0), delta motion vectors are calculated as below:

iDMvHorX = (mvRT - mvLT).getHor() << (iBit - g_aucLog2[cxWidth]);

iDMvHorY = (mvRT - mvLT).getVer() << (iBit - g_aucLog2[cxWidth]);

if ( pu.cu->affineType == AFFINEMODEL_6PARAM )

{

iDMvVerX = (mvLB - mvLT).getHor() << (iBit - g_aucLog2[cxHeight]);

iDMvVerY = (mvLB - mvLT).getVer() << (iBit - g_aucLog2[cxHeight]);

}

else

{

iDMvVerX = -iDMvHorY;

iDMvVerY = iDMvHorX;

}

According to this proposal, secondary delta motion vectors are calculated as mentioned in table below. And then motion vector refinements are calculated using secondary delta motion vectors using the process defined in the original PROF. It should be noted that this secondary delta motion vectors are used only in PROF, not in motion vector derivation for subblocks.

Table 3. Different combinations of flipping and rotation

| F | Ex | Ey | Processing mode | Secondary delta motion vector (DMV2) |
| --- | --- | --- | --- | --- |
| ≥0 | >0 | >0 | Normal case | DMvHorX2 = +DMvHorX DMvHorY2 = +DMvHorY DMvVerX2 = +DMvVerX DMvVerY2 = +DMvVerY |
| ≥0 | >0 | ≤0 | Rotate 90 | DMvHorX2 = +DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = +DMvVerY + (1<<(iBit+mvBits)) DMvVerX2 = -DMvHorX - (1<<(iBit+mvBits)) DMvVerY2 = -DMvHorY - (1<<(iBit+mvBits)) |
| ≥0 | ≤0 | ≤0 | Rotate 180 | DMvHorX2 = -DMvHorX - (2<<(iBit+mvBits)) DMvHorY2 = -DMvHorY + (0<<(iBit+mvBits)) DMvVerX2 = -DMvVerX + (0<<(iBit+mvBits)) DMvVerY2 = -DMvVerY - (2<<(iBit+mvBits)) |
| ≥0 | ≤0 | >0 | Rotate 270 | DMvHorX2 = -DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = -DMvVerY - (1<<(iBit+mvBits)) DMvVerX2 = +DMvHorX + (1<<(iBit+mvBits)) DMvVerY2 = +DMvHorY - (1<<(iBit+mvBits)) |
| <0 | >0 | >0 | Flip Diagonal1 | DMvHorX2 = +DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = +DMvVerY + (1<<(iBit+mvBits)) DMvVerX2 = +DMvHorX + (1<<(iBit+mvBits)) DMvVerY2 = +DMvHorY - (1<<(iBit+mvBits)) |
| <0 | >0 | ≤0 | Flip vertically | DMvHorX2 = +DMvHorX DMvHorY2 = +DMvHorY DMvVerX2 = -DMvVerX DMvVerY2 = -DMvVerY - (2<<(iBit+mvBits)) |
| <0 | ≤0 | ≤0 | Flip Diagonal2 | DMvHorX2 = -DMvHorX - (2<<(iBit+mvBits)) DMvHorY2 = -DMvHorY DMvVerX2 = +DMvVerX DMvVerY2 = +DMvVerY |
| <0 | ≤0 | >0 | Flip Horizontally | DMvHorX2 = -DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = -DMvVerY - (1<<(iBit+mvBits)) DMvVerX2 = -DMvHorX - (1<<(iBit+mvBits)) DMvVerY2 = -DMvHorY - (1<<(iBit+mvBits)) |

The operation in the above table can be implemented using linear equations:

int p[2 * 4][2 * 4] =

{

{+1, +0, +0, +1, +0, +0, +0, +0},

{+0, +1, -1, +0, -1, +1, -1, -1},

{+0, -1, +1, +0, -1, -1, +1, -1},

{-1, +0, +0, -1, -2, +0, +0, -2},

{+0, +1, +1, +0, -1, +1, +1, -1},

{+1, +0, +0, -1, +0, +0, +0, -2},

{-1, +0, +0, +1, -2, +0, +0, +0},

{+0, -1, -1, +0, -1, -1, -1, -1}

};

int iOperationMode = 4*((F<0) ? 1 : 0) + 2*((Ex>0) ? 0 : 1) + ((Ey> 0) ? 0 : 1);

int* pp = p[iOperationMode];

iDMvHorX2 = pp[0] * iDMvHorX + pp[1] * iDMvVerX + (pp[4] << (iBit + 4));

iDMvHorY2 = pp[0] * iDMvHorY + pp[1] * iDMvVerY + (pp[5] << (iBit + 4));

iDMvVerX2 = pp[2] * iDMvHorX + pp[3] * iDMvVerX + (pp[6] << (iBit + 4));

iDMvVerY2 = pp[2] * iDMvHorY + pp[3] * iDMvVerY + (pp[7] << (iBit + 4));

## Rotation/Flipping of predicted subblock

Finally, samples of subblock are rotated or flipped according to the processing mode.

## Point cloud coding use case

In this use case, the YUV files, generated according to point cloud coding (PCC), are used as input test sequences. In PCC, each point cloud frame is partitioned and projected to several patches, and then these patches are packed into a YUV picture. During the packing step, to minimize the YUV picture size and preserve the spatial continuity of the YUV content, patches in different frames are packed in different orientations. Basically, 8 different orientation (combinations of flipping and rotation by 0/90/180/270) are defined for packing of patches in PCC. In practice two orientation (i.e. 0 degree and swap) are used at encoder side optimization. However, in another scenario, patch stabilization is employed by pre-processing of a group of frames (e.g. 16 frames) to find the best packing for that group. Since the 2D grid has a finite resolution, it is possible that two or more 3D points mapped onto the same pixel position. Therefore, in order to resolve this issue, an optional second image layer is introduced to cover inner and outer surface points. The points with lowest depth are stored in the first layer and the second layer may collect the points with the highest depth value. The second picture is visually identical to the first picture except for few extra pixels and patches. Hence, in this proposal, to reduce the simulations time and use GOP structure of JVET-CTC, only the first frames are kept (i.e. the second frames are dropped from the generated YUV file).

An example of point cloud YUV picture is illustrated in Figure 6 for “Longdress” test sequence, where some of patches have different orientations in different pictures. For example, back side of the lady in the left side of picture #0, is flipped (swapped) and placed in center of picture #3. As another example, the front side of the lady at top of picture #0 is flipped and packed on the lefts side of picture #3.

| Picture #0 | Picture #3 |
| --- | --- |

Figure 6 An example of point cloud content projected into YUV pictures, “Longdress” test sequence

The affine motion estimation algorithm in VTM7.0 does not have capability of finding the proper affine motion vector for these cases. To capture these motions, a meta data file is extracted from PCC codec, and fed to affine motion estimation function in VTM. Most of this information is already generated in the PCC code (and printed to its output log). This meta data file includes the following information for patch in each YUV picture: patch location (top-left corner) in YUV picture, patch size (i.e. width and height), patch orientation (0 means no swapping, and 1 mean swapping), patch unique ID (we have added this extra data).

In our affine motion estimation algorithm, based on the location of a block and those meta data, an initial affine motion vector is derived for that block. Then this motion vector is fed to the existing affine motion estimation algorithm as the starting point, and the existing affine motion estimation in VTM7.0 does the motion vector search. Other than this, no other change is made in the affine motion estimation algorithm of VTM7.0.

It should be highlighted that this modification in affine motion estimation is performed when the patch has rotation/flipping, not the normal translational motion vector. This means that the observed gain is mainly because of proposed method for affine motion compensation (Table 5 and Table 6).

# Objective results

## JVET-CTC case

Results according to the JVET CTC are provided in the attached spreadsheet. The summary of the results, provided below, shows this method has negligible effect on RD performance of the codec under CTC condition.

Table 4. RD performance for CTC

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 % | 0.01 % | 0.00 % | 101 % | 100 % |
| Class A2 | -0.03 % | -0.04 % | -0.02 % | 102 % | 101 % |
| Class B | 0.01 % | -0.01 % | 0.03 % | 102 % | 99 % |
| Class C | 0.00 % | 0.01 % | -0.05 % | 100 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | -0.01 % | -0.01 % | 101 % | 100 % |
| Class D | 0.02 % | -0.07 % | 0.04 % | 100 % | 99 % |
| Class F | -0.02 % | -0.04 % | -0.02 % | 100 % | 98 % |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01 % | -0.15 % | 0.16 % | 101 % | 99 % |
| Class C | 0.03 % | -0.23 % | -0.03 % | 101 % | 99 % |
| Class E | 0.10 % | 0.35 % | -0.03 % | 101 % | 99 % |
| Overall | 0.03 % | -0.05 % | 0.05 % | 101 % | 99 % |
| Class D | -0.01 % | 0.08 % | -0.05 % | 101 % | 99 % |
| Class F | 0.00 % | -0.30 % | 0.57 % | 101 % | 99 % |

## Non-CTC: Point cloud coding use case

in PCC there are 5 test sequences including Longdress, Loot, Queen, RedAndBlack, and Soldier. Queens has 250 frames, and all the others have 300 frames. According to PCC method, each test data is divided to group of frames which include 32 frames. Each of these PCC contents is divided into 10 GOF, each has 32 frames (except the last GOF). Each GOF in each PCC content has its own resolution (size of different GOFs are around 1280x1280, a bit bigger or a bit smaller). This means that each GOF is treated as a separate YUV file for encoding.

Details of simulation results (for each GOF) are available in the attached spreadsheet. Summary of results are provided in Table 5. Table 6 presents the results when only the modified affine motion estimation is enabled (not the proposed affine motion compensation modification). Comparing results in these two tables that thee RD performance gain in Table 5 is mainly because of hte proposed modification in this proposal, not the modified affine motion estimation.

Table 5. RD performance for PCC use case (proposed method + modified affine motion estimation)

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Longdress | -0.36 % | -0.45 % | -0.37 % | 99 % | 99 % |
| Loot | -3.98 % | -4.43 % | -3.99 % | 99 % | 97 % |
| Queen | -2.26 % | -2.31 % | -2.11 % | 99 % | 99 % |
| Red&Black | -0.50 % | -0.49 % | -0.38 % | 100 % | 98 % |
| Soldier | -6.97 % | -10.37 % | -10.08 % | 99 % | 100 % |
| Overall | -2.81 % | -3.61 % | -3.39 % | 99 % | 98 % |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Longdress | -0.76 % | -0.94 % | -0.84 % | 99 % | 100 % |
| Loot | -5.25 % | -5.86 % | -5.09 % | 99 % | 97 % |
| Queen | -2.28 % | -2.69 % | -2.28 % | 99 % | 100 % |
| Red&Black | -0.77 % | -1.03 % | -0.70 % | 98 % | 95 % |
| Soldier | -6.53 % | -10.32 % | -9.94 % | 98 % | 98 % |
| Overall | -3.12 % | -4.17 % | -3.77 % | 99 % | 98 % |

Table 6. RD performance for PCC use case (Only modified affine motion estimation)

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Longdress | -0.13 % | -0.18 % | -0.15 % | 50 % | 102 % |
| Loot | -0.21 % | -0.30 % | -0.05 % | 51 % | 97 % |
| Queen | -0.02 % | 0.03 % | -0.01 % | 58 % | 98 % |
| Red&Black | -0.01 % | 0.20 % | 0.08 % | 100 % | 100 % |
| Soldier | -0.24 % | -0.02 % | -0.13 % | 100 % | 101 % |
| Overall | -0.12 % | -0.06 % | -0.05 % | 68 % | 100 % |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Longdress | -0.08 % | -0.30 % | -0.13 % | 52 % | 98 % |
| Loot | -0.30 % | -0.10 % | -0.38 % | 52 % | 98 % |
| Queen | -0.04 % | -0.49 % | -0.27 % | 59 % | 98 % |
| Red&Black | -0.11 % | 0.15 % | -0.08 % | 100 % | 98 % |
| Soldier | -0.36 % | -0.73 % | -0.53 % | 99 % | 101 % |
| Overall | -0.18 % | -0.30 % | -0.28 % | 69 % | 99 % |

(Highlighted encoding times in above tables are not accurate as anchor and test methods have been executed on different clusters)

# Conclusion

The method in this proposal enhances the functionality of affine motion compensation and fix PROF for the cases that there is large rotation or flipping in video contents. It almost does not change the RD performance of the codec under CTC, and it is expected to provide RD performance gain in special cases like coding of point cloud content or 360-degree content in 3x2 layout of cubemap projection. The observed bdrate for YUV generated by PCC is -2.81% and -3.12% for RA and LDB, respectively. Hence, it is requested to adopt this method into current VVC draft.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).