| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0526 |
| --- | --- |

| Title: | GEO support for 4xN and Nx4 CU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Limin Wang | Tel: Email: | +1 858-674-6883 krit.panusopone@nokia.com seungwook.hong@nokia.com limin.2.wang@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Geometric merge mode (GEO) is proposed in 15th Gothenburg JVET meeting. This contribution extends GEO support to 4xN and Nx4 CU. Using VTM-7.0 as a reference, simulation results show that allowing all 82 modes as in CE4 common base have luma BD-rate of approximately -0.33% and -0.70% for RA and LB configurations, respectively. Simulation results show that allowing all 82 modes as in CE4 combined test 1 have luma BD-rate of approximately -0.29% and -0.67% for RA and LB configurations, respectively.

# Introduction

GEO operates using two uni-directional merge motion vectors to produce a final predicted block based on blending of two merge prediction blocks.  Blending masks determines how CU is split into 2 partitions. CE4 [1] investigates inter prediction with geometric partitioning (GEO) based on the technique proposed in JVET-P0884/P0885 [2],[3]. In CE4 [1], GEO is implemented on top of VTM-7.0 as a common base for CE4 experiments. CE4 common base allows 82 mode partition for CU width and CU height larger than or equal to 8 and smaller than 128.

A simplified GEO [4] combines several CE4 tests to reduce GEO complexity. In method 1, combined test 1, includes CE4-1, CE4-2.1, CE4-3.1, decreases number of GEO partitions, allows GEO block shapes, and skips SBT.

# Proposed Solution

In method 1, like GEO in CE4 common base, a 4xN and Nx4 CU using GEO has 2 partitions and each partition has its own uni-prediction temporal prediction MV. Moreover, same signaling is used to signal GEO prediction mode for 4xN and Nx4 CU using GEO.

Syntax to support GEO extension to 4xN and Nx4 is slightly different because GEO is now allowed for 4xN and Nx4 CU but no 4x4, 4x8, and 8x4. The related syntax is shown in the following merge data syntax table.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

In method 2, like in [4], allows 64 partition modes and limits GEO to CU smaller than 128. 4xN and Nx4 CU using GEO has 2 partitions and each partition has its own uni-prediction temporal prediction MV. Moreover, same signaling is used to signal GEO prediction mode for 4xN and Nx4 CU using GEO.

# Experimental Results

The proposed method 1 is implemented on top of VTM-7.0 CE4 common base. Experimental results are shown in Tables 1 and 2 against the VTM-7.0 under RA and LB CTC [5], respectively. Additionally, experimental results are shown in Tables 3 and 4 against the CE4 common base under RA and LB CTC, respectively.  The proposed method 2 is implemented on top of VTM-7.0 combined test 1. Experimental results are shown in Tables 5 and 6 against the VTM-7.0 under RA and LB CTC [5], respectively.  Similarly, experimental results are shown in Tables 7 and 8 against the CE4 combined test 1 under RA and LB CTC, respectively.

Table 1. Simulation result of method 1, compared to VTM-7.0, for RA

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.13% | -0.26% | 106% | 102% |
| Class A2 | -0.25% | -0.19% | -0.09% | 106% | 102% |
| Class B | -0.18% | -0.16% | -0.22% | 106% | 101% |
| Class C | -0.72% | -1.10% | -0.99% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.41% | -0.41% | 106% | 101% |
| Class D | -0.60% | -0.94% | -0.87% | 105% | 100% |
| Class F | -0.35% | -0.44% | -0.40% | 104% | 100% |

Table 2. Simulation result of method 1, compared to VTM-7.0, for LB

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.59% | -0.55% | 108% | 99% |
| Class C | -0.85% | -1.32% | -1.29% | 107% | 98% |
| Class E | -1.00% | -0.34% | -0.55% | 103% | 98% |
| Overall | -0.70% | -0.77% | -0.80% | 106% | 98% |
| Class D | -0.85% | -1.34% | -1.54% | 107% | 101% |
| Class F | -0.64% | -1.05% | -0.61% | 105% | 98% |

Table 3. Simulation result of method 1, compared to CE4 common base, for RA

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.08% | 103% | 100% |
| Class A2 | -0.03% | 0.00% | -0.08% | 104% | 101% |
| Class B | -0.04% | -0.05% | -0.05% | 102% | 99% |
| Class C | -0.14% | -0.12% | -0.10% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.06% | -0.07% | 103% | 100% |
| Class D | -0.18% | -0.28% | -0.26% | 102% | 97% |
| Class F | -0.03% | -0.06% | -0.07% | 102% | 99% |

Table 4. Simulation result of method 1, compared to CE4 common base, for LB

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.08% | 0.05% | 103% | 99% |
| Class C | -0.06% | -0.07% | -0.40% | 102% | 98% |
| Class E | 0.17% | -0.15% | 0.33% | 102% | 99% |
| Overall | 0.01% | -0.09% | -0.03% | 102% | 99% |
| Class D | -0.17% | -0.89% | -0.50% | 102% | 100% |
| Class F | -0.13% | -0.15% | -0.31% | 102% | 99% |

Table 5. Simulation result of method 2, compared to VTM-7.0, for RA

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.16% | -0.22% | 198% | 159% |
| Class A2 | -0.20% | -0.17% | -0.10% | 195% | 159% |
| Class B | -0.15% | -0.13% | -0.18% | 195% | 154% |
| Class C | -0.67% | -1.10% | -1.10% | 195% | 151% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.40% | -0.42% | 196% | 155% |
| Class D | -0.52% | -0.97% | -0.79% | 192% | 155% |
| Class F | -0.31% | -0.50% | -0.41% | 184% | 147% |

Table 6. Simulation result of method 2, compared to VTM-7.0, for LB

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.43% | -0.29% | 103% | 98% |
| Class C | -0.89% | -1.37% | -1.19% | 103% | 97% |
| Class E | -1.02% | 0.03% | -0.63% | 100% | 97% |
| Overall | -0.67% | -0.63% | -0.68% | 102% | 97% |
| Class D | -0.83% | -0.83% | -1.03% | 104% | 101% |
| Class F | -0.60% | -0.25% | -0.10% | 102% | 98% |

Table 7. Simulation result of method 2, compared to CE4 combined test 1, for RA

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 combined test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.01% | 101% | 98% |
| Class A2 | -0.04% | -0.01% | -0.01% | 101% | 99% |
| Class B | -0.02% | -0.02% | -0.07% | 101% | 99% |
| Class C | -0.13% | -0.19% | -0.26% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.05% | -0.10% | 101% | 99% |
| Class D | -0.11% | -0.45% | -0.21% | 101% | 100% |
| Class F | -0.05% | -0.17% | -0.13% | 100% | 99% |

Table 8. Simulation result of method 2, compared to CE4 combined test 1, for LB

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 combined test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.05% | -0.02% | 101% | 98% |
| Class C | -0.17% | -0.24% | -0.32% | 101% | 99% |
| Class E | -0.05% | 0.14% | -0.74% | 100% | 100% |
| Overall | -0.08% | -0.03% | -0.30% | 100% | 99% |
| Class D | -0.17% | 0.09% | -0.71% | 102% | 103% |
| Class F | -0.11% | 0.59% | -0.09% | 100% | 101% |

# Conclusions

This contribution presents simulation results of GEO extension to support 4xN and Nx4 CU. The results show method 1 has a BD-rate performance of -0.06% and 0.01%, compared to CE4 common base under RA and LB coding conditions, respectively. Against CE4 combined test 1, method 2 has a BD-rate performance of -0.05% and -0.08%, compared to CE4 combined test 1 under RA and LB coding conditions, respectively

# References

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Oct. 2019.

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVE-P0264, and JVET-P0304)”, JVET-P0884, Oct. 2019.

- K. Reuze, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, R.-L. Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVE-P0264, and JVET-P0304)”, JVET-P0885, Oct. 2019.

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, K. Reuzé, H. Huang, T. Hsieh, V. Seregin, C.-C Chen, H. E. Egilmez, W.-J Chien, M. Karczewicz, R.-L Liao, J. Chen, Y. Ye, J. Luo, L. Xu, F. Chen, L. Wang, Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, M. Bläser, J. Sauer, “CE4 related: Combined Test of CE4-1, CE4-2 and CE4-3”, JVET-Q0061, Jan. 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).