| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0527 |
| --- | --- |

| Title: | AHG9: Gradual Decoding Refresh for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong Krit Panusopone | Tel: Email: | +1 858-335-9196 limin.2.wang@nokia.com seungwook.hong@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes Gradual Decoding Refresh (GDR) for VVC, in which a GDR picture is separated by a virtual boundary into clean area and dirty area. For CUs in clean area, the virtual boundary is treated as picture boundary and all the rules associated with pictures boundary in VVC apply. For CUs in dirty area, the virtual boundary is treated as no boundary. In addition, for CUs in clean area, a separate HMVP table is maintained and updated with the candidates only coming from previously-coded CUs in clean area.

# Introduction

Picture Header was adopted in VVC at the 16th JVET meeting, Geneva, Oct. 2019, which includes Virtual Boundary syntax [1]. Virtual boundary can be vertical or horizontal line(s). With virtual boundary syntax included in picture header, a GDR picture can define its own virtual boundaries. A typical GDR picture consists of clean area and dirty area, where clean area may contain a forced intra area next to dirty area for progressive intra refresh (PIR). A virtual boundary signaled at Picture Header can be used to indicate the boundary between clean area and dirty area of a GDR picture.

Fig. 1 illustrates the basic concept of (vertical) GDR, where with forced intra coded areas (green) moving from left to right, the clean area (yellow) expends gradually from random access point of POC(n) to recovery point of POC(N+n). And a virtual boundary (red) separates the clean area and the dirty area of a GDR picture.

Fig. 1. GDR pictures where clean area and dirty area are separated by a virtual boundary.

For exact_match at recovery point, CUs in clean area cannot use any coding information (e.g. reconstructed pixels, code mode, MVs, refIdx, etc.) from dirty area. It is the encoder responsibility to make sure exact_match at recovery point.

Practically, a VVC encoder needs to impose the necessary restrictions on almost all the possible coding tools for CUs in clean area and make sure they will not touch any coding information in dirty area. For example,

- For intra CUs in clean area, encoder with GDR functionality needs to check and make sure that intra predictions will not use any reference samples in dirty area of the current picture.

- For inter CUs in clean area, encoder with GDR functionality needs to check and make sure that the (interpolated) prediction blocks will not use any reconstructed pixels in dirty areas of reference pictures.

- For merge mode CUs in clean area, encoder with GDR functionality needs to check and make sure that temporal candidates in dirty areas of reference pictures will not be included in the merge list.

- For affine mode CUs in clean area, encoder with GDR functionality needs to check and make sure that the (interpolated) prediction blocks for each of 4x4 subblocks will not use any reconstructed pixels in dirty areas of reference pictures.

- For triangle mode CUs in clean area, encoder with GDR functionality needs to perform validation at a proper stage, otherwise part of motion information may not be available.

- For inter CUs in clean area, if needed to build the merge list using HMVP, encoder with GDR functionality needs to avoid selecting the candidates associated with CUs in dirty area of the current picture.

# It can be very complicated and costly to impose and validate the restrictions on the coding tools for CUs in clean area, which will likely lead to expensive encoder with GDR functionality, as compared to regular encoder.

# Proposed Solution

To avoid such expensive validation process, it is proposed to make a few minor changes to the current VVC spec for GDR applications. Specifically,

- For CUs in clean area, virtual boundaries are treated as picture boundaries. All the clauses associated with picture boundaries in the current VVC spec apply.

- For CUs in dirty area, virtual boundaries are treated as no boundaries.

- In addition, for CUs in clean area, a separate HMVP table is maintained and updated, where the candidates only come from previously-coded CUs in clean area.

A syntax called gdr_virtual_boundaries_treated_as_pic_boundaries_flag is added in SPS syntax table as follows.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ...... |  |
| gdr_enabled_flag | u(1) |
| if( gdr_enabled_flag ) |  |
| gdr_virtual_boundaries_treated_as_pic_boundaries_flag | u(1) |
| …… |  |
| } |  |

gdr_virtual_boundaries_treated_as_pic_boundaries_flag equal to 1 specifies that virtual boundaries of each coded gdr picture in the CLVS are treated as picture boundaries for CUs in clean area in the decoding process. gdr_virtual_boundaries_treated_as_pic_boundaries_flag equal to 0 specifies that virtual boundaries of each coded gdr picture in the CLVS are not treated as picture boundaries for CUs in clean area in the decoding process. When not present, the value of gdr_virtual_boundaries_treated_as_pic_boundaries_flag is inferred to be equal to 0.

Let ( xCb, yCb ) specify the top-left sample of a current coding block relative to the top-left luma sample of the current picture, cbWidth specify the width of the current coding block in luma samples, and cbHeight specify the height of the current coding block in luma samples.

For vertical gdr applications, with gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or virtual boundary treated as “picture boundary”), a current CU is in clean area if xCb + cbWidth is smaller than (vertical) virtual boundary 8 * ph_virtual_boundaries_pos_x[ 0 ] specified in Picture Header syntax table of VVC [1].

For horizontal gdr applications, with gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or horizontal boundary treated as “picture boundary”), a current CU is in clean area if yCb + cbHeight is smaller than (horizontal) virtual boundary 8 * ph_virtual_boundaries_pos_y[ 0 ] specified in Picture Header syntax table of VVC [1].

With gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or virtual boundary treated as “picture boundary”), for intra CUs in clean area, the reference samples in dirty area are considered “not available”, and therefore, if needed, they will be replaced by the reference samples from the clean area according to the current VVC design.

Fig. 2 shows an intra CU (green) in clean area, some of its reference samples (orange) are in dirty area and they will be replaced by the reference samples in clean area.

Fig. 2. Reference samples in dirty area are considered “not available”.

With gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or virtual boundary treated as “picture boundary”), for inter CUs in clean area, the reconstructed pixels in dirty area of reference pixels will be padded from the reconstructed pixels in clean area (or set to a pre-determined value, e.g. 2BD-1), which will give prediction blocks the freedom over the entire reference pictures, instead of being limited to the clean areas of reference pictures.

With gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or virtual boundary treated as “picture boundary”), for inter CUs in clean area, the coding information in dirty areas of reference pictures are considered “not available”, which will prevent CUs in clean area from using the coding information in dirty areas of reference pictures via TMVP.

Fig. 3 shows reference picture for CUs in clean area, where the reconstructed pixels in dirty area are padded from the reconstructed pixels in clean area and the coding information in dirty area are considered “not available”.

Fig. 3. Reference picture for CUs in clean area.

With gdr_virtual_boundaries_treated_as_pic_boundaries_flag set to 1 (or virtual boundary treated as “picture boundary”), for CUs in clean area, a separate HMVP table is maintained and updated, which contains only the coding information of previously-coded CUs in clean area. The coding information associated with CUs in dirty area will therefore not be included in the merge list.

Fig. 4 shows an example, where CU(14) is a current CU in clean area, and the candidates in regular HMVP table include CUs 13, 12, 11, 10, 9, where CUs 10 and 9 are in dirty area, and the candidates in separate HMVP table for CUs in clean area contain CUs 13, 12, 11, 3, 2 - they are all in clean area.

Fig. 4. HMVP table for CUs in clean area contains the candidates from CUs in clean area.

# Experimental Results

The proposed GDR (with vertical strips set to pic_width / frame_rate) was implemented on VTM7.0. Simulations were conducted over JVET test sequences, and each test is performed using low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

- The first picture is coded as an IDR picture,

- The GOP size is 1.

- Each picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

- SEIDecodedPictureHash is set to 1.

- LoopFilterAcrossVirtualBoundariesDisabledFlag is 1,

- HashME is off for class F,

- GDR refresh period is set to frame rate.

The test results are compared with GDR anchor [2-3]. Anchor bitstreams are encoded using low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

- The first picture is coded as an IDR picture,

- The GOP size is 1.

- Each picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and Qpoffset are set equal to 0

- SEIDecodedPictureHash is set to 1.

- CRA picture is inserted every second.

All the GDR test bitstreams were checked for leaks, and none of them shows leak.

The performance results are shown in Table 1 in terms of BD rate. The overall performance loss against GDR anchor is about 3.27%. Note that a loss for GDR should be expected because CUs in clean area are not allowed to use any coding information from dirty areas.

Table 1. Simulation Results of proposed GDR

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 GDR Anchor vs VTM 7.0 GDR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.57% | 4.67% | 7.80% | 98% | #NUM! |
| Class C | 2.24% | 3.41% | 3.99% | 99% | #NUM! |
| Class E | 7.49% | 11.34% | 13.38% | 95% | #NUM! |
| Overall | 3.27% | 5.92% | 7.93% | 97% | #NUM! |
| Class D | 2.80% | 5.68% | 6.27% | 98% | #NUM! |
| Class F | 9.64% | 11.51% | 12.42% | 99% | #NUM! |

For information purpose, [4] reported an overall performance loss of an encoder only GDR (CE2.1b) against CE2 GDR anchor (VTM6.0) is 6.54%, as shown in Table 2, where virtual boundaries are slice-level syntax.

Table 2. Simulation Results of CE2.1b from [4]

|  | Low delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 CE2 GDR Anchor |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 2.86% |  | 6.15% |  | 9.64% | 110% | 101% |
| Class C | 5.62% |  | 6.74% |  | 7.33% | 102% | 102% |
| Class E | 13.90% |  | 16.21% |  | 18.72% | 108% | 103% |
| Overall | 6.54% |  | 8.86% |  | 11.14% | 107% | 102% |
| Class D | 9.84% |  | 12.67% |  | 13.65% | 104% | 102% |
| Class F | 32.50% |  | 39.65% |  | 40.78% | 123% | 107% |

As seen, the proposed GDR is able to reduce the loss, as compared to encoder only GDR, by around 3%.

# Conclusions

The proposed GDR imposes no restrictions on the coding tools for CUs in clean area and avoid complicated validation process (potentially low the cost for VVC encoder with GDR functionality), making code structure much clean and simple. The proposed GDR improves coding performance, as compared to encoder-only GDR, by around 3%.

# References

- B. Bross, J. Chen,  S. Liu,  and Y.-K. Wang, “Versatile Video Coding”, JVET-P2001-v14, Oct 2019.

- K. Kazui, J.M Thiesse, Hendry, L. Wang and K. Kawamura, “Description of Core Experiment 2 (CE2): Gradual decoding refresh”, JVET-O2022, July 2019.

- K. Kazui, J.M Thiesse, Hendry, L. Wang and K. Kawamura, “CE2: Summary Report on Gradual Decoding Refresh”, JVET-P0022, Oct. 2019.

- D. Gommelet, J.M. Thiesse, and D. Nicholson, “CE2: Gradual Decoding Refresh (GDR) using encoder and normative restrictions (Tests 2.1.a, 2.1.b and 2.1.c)”, JVET-P0193 Oct. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).