| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0529-v2 |
| --- | --- |

| Title: | On LFNST index and MTS index signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Man-Shu Chiang, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, man-shu.chiang, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 7, when the last significant coefficient position of a luma transform block (TB) is in the multiple transform selection (MTS) zero-out region (i.e., the TB excluding the top left 16x16 region), this reveals that MTS is not selected at encoder, so the MTS index is not parsed and inferred to 0 (i.e., indicating DCT-II) at decoder. However, redundant MTS index signalling is not avoided thoroughly by using the current design. The MTS index is still signalled but always equal to 0 when there is any non-last significant coefficient in the MTS zero-out region. This contribution proposes to parse low frequency non-separable transform (LFNST) index and MTS index right after parsing the first nonzero TB’s last significant coefficient position. If the MTS index is not equal to 0, coefficients in the MTS zero-out region will not be parsed and inferred to 0. In this way, there is no redundancy of MTS index signalling or coefficient signalling in MTS blocks. Since LFNST index is parsed in the first nonzero TB, the LFNST index parsing checks related to the last significant coefficient position are only considered for the first nonzero TB. Since both LFNST index and MTS index can be obtained right after parsing the first nonzero TB’s last significant coefficient position, the inverse transform process can be performed much earlier before parsing the entire coding unit (CU) as needed in the current VVC design. It is asserted that the proposed method can reduce the latency between inverse transform process and parsing stage and is beneficial to hardware implementation. In common test condition (CTC), the results shows 0.01%, 0.01% and 0.03% luma BD-rates under AI, RA, and LB, respectively.

# Introduction

In VVC Draft 7 [1], MTS provides various core transform modes in addition to DCT-II. For intra subpartitions (ISP) and subblock transform (SBT), the core transform mode for MTS is decided implicitly. For other cases, an MTS index is signalled to indicate the selected core transform mode. According to the adopted JVET-P1026 [2], to parse the transform syntax, the MTS index is parsed at the end of one CU. Moreover, when MTS is applied, transform coefficients outside the top left 16x16 region are zeroed out for luma TB. Therefore, if any nonzero transform coefficient is located in the MTS zero-out region (i.e., the TB excluding the top left 16x16 region), this means MTS is not selected at encoder, and the MTS index is not parsed and inferred to 0 at decoder. In VVC Draft 7, a variable MtsZeroOutSigCoeffFlag is used to reduce redundant MTS index signalling as follows.

- Before parsing each TB in a CU, MtsZeroOutSigCoeffFlag is initialized as 1.

- For each luma TB, if the last significant coefficient position is in the MTS zero-out region, MtsZeroOutSigCoeffFlag is changed to be 0.

- If MtsZeroOutSigCoeffFlag is equal to 1, the MTS index is parsed; otherwise, the MTS index is inferred to be 0.

However, using only the last significant coefficient position fails to prevent redundant MTS index signalling thoroughly. When the last significant coefficient position is located in the top left 16x16 region, a non-last significant coefficient still could locate in the MTS zero-out region. In VVC Draft 7, a bitstream conformance requirement is imposed to forbid this case. In this contribution, it is proposed to signal LFNST index and MTS index before signalling coefficients in the first nonzero TB.

# Proposed method

It is proposed to signal LFNST index and MTS index right after signalling the first nonzero TB’s last significant coefficient position. Since the MTS index is parsed before parsing non-last coefficients, if the MTS index indicates the core transform mode is not DCT-II, coefficients in the MTS zero-out region will not be parsed. Besides, the position of signalling LFNST index is also changed. In the proposed method, the LFNST index is signalled right before the MTS index, and the LFNST index parsing checks related to the last significant coefficient position are only considered for the first nonzero TB. If the LFNST index is parsed at the first nonzero TB and is not equal to 0, both the x-position and y-position of the last significant coefficient of every remaining nonzero TBs in the same coding unit (CU) are constrained to be smaller than 4 by related syntax design. To prevent violating the current normative rule of the last significant coefficient position in the LFNST block for 4x4 or 8x8 TB in VVC Draft 7, a bitstream conformance requirement is added that in one CU, the LFNST index shall be equal to 0 if any 4x4 or 8x8 TB in the CU has last significant coefficient position larger than 7 in diagonal scan.

Since both LFNST index and MTS index can be obtained right after parsing the first nonzero TB’s last significant coefficient position, the inverse transform process can be performed much earlier before parsing the entire CU as needed in the current VVC design. It is asserted that the proposed method can reduce the latency between inverse transform process and parsing stage and is beneficial to hardware implementation.

# Proposed changes to VVC draft text

In VVC Draft 7, the changes for the proposed method are highlighted as follows.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_cbf_flag ) { |  |
| … |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| IsLfnstParsed = 0 |  |
| if ( ! ( sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( cbWidth, cbHeight) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) ) { |  |
| IsLfnstParsed = 1 |  |
| } |  |
| IsMtsParsed = 0 |  |
| if( ! ( Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && (( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )))) { |  |
| IsMtsParsed = 1 |  |
| } |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth < 6 && log2TbHeight = = 5 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( IsLfnstParsed == 1 && sps_lfnst_enabled_flag = = 1 && lfnst_idx ! = 0 && transform_skip_flag[ x0 ][ y0 ][ cIdx ] = = 0 && log2TbWidth > = 2 && log2TbHeight >= 2 ){ |  |  |
| log2ZoTbWidth = 2 |  |  |
| log2ZoTbHeight = 2 |  |  |
| } |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| if( IsLfnstParsed == 0 && Min( log2TbWidth, log2TbHeight) >= 2 && ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ ( LastSignificantCoeffX != 0 && LastSignificantCoeffY != 0 ) ) && ( LastSignificantCoeffX < 4 && LastSignificantCoeffY < 4 ) && !( ( ( ( LastSignificantCoeffX + LastSignificantCoeffY ) > 3 ) ¦¦ ( ( LastSignificantCoeffX + LastSignificantCoeffY ) == 3 && LastSignificantCoeffX > LastSignificantCoeffY )) && (log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| lfnst_idx | ae(v) |
| IsLfnstParsed = 1 |  |
| if( IsMtsParsed = = 0 && ( LastSignificantCoeffX <= 15 && LastSignificantCoeffY <= 15 ) && lfnst_idx = = 0 && cIdx = = 0) |  |
| mts_idx | ae(v) |
| IsMtsParsed = 1 |  |
| if( sps_mts_enabled_flag && mts_idx > 0 && cIdx = = 0 ) |  |
| log2ZoTbWidth = Min( log2ZoTbWidth, 4 ) |  |
| if( sps_mts_enabled_flag && mts_idx > 0 && cIdx = = 0 ) |  |
| log2ZoTbHeight = Min( log2ZoTbHeight, 4 ) |  |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| if( log2TbWidth + log2TbHeight > 3 ) { |  |
| if( log2TbWidth < 2 ) { |  |
| log2SbW = log2TbWidth |  |
| log2SbH = 4 − log2SbW |  |
| } else if( log2TbHeight < 2 ) { |  |
| log2SbH = log2TbHeight |  |
| log2SbW = 4 − log2SbH |  |
| } |  |
| } |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| QState = 0 |  |
| … |  |
| } |  |  |

# Experiment results

The proposed method has been implemented on VTM7.0 [3]. Table 1 shows the results of the proposed method under the CTC [4]. Note that the runtime are not accurate.

Table 1. Results of the proposed method

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.16% | 0.29% | 100% | 99% |
| Class A2 | -0.01% | 0.22% | 0.12% | 100% | 98% |
| Class B | 0.01% | 0.09% | 0.19% | 100% | 98% |
| Class C | 0.02% | 0.08% | 0.15% | 100% | 98% |
| Class E | 0.02% | 0.02% | 0.24% | 101% | 99% |
| Overall | 0.01% | 0.11% | 0.20% | 100% | 98% |
| Class D | 0.04% | 0.06% | 0.27% | 101% | 97% |
| Class F | 0.01% | 0.03% | 0.22% | 99% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.14% | 0.06% | 100% | 98% |
| Class A2 | -0.03% | 0.09% | 0.16% | 100% | 108% |
| Class B | 0.01% | 0.01% | 0.23% | 99% | 100% |
| Class C | 0.01% | 0.07% | 0.10% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.15% | 100% | 101% |
| Class D | 0.03% | -0.20% | 0.03% | 100% | 98% |
| Class F | -0.03% | 0.06% | -0.07% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.17% | 0.09% | 101% | 99% |
| Class C | 0.06% | 0.06% | 0.00% | 100% | 99% |
| Class E | -0.06% | 0.33% | 0.15% | 99% | 99% |
| Overall | 0.03% | 0.03% | 0.08% | 100% | 99% |
| Class D | 0.01% | -0.25% | -0.16% | 99% | 97% |
| Class F | 0.15% | 0.04% | 0.52% | 100% | 101% |

# Conclusions

This contribution presents one method to remove the MTS index signalling redundancy. In the proposed method, MTS index and LFNST index are parsed right after parsing last significant coefficient position in the first nonzero TB. Results reportedly show 0.01%, 0.01% and 0.03% luma BD-rates under AI, RA, and LB, respectively.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

- S. De-Luxán-Hernández, V. George, G. Venugopal, J. Brandenburg, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, M. Koo, M. Salehifar, J. Lim, S. Kim, and J. Lainema, “Non-CE6: Combination of JVET-P0196 and JVET-P0392 on applying LFNST for ISP blocks and simplifying the transform signalling,” Document of Joint Video Experts Team, JVET-P1026.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).