| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0531-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0494 (CE5-related: Joint clip operation for CCALF and chroma ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Olena Chubach No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 olena.chubach@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This document reports results of crosschecking a CC-ALF-related contribution in which it is suggested to perform only one range-clipping operation after chroma samples processed by ALF and CCALF. When both chroma ALF and CCALF are enabled, chroma samples from chroma ALF module are range-clipped first, then added with the output from CCALF and range-slipped again. Such a double-clipping on chroma samples is asserted to be unnecessary. In JVET-Q0494, it is proposed to remove the first range-clipping performed right after chroma ALF module. The proposed method is applied to the CE5 anchor.

Verification

Kwai has provided the source code of their proposed method described in JVET-Q0494 [1]. Verification for the proposed test was carried out:

- The proposed method is applied to the CE5 anchor.

The proposed changes were implemented on top of CE5 common base software and tested based on the conditions specified in CE5 description [2]. The software matches the proposed changes in the contribution. The tests were conducted under the common test conditions (CTC) defined in [3] using the VTM7.0 software [4].

Table 1 shows the results using CE5 common base results as the anchor and enabling the proposed method in the test. The BD-rates of the test from our experimental results match exactly those in JVET-Q0494. The runtime varies within few percent, which is caused by inhomogeneous computing clusters.

Table 1. The results of test 1 in JVET-Q0494 using CE-5 common base results as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 100% | 106% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 101% |

- References

[1] Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang, “CE5-related: Joint clip operation for CCALF and chroma ALF,” Document of Joint Video Experts Team, JVET-Q0494.

[2] C.-Y. Chen, A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering,” Document of Joint Video Experts Team, JVET-P2025.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[4] VTM7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).