| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0537 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0311 (Non-CE5: On CC-ALF padding for ALF virtual boundaries) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution presents a crosscheck report of JVET-Q0311 (Non-CE5: On CC-ALF padding for ALF virtual boundaries). The experimental result of tester match with those provided by proponents.

The test1 disables CC-ALF process below the virtual boundary (i.e. CTU size - 4 in luma position).

The test2 modifies CC-ALF padding process

It is also reported that the test1’s YUV bdrate change is {0.00 %, 0.22 %, 0.19 %}, {0.00 %, 0.16 %, 0.22 %} and {0.00 %, 0.15 %, 0.24 %} on average compared to CC5 common software in AI, RA and LB respectively and test2’s bdrate change is {0.00 %, 0.00 %, 0.01 %}, {0.00 %, -0.01 %, 0.00 %} and {0.01 %, -0.07 %, 0.11 %} on average compared to CC5 common software in AI, RA and LB respectively

Experimental result

Test1: Disable CC-ALF process below the virtual boundary

Anchor: CE5 common base

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 common base(sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.14% | 0.16% | 100% | 100% |
| Class A2 | 0.00% | 0.23% | 0.38% | 99% | 100% |
| Class B | 0.00% | 0.40% | 0.30% | 100% | 100% |
| Class C | 0.00% | 0.08% | 0.08% | 100% | 100% |
| Class E | 0.00% | 0.16% | 0.04% | 99% | 100% |
| Overall | 0.00% | 0.22% | 0.19% | 100% | 100% |
| Class D | 0.00% | 0.07% | 0.02% | 99% | 100% |
| Class F | 0.01% | 0.19% | 0.14% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over CE5 common base(sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.32% | 100% | 100% |
| Class A2 | 0.02% | 0.23% | 0.28% | 100% | 100% |
| Class B | 0.01% | 0.36% | 0.24% | 100% | 100% |
| Class C | -0.02% | 0.03% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.16% | 0.22% | 100% | 100% |
| Class D | -0.02% | 0.15% | 0.28% | 100% | 100% |
| Class F | 0.01% | 0.16% | 0.10% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5 common base(sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.30% | 100% | 100% |
| Class C | 0.00% | 0.04% | 0.05% | 100% | 100% |
| Class E | 0.01% | 0.28% | 0.39% | 99% | 100% |
| Overall | 0.00% | 0.15% | 0.24% | 100% | 100% |
| Class D | -0.02% | -0.33% | 0.36% | 99% | 100% |
| Class F | 0.00% | 0.16% | 0.47% | 99% | 100% |

Test2: Modify CC-ALF padding process

Anchor: CE5 common base

|  | Over CE5 common base(sharp) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.01% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.02% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over CE5 common base(sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.17% | 0.03% | 100% | 100% |
| Class A2 | -0.01% | 0.08% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.09% | -0.16% | 100% | 100% |
| Class C | -0.01% | -0.09% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.04% | 0.05% | 100% | 100% |
| Class F | -0.01% | 0.05% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5 common base(sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.17% | 0.12% | 100% | 100% |
| Class C | -0.01% | 0.06% | -0.04% | 100% | 100% |
| Class E | 0.08% | -0.07% | 0.28% | 100% | 100% |
| Overall | 0.01% | -0.07% | 0.11% | 100% | 100% |
| Class D | -0.03% | -0.26% | -0.50% | 99% | 100% |
| Class F | 0.00% | 0.54% | 0.39% | 100% | 100% |

References

[1] Y. Wang, H. Liu, L. Zhang, K. Zhang, Z. Deng, Y. Wang, “Non-CE5: On CC-ALF padding for ALF virtual boundaries”, JVET-Q0311, JVET 17th Meeting: Brussels, BE, 7–17 January 2020

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).