| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0539 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0104 (On worst-case complexity of LFNST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomohiro Ikai | Tel: Email: | zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution presents a crosscheck report of JVET-Q0104 (On worst-case complexity of LFNST). The experimental result of tester match with those provided by proponents.

The test1 uses 64 primary transform coefficients rather than 48 primary transform coefficients in VVC-7 in 8x8 blocks in LFNST case. The transform matrices are replaced with new matrices.

The test2 uses 10 LFNST coefficients rather than 8 LFNST coefficients in VVC-7 in 8x8 blocks.

In both tests, the worst-case complexity in terms of number of multiplies per pixel is reportedly unchanged because the worst-case exists in 4x4 blocks.

It is also reported that the test1’s bdrate change is -0.09 % and -0.06 % on average in AI and RA respectively and test2’s bdrate change is -0.02 % and -0.02 % on average in AI and RA respectively.

Experimental result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.12% | 0.14% | 101% | 100% |
| Class A2 | -0.07% | -0.01% | -0.10% | 101% | 100% |
| Class B | -0.07% | 0.04% | -0.02% | 100% | 100% |
| Class C | -0.07% | 0.05% | 0.04% | 101% | 100% |
| Class E | -0.09% | -0.01% | 0.20% | 101% | 100% |
| Overall | -0.09% | 0.00% | 0.04% | 101% | 100% |
| Class D | -0.03% | 0.02% | -0.10% | 100% | 100% |
| Class F | -0.08% | -0.05% | 0.02% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.04% | 0.02% | 101% | 100% |
| Class A2 | -0.08% | 0.11% | -0.03% | 101% | 100% |
| Class B | -0.06% | 0.20% | 0.18% | 100% | 100% |
| Class C | -0.03% | 0.10% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.11% | 0.06% | 100% | 100% |
| Class D | -0.05% | -0.09% | -0.03% | 100% | 100% |
| Class F | -0.07% | 0.06% | 0.09% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | 0.05% | 100% | 100% |
| Class A2 | -0.01% | 0.05% | -0.08% | 100% | 100% |
| Class B | -0.02% | -0.02% | -0.09% | 100% | 100% |
| Class C | -0.03% | -0.09% | 0.00% | 100% | 100% |
| Class E | -0.03% | -0.05% | 0.05% | 99% | 100% |
| Overall | -0.02% | -0.06% | -0.02% | 100% | 100% |
| Class D | -0.01% | -0.07% | -0.15% | 100% | 100% |
| Class F | 0.00% | -0.08% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | -0.05% | 0.00% | 100% | 100% |
| Class B | -0.03% | 0.04% | 0.16% | 100% | 100% |
| Class C | -0.02% | -0.06% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | 0.05% | 100% | 100% |
| Class D | -0.03% | -0.31% | -0.03% | 100% | 100% |
| Class F | 0.01% | 0.01% | -0.02% | 100% | 100% |

References

[1] H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “On worst-case complexity of LFNST”, JVET-Q0104

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).