| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0555 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0392 (On the coding of cclm_mode_flag) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ramin Ghaznavi-Youvalari | Email: | ramin.ghaznavi-youvalari@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-Q0392 (On the coding of cclm_mode_flag). The simulation results reportedly match those provided by the proponents. Moreover, the description of the proposal matches with the software and spec text.

# Introduction

The document JVET-Q0392 proposed to to have a separate context for cclm_mode_flag when the co-located luma block is coded with MIP mode.

# Experimental results

The simulations were performed by using the code provided by the proponent in VTM-7.0. Table 1 shows the results in all intra (AI) and random access (RA) with CTC configuration [1].  Table 2 demonstrates the results of AI and RA configurations when the dual-tree is disabled.

Table 1: CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02 % | -0.10 % | 0.15 % | 101 % | 101 % |
| Class A2 | -0.01 % | 0.01 % | -0.08 % | 100 % | 100 % |
| Class B | 0.00 % | 0.04 % | -0.03 % | 101 % | 102 % |
| Class C | 0.00 % | 0.02 % | 0.03 % | 101 % | 104 % |
| Class E | -0.01 % | -0.08 % | 0.09 % | 101 % | 105 % |
| Overall | -0.01 % | -0.01 % | 0.03 % | 101 % | 103 % |
| Class D | 0.01 % | 0.12 % | -0.01 % | 101 % | 105 % |
| Class F | -0.01 % | 0.05 % | -0.09 % | 101 % | 104 % |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01 % | -0.20 % | 0.11 % | 100 % | 100 % |
| Class A2 | -0.05 % | 0.09 % | 0.00 % | 100 % | 100 % |
| Class B | -0.03 % | 0.03 % | 0.10 % | 100 % | 99 % |
| Class C | -0.01 % | 0.03 % | 0.01 % | 100 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.02 % | 0.00 % | 0.06 % | 100 % | 100 % |
| Class D | -0.01 % | -0.13 % | -0.17 % | 100 % | 101 % |
| Class F | -0.03 % | 0.05 % | -0.11 % | 100 % | 100 % |

Table 2: Results (dual-tree off )

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08 % | 0.04 % | 0.30 % | 99 % | 99 % |
| Class A2 | -0.05 % | -0.11 % | -0.11 % | 100 % | 99 % |
| Class B | -0.07 % | -0.13 % | 0.04 % | 100 % | 100 % |
| Class C | -0.04 % | -0.09 % | -0.06 % | 99 % | 100 % |
| Class E | -0.03 % | -0.38 % | -0.48 % | 100 % | 100 % |
| Overall | -0.06 % | -0.13 % | -0.05 % | 100 % | 100 % |
| Class D | -0.01 % | 0.15 % | 0.04 % | 99 % | 100 % |
| Class F | -0.02 % | -0.03 % | -0.13 % | 100 % | 100 % |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | 0.11 % | 0.13 % | 100 % | 100 % |
| Class A2 | -0.05 % | -0.07 % | -0.07 % | 100 % | 100 % |
| Class B | -0.01 % | -0.05 % | 0.16 % | 100 % | 100 % |
| Class C | -0.03 % | 0.02 % | 0.15 % | 100 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.03 % | -0.01 % | 0.11 % | 100 % | 100 % |
| Class D | -0.01 % | 0.25 % | 0.26 % | 100 % | 106 % |
| Class F | -0.04 % | 0.05 % | -0.05 % | 100 % | 100 % |

# Conclusion

In this contribution, the result of JVET-Q0392 has been confirmed.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).