| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET- Q0556 |
| --- | --- |

| Title: | Aligning intra-prediction of TS blocks with BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Gosala Kulupana 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Email: | saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to disable PDPC (Position Dependent Prediction Combination) and reference sample filtering on intra-predicted blocks, in case the corresponding residual blocks are Transform Skipped (TS). By doing so, handling of prediction in TS blocks is aligned with BDPCM blocks. The proposed alignment brings an average luma BD-rates of -0.10% and -0.05%, for Class TGM and Class F AI respectively with no encoder or decoder run time increment. For CTC, there is no performance change. All the coding performances are measured with respect to the VTM 7.0 anchor [1].

# Proposed method

The current VVC draft specification makes use of BDPCM [2] for screen content video sequences. Transform Skip is inferred in BDPCM blocks, and the transform skip residual coding mechanism is used for compressing the residual coefficients. Moreover, when intra-predicting BDPCM blocks, PDPC is not applied and the reference sample filtering is disabled. From a unification point of view, it is desirable to align the intra-prediction step of TS blocks with that of BDPCM blocks, given as they both make use of the same transform as well as residual coding mechanism. For this reason in this proposal it is proposed to disable reference sample filtering and PDPC operations for Transform skipped blocks.

The spec changes are as below:

Reference sample filtering related modifications

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to one of the following values: 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, 80, then refFilterFlag is set equal to 1.

  - transform_skip_flag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 1, then refFilterFlag is set equal to 0.

- Otherwise, refFilterFlag is set equal to 0.

PDPC related modifications

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, and the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as inputs, and the output is the modified predicted sample array predSamples:

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

  - transform_skip_flag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

# Performance results

The performance of the proposed method is summarised in Table 1. The tests were performed using the JVET common test conditions [2] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 7.0 [1], compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1 – disabling PDPC and reference sample filtering in TS blocks compared to VTM-7.0.

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | 0.01% | -0.04% | -0.01% | 101% | 98% |  |
| Class A2 | 0.00% | 0.00% | -0.05% | 102% | 100% |  |
| Class B | 0.00% | 0.06% | -0.07% | 101% | 98% |  |
| Class C | 0.00% | 0.03% | 0.00% | 102% | 97% |  |
| Class E | 0.01% | -0.07% | 0.07% | 101% | 100% |  |
| Overall | 0.00% | 0.01% | -0.02% | 101% | 98% |  |
| Class D | -0.01% | -0.04% | -0.03% | 102% | 98% |  |
| Class F | -0.05% | -0.10% | -0.01% | 102% | 100% |  |
| Class SCC | -0.10% | -0.11% | -0.12% | 100% | 97% |  |
|  |  |  | Random access Main10 |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | 0.00% | -0.09% | -0.07% | 101% | 101% |  |
| Class A2 | -0.01% | 0.06% | 0.05% | 101% | 100% |  |
| Class B | 0.01% | 0.01% | 0.18% | 101% | 100% |  |
| Class C | -0.02% | 0.03% | 0.16% | 101% | 102% |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.10% | 101% | 101% |  |
| Class D | 0.02% | -0.07% | -0.10% | 101% | 101% |  |
| Class F | -0.04% | 0.01% | -0.03% | 100% | 99% |  |
| Class SCC | -0.05% | -0.15% | -0.09% | 99% | 98% |  |

# References

- VTM-7.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, November 2019

- M. Karczewicz, and M. Coban, “CE8-related: Quantized residual BDPCM”, JVET-N0413, Geneva, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).