| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0559 |
| --- | --- |

| Title: | CCALF COEFFICIENT DERIVATION USING COMBINED NEIGHBORS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Limin Wang Jani Lainema | Tel: Email: | +1 858-674-6883 krit.panusopone@nokia.com seungwook.hong@nokia.com limin.2.wang@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution describes CCALF filter configuration that uses combined luma neighboring pixels. With the proposed combination, less correlated pixels provide less contribution to CCALF process resulting in improving performance. Simulation results show the proposed method 1 has BD-rate of approximately -0.02%, 0.24%, 0.05% for AI, -0.01%, 0.02%, 0.01% for RA, and 0.00%, 0.02%, -0.19% for LB, respectively, compared to CE5 common base. For the proposed method 2, simulation results show BD-rate of approximately 0.01%, -0.42%, -0.89% for AI, 0.00%, -0.67%, -0.93% for RA, and -0.01%, -1.31%, -1.53% for LB, respectively, compared to CE5 common base.

# Introduction

Cross-Component Adaptive Loop Filter (CCALF) is part of adaptive loop filter (ALF). It uses luma sample values to correct chroma samples. Specifically, CCALF applies a linear filter to luma samples in collocated region as coding chroma sample. Filter output is then added to the chroma sample value.

CE5 [1] investigates CCALF based on the technique proposed in JVET-P1008 [2]. In CE5, CCALF is implemented on top of VTM-7.0 as a common base for CE5 experiments. CE5 common base uses 3x4 filter shape with 8 coefficients as shown in Fig. 1. Fig 1 also shows filtering operations of CE5 common base.

offset0     = 0;

offset1     = strideCross;

offset2     = -strideCross;

offset3     = 2 * strideCross;

sum = filterCoeff[0] * srcCross[offset2 + jj2];

sum += filterCoeff[1] * srcCross[offset0 + jj2 - 1];

sum += filterCoeff[2] * srcCross[offset0 + jj2];

sum += filterCoeff[3] * srcCross[offset0 + jj2 + 1];

sum += filterCoeff[4] * srcCross[offset1 + jj2 - 1];

sum += filterCoeff[5] * srcCross[offset1 + jj2];

sum += filterCoeff[6] * srcCross[offset1 + jj2 + 1];

sum += filterCoeff[7] * srcCross[offset3 + jj2];

Fig. 1. CE5 common base filter shape and its filtering operations

# Proposed Solution

Two solutions are proposed in this contribution. Method 1 uses the same 3x4 filter shape as in CE5 anchor. However, the left most pair samples are combined, and average value of the pair is used in filtering process. Similar operation applies to the right most pair samples, resulting in 6-tap filter for this solution. Fig. 2 shows filter shape and filtering operations of method 1.

Left most pair samples

Right most pair samples

offset0     = 0;

offset1     = strideCross;

offset2     = -strideCross;

offset3     = 2 * strideCross;

sum = filterCoeff[0] * srcCross[offset2 + jj2];

sum += filterCoeff[1] * ((srcCross[offset0 + jj2 - 1] + srcCross[offset1 + jj2 - 1] + 1)>>1);

sum += filterCoeff[2] * srcCross[offset0 + jj2];

sum += filterCoeff[3] * srcCross[offset1 + jj2];

sum += filterCoeff[4] * ((srcCross[offset0 + jj2 + 1] + srcCross[offset1 + jj2 + 1] + 1)>>1);

sum += filterCoeff[5] * srcCross[offset3 + jj2];

Fig. 2. Method 1 filter shape and its filtering operations

Method 2 uses the same 5x4 filter shape as in CE5-1.1 [3]. However, the left most pair samples are combined, and average value of the pair is used in filtering process. Similar operation applies to the right most pair samples, second-to-left most pair samples, and second-to-right most pair samples, resulting in 8-tap filter for this solution. Fig. 3 shows filter shape and filtering operations of method 2.

Left most pair samples

Right most pair samples

Second-to-left most pair samples

Second-to-right most pair samples

offset0     = 0;

offset1     = strideCross;

offset2     = -strideCross;

offset3     = 2 * strideCross;

sum = filterCoeff[0] * srcCross[offset2 + jj2];

sum += filterCoeff[1] * ((srcCross[offset0 + jj2 - 2] + srcCross[offset1 + jj2 - 2] + 1)>>1);

sum += filterCoeff[2] * ((srcCross[offset0 + jj2 - 1] + srcCross[offset1 + jj2 - 1] + 1)>>1);

sum += filterCoeff[3] * srcCross[offset0 + jj2];

sum += filterCoeff[4] * srcCross[offset1 + jj2];

sum += filterCoeff[5] * ((srcCross[offset0 + jj2 + 1] + srcCross[offset1 + jj2 + 1] + 1)>>1);

sum += filterCoeff[6] * ((srcCross[offset0 + jj2 + 2] + srcCross[offset1 + jj2 + 2] + 1)>>1);

sum += filterCoeff[7] * srcCross[offset3 + jj2];

Fig. 3. Method 2 filter shape and its filtering operations

# Experimental Results

The proposed method 1 is implemented on top of CE5 common base. Experimental results are shown in Tables 1, 2 and 3 against the VTM-7.0 and the CE5 common base under AI, RA and LB CTC [4], respectively.

Table 1. Simulation result of method 1 for AI

|  | All Intra |  |  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | -4.89% | -5.23% | 209% | 165% | -0.01% | 0.05% | 0.08% | 110% | 165% |
| Class A2 | 0.11% | -7.96% | -12.40% | 209% | 167% | 0.00% | -0.01% | -0.17% | 111% | 167% |
| Class B | 0.14% | -13.12% | -10.48% | 211% | 161% | -0.01% | 0.18% | 0.09% | 112% | 161% |
| Class C | 0.16% | -3.85% | -3.55% | 209% | 153% | -0.03% | 0.06% | 0.04% | 111% | 153% |
| Class E | 0.21% | -8.70% | -2.28% | 209% | 158% | -0.03% | 1.01% | 0.16% | 110% | 158% |
| Overall | 0.15% | -8.09% | -7.02% | 210% | 160% | -0.02% | 0.24% | 0.05% | 111% | 160% |
| Class D | 0.09% | -3.44% | -1.89% | 208% | 161% | -0.01% | -0.05% | -0.18% | 111% | 161% |
| Class F | 0.14% | -5.82% | -6.01% | 188% | 159% | -0.03% | 0.06% | 0.05% | 101% | 159% |

Table 2. Simulation result of method 1 for RA

|  | Random Access |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -5.87% | -6.02% | 102% | 108% | 0.01% | -0.15% | 0.08% | 56% | 108% |
| Class A2 | 0.11% | -10.81% | -15.83% | 102% | 112% | 0.00% | 0.19% | 0.22% | 56% | 112% |
| Class B | 0.17% | -17.26% | -13.60% | 102% | 107% | 0.00% | 0.11% | -0.07% | 56% | 107% |
| Class C | 0.10% | -3.95% | -3.44% | 102% | 105% | -0.02% | -0.07% | -0.10% | 55% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.13% | -10.14% | -9.82% | 102% | 108% | -0.01% | 0.02% | 0.01% | 56% | 108% |
| Class D | 0.07% | -3.98% | -2.88% | 101% | 106% | -0.03% | -0.12% | 0.01% | 56% | 106% |
| Class F | 0.15% | -3.84% | -3.89% | 102% | 107% | -0.02% | 0.10% | -0.05% | 56% | 107% |

Table 3. Simulation result of method 1 for LB

|  | Low delay B |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.17% | -19.37% | -17.42% | 102% | 104% | 0.00% | -0.08% | -0.17% | 57% | 104% |
| Class C | 0.12% | -5.15% | -3.95% | 102% | 101% | 0.03% | 0.13% | -0.15% | 57% | 101% |
| Class E | 0.04% | -12.82% | -4.28% | 102% | 101% | -0.05% | 0.05% | -0.29% | 58% | 101% |
| Overall | 0.12% | -12.99% | -9.64% | 102% | 102% | 0.00% | 0.02% | -0.19% | 57% | 102% |
| Class D | 0.07% | -4.82% | -3.81% | 102% | 102% | -0.01% | -0.28% | -0.78% | 58% | 102% |
| Class F | 0.13% | -8.04% | -6.16% | 102% | 102% | 0.04% | -0.04% | 0.33% | 58% | 102% |

The proposed method 2 is implemented on top of CE5-1.1 software. Experimental results are shown in Tables 4, 5 and 6 against the VTM-7.0 and the CE5 common base under AI, RA and LB CTC [4], respectively.

Table 4. Simulation result of method 2 for AI

|  | All Intra |  |  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | -5.21% | -6.32% | 210% | 159% | 0.02% | -0.27% | -1.09% | 111% | 159% |
| Class A2 | 0.11% | -8.79% | -13.12% | 211% | 160% | 0.00% | -1.01% | -1.05% | 112% | 160% |
| Class B | 0.15% | -13.88% | -11.41% | 212% | 158% | 0.00% | -0.68% | -1.03% | 112% | 158% |
| Class C | 0.20% | -4.38% | -4.20% | 209% | 151% | 0.01% | -0.48% | -0.63% | 111% | 151% |
| Class E | 0.26% | -9.17% | -3.05% | 211% | 158% | 0.02% | 0.50% | -0.63% | 111% | 158% |
| Overall | 0.18% | -8.69% | -7.85% | 210% | 157% | 0.01% | -0.42% | -0.89% | 112% | 157% |
| Class D | 0.11% | -4.02% | -2.41% | 208% | 161% | 0.01% | -0.69% | -0.74% | 112% | 161% |
| Class F | 0.17% | -6.56% | -6.61% | 189% | 157% | 0.01% | -0.77% | -0.61% | 101% | 157% |

Table 5. Simulation result of method 2 for RA

|  | Random Access |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -6.39% | -7.06% | 196% | 162% | 0.03% | -0.73% | -1.04% | 107% | 162% |
| Class A2 | 0.10% | -11.68% | -16.49% | 193% | 164% | -0.02% | -0.81% | -0.64% | 106% | 164% |
| Class B | 0.19% | -17.89% | -14.57% | 193% | 157% | 0.02% | -0.73% | -1.29% | 106% | 157% |
| Class C | 0.11% | -4.31% | -3.94% | 193% | 153% | -0.02% | -0.43% | -0.62% | 105% | 153% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | -10.73% | -10.62% | 194% | 158% | 0.00% | -0.67% | -0.93% | 106% | 158% |
| Class D | 0.07% | -4.21% | -3.73% | 190% | 155% | -0.03% | -0.38% | -0.92% | 106% | 155% |
| Class F | 0.17% | -4.53% | -4.48% | 187% | 149% | 0.01% | -0.63% | -0.66% | 103% | 149% |

Table 6. Simulation result of method 2 for LB

|  | Low delay B |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 commonbase |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.16% | -20.68% | -18.34% | 102% | 101% | -0.01% | -1.78% | -1.23% | 57% | 101% |
| Class C | 0.10% | -6.61% | -4.84% | 103% | 101% | 0.01% | -1.42% | -1.09% | 58% | 101% |
| Class E | 0.07% | -13.18% | -6.49% | 105% | 100% | -0.03% | -0.39% | -2.62% | 60% | 100% |
| Overall | 0.12% | -14.12% | -10.88% | 103% | 101% | -0.01% | -1.31% | -1.53% | 58% | 101% |
| Class D | 0.04% | -5.91% | -4.55% | 103% | 101% | -0.04% | -1.44% | -1.62% | 58% | 101% |
| Class F | 0.15% | -9.14% | -6.99% | 104% | 100% | 0.06% | -1.30% | -0.57% | 59% | 100% |

# Conclusions

CCALF configuration using combined luma neighboring pixels is proposed in this contribution. Method 1, using 3x4 filter shape and 6-tap, has a combined BD-rate performance of -1.39%, -1.89% and -2.17%, compared to VTM-7.0 under AI, RA and LB coding conditions, respectively. Method 2, using 5x4 filter shape and 8-tap, has a combined BD-rate performance of -1.51%, -2.02% and -2.41%, compared to VTM-7.0 under AI, RA and LB coding conditions, respectively.

# References

- C.-Y. Chen, A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering”, JVET-P2025, Oct. 2019.

- K. Misra, F. Bossen, A. Segall, N. Hu, J. Dong, V. Seregin, M. Karczewicz, P. Onno, C. Gisquet, G. Laroche, J. Li, C.S. Lim, C.-W. Kuo, J. Nam, J. Choi, J. Lim, S. Kim, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, “CE5-related: On the design of CC-ALF”, JVET-P1008, Oct. 2019.

- A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina, “CE5-1.1: CCALF with 5x4 filter shape and 8 filter coefficients”, JVET-Q0085, Jan. 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).